# Introduction

Basic Idea behind Fisa Disk Server is to utilize those Disks, which are rejected by the Dos & Windows 95 Operating System. Fisa Disk Server Format the Rejected Disk in Special Format Named Fisa Format, Which is Enhanced Form of Normal Dos & Windows Format. The File System Placed on the Disk is Logically same as that of Dos ( FAT ), but the Physical Appearance of File System Structure on the Disk is Bit Different from Dos. A Special Device Driver is designed to handle the above Format. The Device Driver Logically Show the File System as Normal Dos File System. By doing this, Any Rejected Defective Disk Become Usable By Means of Device Driver and can be Access as Normal Dos Disk.

# How It Works

In Dos & Windows 95 File System ( FAT ), Disk is divided into Two Parts

- System Area

- Data      Area

System Area is Comprise of three Fields Boot Record, File Allocation Table and Root Directory Structure.

If any Sector Belongs to Data Area Goes Bad, the File System has the Capability to Mark it As a Bad Sector but there is no Such Provision for the System Area Sectors. Therefore if any Sector Belongs to System Area Goes Bad then Disk Become Unusable for Operating System.

While Fisa Format Organize the Disk in Similar Manner but Manages a Sector Map for System Area Sectors. This Map Resides in Boot Record and Boot Record Itself Resides in any Sector of the Disk. This Boot Record Has Special Signature to Recognize it as Fisa Boot Record.

An Installable Block Device Driver (Fisa_drv.exe) Adds A Logical Drive in Operating System’s Drive Space. This Logical Drive Uses the Existing Physical Floppy Drive (A or B) As its Physical Drive and has Capability to Handle Disk With Fisa Format. In Dos & Windows 95 Logical Block Devices are viewed as Linear Chain of Sectors.    And They must Have Boot Record at Sector # 0 and Copies of FAT in Successive Sector (i.e. 1,2,3,4,5....) and then Root Sectors and Then Data Area in successive Scoters. Fisa_Drv give the Logical Access to Disk as Operating System Desires. I.e. all Sectors are viewed as Linear Chain but physically there are not, Because of Sector Map.

# Compatibility

It is fully compatible With Dos & Windows 95 Operating Systems and Standard Disk Utilities, Which Works on Logical Drives Such as Scandisk.exe, Defrag.exe, DoubleSpace etc.

Since It Share the Same Physical Floppy Drive As of Logical Drive A or B, It is a Great Chance of Malfunctioning.    That’s why Fisa Disk Format Divide the Disk into TWO Parts.    One is Consist of Small Normal Dos Disk (size = 2.5 KB) and 2nd is Consist All Remaining Space. 2nd Part is Fisa Disk.

# Drawback

Disk with Fisa Disk Format Cannot Access By Dos with out Device Driver (Fisa_drv.exe). The Only Part Access By Dos With out Device Driver is a Small Disk of Size (2.5 KB).    Therefore Fisa Disk Are Not Bootable Disks and they are Only Secondary Storage Media.

# Extendibility

This Logical Format Can Be Extend To Handle The Hard Disks.