base system operation. Therefore, adjust the VOX only until you get it set appropriately for your riding needs. System use will seem easier at higher speeds as one naturally raises one’s voice when in high noise conditions. The system incorporates side-tone (the ability to hear your own voice through the speakers), which has the tendency to cause one to speak quieter. As you practice using the system and become more comfortable, system operation will become natural. If you experience difficulty, contact your supplier for help or a demonstration that may shorten the learning curve on how to get the most out of your system.

ADJUSTING THE VOX

Turning the black knob counter-clockwise stiffens the VOX making the microphones harder to activate, while increasing the speed at which the system will reject unwanted noise. Rotating it clockwise will make it easier to activate the VOX, but will lower the speed at which the VOX system initiates. Adjust the VOX to find the optimum position for your riding style and environment. As your gain experience with the system you should find that you can stiffen the VOX while maintaining the same level of comfort with the system’s operation.

OPTIONAL ACCESSORIES

A variety of accessories are available to enhance your system. Ask your dealer to show you the range of accessories. Additionally, all items included in this kit are available separately.

BIKE-TO-BIKE AND CELLULAR PHONE

The system can be expanded to incorporate a bike-bike transceiver powered from the intercom unit or powered by its own battery. Extension leads and push-to-talk systems are also available. Alternatively a cellular mobile phone can be connected enabling you to receive and send calls.

MUSIC OR RADAR DETECTOR

Portable and bike-based music system and radar detector interfaces are available.

NOTES

•	The system you have purchased contains highly advanced noise cancelling circuitry. The microphone is designed to cancel out sound that is not directed at it with the proper angle and amplitude. It will reject voice as well as any helmet noise, if it is not used correctly. When you first use the system it may seem different to other systems. This is a highly accurate observation. With practice, the system will soon become quite natural.

•	If you are not sure of how best to fit the system contact your supplier.

•	The system is not waterproof or designed to work in sub-zero temperatures. For use in these environments, it is necessary to provide additional protection for the system

•	The system will look to draw current at all times. For this reason it is important that you to connect to a switched power source that activates and deactivates with the ignition. (Note: the accessory socket on most BMW® motorcycles is not a switched supply.)

•	Interference heard through the system indicates a noisy electrical system on the bike. A ticking’ sound that varies with revs is typically spark plug/cap related, while a ‘whining’ sound that varies with revs is typically alternator related. Check to insure that the ground is correctly made directly to the battery, that a quick connect type connector has not been used, and that the routing of the wires are not in the vicinity of ignition system parts,

PRECAUTIONS

•	If integrating music apply common sense. DO NOT have the volume level beyond that of reason. Law requires that the rider be in control which includes the ability to hear road warnings (ex. emergency vehicles, horns). Be sure to maintain this level of control.

•	Ensure that the leads are positioned to ensure safe riding. In particular be sure that the leads do not impair sight lines and that the din connections are positioned to move freely.