# Remarks

# Volcanic Eruptions

Peak Wind  -  PK WND 28045/15  (>25kts)

Wind Shift  -  WSHFT 30  (>45° <15min  >10kts)

# Tower Visibility  -  TWR VIS 9999  (Vis < 4miles)

Variable Prevailing Visibility  -  VIS 0800V3200 (Vis <3miles)

Sector Visibility  -  VIS NE 4000 (45° arc prevailing or sec <3miles)

Beginning and Ending of Precip  -  SNB05E30SGB20E55

Virga  -  VIRGA or VIRGA SW

Variable Ceiling Height  -  CIG 005V010 (Ceiling < 030)

Obscurations   -  FG SCT000 FU BKN020

Variable Sky Condition  - SCT020 V BKN (Ceiling < 030)

Significant Cloud Types  -  ACSL NW      APPRNT ROTOR CLD NE

Pressure Rising or Falling Rapidly   -   (.06 Hour chg at least .02 chg) - PRESRR or PRESFR

Sea-Level Pressure  - SLP090

Aircraft Mishap  -  (ACFT MSHP)

Runway Conditions  -  LSR12 or RCRNR (02 is worst 25 best traction)

Proximity Qualifier  -  VCSH E (<10miles) or SN DSNT W (>10miles)

Fata Morgana  -  FATA MORGANA GRID N

Grid Wind  -  GRID32012KTS

Surface and Horizon Definitions  -  SFG/HDF

First or Last  -  FIRST or LAST