Intercept Procedures

If you are intercepted by a US Military or law enforcement aircraft, immediately:

- Follow the instructions given by the intercepting aircraft (See chart.)

- Notify ATC, if possible.

- Attempt to communicate with the intercepting aircraft and/or ATC on the emergency frequency 121.5 MHz, giving the identity and position of your aircraft and the nature of the flight.

- If equipped with a transponder, squawk 7700, unless otherwise instructed by ATC.  If any instructions received by radio from any source conflict with those given by the intercepting aircraft by visual or radio signals, request clarification while continuing to comply with the instructions given by the intercepting aircraft.

| Interceping aircraft signal | Meaning | Intercepted aircraft response | Meaning |
| --- | --- | --- | --- |
| Rocks wings. After acknowledgement initiated a slow level turn, normally to the left, onto the desired heading. At night, the pilot will also flash the navigational lights at irregular intervals. | You have been intercepted. Follow Me. | Rocks wings and follows. | I understand and will comply. |
| Performs an abrupt breakaway maneuver consisting of a climbing 90 degree turn, or more, without crossing the intercepted aircraft’s flight path. | You may proceed. | Rocks wings. | I understand and will comply. |
| Circles airport, lowers landing gear, and overfiles runway in the direction of landing. At night, the pilot will also put the landing lights on. | Land at this airport. | Lowers landing gear, follows the intercepting aircraft and lands if the runway is considered safe. At night, the pilot will also put the landing lights on. | I understand and will comply. |

| Intercepted aircraft signal | Meaning | # Intercepting aircraft response | Meaning |
| --- | --- | --- | --- |
| Raises landing gear while flying over runway between 1,000’ and 2,000’, and continues to circle the airport. At night, the pilot of the intercepted aircraft will also flash landing lights while passing over the runway | This airport is inadequate. | If the intercepted aircraft is requesting to go to an alternate airport, the intercepting aircraft raises its landing gear and uses the intercept procedures (listed above.) | Understood, follow me. |
| The pilot switches on and off all available lights at regular intervals. | Cannot comply. | Performs breakaway maneuver. | Understood. |
| The pilot switches on and off all available lights at irregular intervals. | In distress. | Performs breakaway maneuver. | Understood. |