I am not responsible if anything goes wrong, but this is how my Coupe GT is tune, this is the only way my car gets the most horse power out.

I believe that the 80 quattro did not have a Oxygen adjustment in the Throttle body.  If it did do not attempt to adjust, the screw should be all the way IN.  All his is adjust by the computer.

Here are the instructions for tuning the fuel system on a 2.3l NG engine.

All timing must be stock or line up, plus TDC 15 degrees to the left.

Tools:

- Ohm meter.

- Allen Key

- You can make an adapter to fit in between the Fuel pressure Regulator and the wire connections.  Or just peal the rubber back and hold it tight.  Just get a female and a male from an  junk car,  is same one that the oxygen sensor use and the throttle switch use.  Any Audi or VW would have it. (what are you trying to is connect the Ohm meter to the Pressure Reg. While is connected and the connecting is good to read ohms.

Procedure Part 1:

- Engine off

- Connect the Ohm meter to the (+ to + ) and ( - to - ) to the Fuel pressure regulator, or the adapter.

- Turn engine on.

- The Ohm meter will read the resistance,  you want to change it in order to read ZERO, by adjusting the gas flow with the Allen key at the fuel distributor. (You made go left or right )

- Once this reads ZERO Ohms Open the throttle with your hands and see if the resistance change or not, they should stay at Zero.  Make adjustments as you need with the Allen key.

- Connect everything back to normal.

Procedure Part 2 (Setting the computer to new settings.)

- Engine Off.

- Disconnect Oxygen Sensor (from the female-male connection)

- Unplug the blue rubber plug from the exhaust. (between intake manifold and air box it is attached to the intake manifold)

- Turn engine on. (let it run for a minute or two.

- Rev the engine for 9-seconds up to 15 seconds to 2800-RPM to reset new adjustment on the computer.

- Engine off.

- Plug everything back to normal.

This car is suppost to run around 800-to-950 RPMs