Assembly notes for Dual-Atmel ISO7816 Repair Station-Loader

The Dual-Atmel repair station-loader allows you to switch between two atmels without reflashing. The repair station-loader’s two atmels can be flashed independently with different HU or Unlooper code. The repair station-loader has a built in daughter board and three separate voltage regulators to produce accurate repeatable glitching.

Use the Eagle light version 4.0 software from www.cadsoft.de  to print the schematics and the boards. Remember to turn off the placement layers and the top layer before you print the overlays for the boards. CHECK ( www.protronicsmn.com ) my website and DOWNLOAD the latest version of the plans , I am continually updating the plans as new information becomes available.

Assembly is pretty straight forward, it IS important that you use the multiplexer chips that I specified in the parts list. While other multiplexers will work the ones specified will produce the fastest most predictable glitching.  The 74HC04 is critical also, it has to be the 74HC04 to oscillate properly.

Switch SW1 selects which atmel you want to run or flash. To flash the A atmel, select FLASH A ( on the silk screen) , to run A the switch must be moved to the RUN A position.  The dip switch settings are pretty self explanatory, in normal operation switches 2, 5, 6 would be on. Pin 6 disables pin1 of the atmel that is selected ( A or B), for phoenix mode flash code. Pin 5 disables the daughter board.

There are two jumpers on the board( JP1 and JP2), they are included for future tweaks of the regulator voltages.  You can jumper these pads using a piece of wire or install a jumper block, but they have to be jumped in order to work properly. DONOT RUN WITHOUT JUMPERS, you will damage the IC chips.

Circuit board, this is a very difficult board for the home hobbyist to build that’s why there are two different versions of the circuit board. The board released with the free plans is larger with bigger traces than the commercial board version that I am making available. The commercial boards are available separately ( www.protronicsmn.com )for those that don’t have the skill or patience to build their own. You will need double sided presensitized positive etch photo boards. The boards I used for the prototypes were GC part # 21-350 6”x6”  presensitized positive acting printed circuit boards. Printing to both sides of the board is not as difficult as you might think. The trick is aligning the two sides . That is accomplished by aligning the top overlay on the blank board , without removing

the protective cover on the board and using a yellow bug light to prevent exposing the board.  Pick a solder pad hole on each of the four sides of the overlay ( registration holes), using a .031” drill drill a hole through the overlay and the board at each of the four spots. Remove the top overlay and match it up with the bottom overlay. Match the four registration holes through top overlay and drill the four holes through the bottom overlay.  You now have two overlays  (top and bottom) and one circuit board with four  matching holes .  I use a block of foam and a glass sheet to expose the circuit board.

Remove the protective sheet from the top of the board, align the overlay with the holes

you drilled. Now insert a short piece of wire in each drilled hole, this holds the overlay in place, that’s why I use foam , the wire will press into the foam when I lay the glass on

top. Carefully lay the sheet of glass on top of the overlay , expose the board ( I use two

U-V tubes ( GE F15T8-BLB) at 6”  for 5 minutes) . Now turn the board over, remove the protective film from the bottom of the board, align the bottom overlay with the registration holes you drilled, insert a short piece of wire in each hole and lay the sheet of glass on top of  the board. Expose the bottom of the board the same way you exposed the top. Develop and etch the board following the instructions that came with your developer and etchant.  Drill the holes as you normally would. Once the holes are drilled there are several holes that you will push a wire through and solder on both sides , these are vias, no components go in these holes they are just to connect the printed circuits on both sides of the board. This board has 10 vias, you can find where they are by turning off all the layers but the vias layer in Eagle.  Make sure to do a continuity test of ALL the traces before assembling( if you purchased the commercial board it is already tested) , it is much easier to repair bad solder traces BEFORE you have the components installed.

Because the design uses a doublesided board we have to be able to complete the circuit between the two sides of the board. The commercial board uses plated through holes to accomplish this, but for us to do this we have to use pins that can be soldered on both sides. For my prototypes I used machine pin strips ( Digi-key part # ED7064-ND you will need three strips) in place of the sockets .  Be sure to solder BOTH sides of the board at each pin. You will also need to solder BOTH sides of the board for each component. The card slot connector has one top trace that needs to be soldered to the bottom connector on the card slot, this is done by inserting a very thin wire through the hole soldering it to the top trace ( not the hole ) snap the socket in place making sure not to break the wire then solder the to the card slot pin on the bottom of the board

Card Slot holes on top of board viewed from top.

00

0

0

Bend over wire and solder to trace  here -  =====0 – push wire through this hole

0

The same must be done with the serial connector,  the holes to feed wire through are shown below.

Top view of the board top layer

Front

Here

|  |

0 0 0 0

0 0 0 0 0

|  |

Here

The last ones we have to do are on the selector switch , again it is the top view of the top layer.

0

- 0

Here -----0     0----- Here

- 0

0

Don’t forget to solder the wires to the top layer before inserting the switch , serial connector and cardslot.

Also don’t forget to solder the two jumper wires at jp1 and jp2 .

The two atmels are labeled A and B , looking from the top with the cardslot on your right move the slide switch to the down position. In this position you can flash the A atmel with what ever code you want , say HU-Loader code. You then move the slide switch to the up position to run the code . The up position also allows you to flash the B atmel, to run the B atmel you move the slide switch to the down position. It seems complicated but it is actually very simple, in the Run A position ( Up)  you can run A and flash B, in the Run B position ( down) you can run B and flash A.

To make the Repair station glitch even faster install a 4.7 k  ¼  watt resistor between           pins 7 and 8 of  IC5 to act as a pullup resistor for the opamp(IC5).

Flash the Repair Station with HU loader or Unlooper flash code, NOT BL.EXE  ( its for bootloaders ).

## Commercial Board Notes

1.  You can ignore all of the jumper wire notes above except for two , JP1 and JP2,  they are included for future tweaks of the regulator voltages.  You can jumper these pads using a piece of wire or install a jumper block, but they have to be jumped in order to work properly. DONOT RUN WITHOUT JUMPERS, you will damage the IC chips. The other jumpers are replaced by the plated through holes on the manufactured board.

2. The 74HC4053’s ( IC7 and IC8) are installed the reverse of the rest of the IC’s on the board, follow the silkscreen on the board to install them properly .

3. The 6 position dip switch (S4) should be installed with the switch NUMBERS matching the silkscreen numbers , they changed the switch after we had the boards made.

## General Assembly hints for both Boards

- If you use the capacitors as listed in the parts list the parts shipped to you can be confusing, the part numbers on the parts don’t give the actual values .

C3 - .0047ufd will have    472  as the part number written on it.

C4 - .047ufd will have      473  as the part number written on it.

C3 - .1ufd will have          104  as the part number written on it.

C4,C5 27pf  will have    27 or 27J  as the part number written on it.

2. The capacitors C1,C2,C8,C9 are all polarized electrolytics, they have a plus on one side and a negative on the other. Make sure to match the - on the board with the negative on the capacitors.

3. Install the IC sockets first, then the regulators, then the capacitors and resistors, install the connectors last, it makes it easier to install the components.

4. To make the Repair station glitch even faster install a 4.7 k  ¼  watt resistor between           pins 7 and 8 of  IC5 to act as a pullup resistor for the opamp(IC5) this is particularly effective if you get sent the HA17358 as a sub for the LM358  .

5. Flash the Repair Station with HU loader or Unlooper flash code, NOT BL.EXE  ( its for bootloaders ).

6. You MUST use a straight thru serial cable, like a serial extension cable, not a null modem or modem cable.

7. Make sure you have pin 6 of the dip switch S4 turned on in order to flash the Atmels.

8. The normal settings for S4 are 2,5,6 on and 1,3,4 off. 1,2,3,4 change the phase and freq of the clock only 1 of these four can be on at a time, you can try different settings to glitch stubborn cards, 5 turns of the daughter board  and 6 turns off pin 1 of the atmels for use in phoenix mode.

9. I have seen a number of people substitute the card slot and find their Repair Station won’t flash, there is a switch on the cardslot that is a normally closed switch, the sub is normally open. The switch is used to prevent having a card in the Repair when it is being flashed, as well as to notify software when the card is inserted. The correct switch will show a 00 continuity reading  with no card in the Repair Station.

I have put together a kit of parts including the circuit board.   If you are interested in such a kit visit my website at http://www.protronicsmn.com/.

Email at timeshifte@protronicsmn.com

Visit my website at http://www.protronicsmn.com/

All materials presented here are for educational, NON-COMMERCIAL and testing purposes only, any other use is strictly forbidden. No fee may be charged for the distribution or compilation of theses files without the express written permission of the author. All circuit boards, layouts, assembly files  and circuit designs are copywritten and remain the property of Timeshifte.  By downloading the files distributed here you agree to abide by the above rules.