MAXIMIZING NEUTRON OUTPUT OF AN IEC FUSION DEVICE

Carl A. Willis and David Barron (Dr. Rexford Adelberger)

Department of Physics

Guilford College

5800 West Friendly Avenue

Greensboro, NC 27410

Most research in controlled nuclear fusion has focused on inertial and magnetic confinement of fuel nuclei.  There is a less investigated technique known as inertial electrostatic confinement (IEC).  We are conducting experiments with an IEC device known as a Farnsworth Fusor, which uses a strong electric field between concentric spherical electrodes to radially accelerate deuterons to a central point of collision.  Our test fusor is composed of an 8-inch diameter stainless steel spherical vacuum vessel that contains deuterium gas at about 10-3 Torr.  A spherical wire grid cathode, held at 15-60 kV, is positioned concentrically within the chamber which acts as the anode.  Fusion of deuterons occurs in the space inside the cathode grid.  We are investigating the relationship between the cathode radius and the rate of deuteron fusion at the center.  A polyethylene-moderated boron trifluoride counter is used to measure the 2.5 MeV neutron flux resulting from the 2H + 2H  3He + n fusion reaction.  We believe our experiment will reveal the optimum ratio of cathode / anode radius for fusor operation.