RADIO CONTROL PROGRAM FOR WINDOWS

By Lydia Rosa

Copyright 1995,1997,1998,1999,2000,2001,2002  by Lydia Rosa

CONTENTS

WHAT'S NEW IN THIS RELEASE ............................................... 1

DISCLAIMERS .............................................................. 3

REQUIREMENTS ............................................................. 4

INTRODUCTION ............................................................. 5

Defining Some Terms .................................................... 5

Program Overview ....................................................... 6

STARTUP .................................................................. 8

Installation ........................................................... 8

Getting Started ........................................................ 8

Some simple operations ................................................. 8

THE DISPLAY .............................................................. 9

MAIN PANEL .............................................................. 10

Channel ............................................................... 10

Memory Scroll Buttons ................................................. 10

Operation Buttons ..................................................... 10

Manual Mode ......................................................... 10

Scan Mode ........................................................... 10

Search Mode ......................................................... 13

Load ................................................................ 15

Set ................................................................. 16

Mem->VFO ............................................................ 16

VFO->Mem ............................................................ 16

Radio->Search ....................................................... 16

Search->Radio ....................................................... 16

Start Load/Set ...................................................... 16

Stop Load/Set ....................................................... 16

Swap VFO  Button .................................................... 16

Speed Control ......................................................... 17

Global Switches ....................................................... 17

Memory Sync ......................................................... 17

Priority ............................................................ 17

Banks On ............................................................ 17

Resume .............................................................. 17

Don't Update Memory ................................................. 17

Log To File ......................................................... 18

Light on Signal ..................................................... 18

Scan Trunk Freqs .................................................... 18

CTCSS Enable ........................................................ 18

Status Controls ....................................................... 18

Radio Selection ..................................................... 18

Signal Meter ........................................................ 18

Main Panel Menu ....................................................... 19

File menu ........................................................... 19

View Menu ........................................................... 19

Clear Everything .................................................... 19

Parms ............................................................... 19

Global Parms ...................................................... 19

Priority ........................................................ 19

Delay ........................................................... 19

Resume .......................................................... 20

Font ............................................................ 20

Font Size ....................................................... 20

Display Unused Aux .............................................. 20

File Save Format ................................................ 20

Load Frequency DBF on Start ..................................... 20

Radio Parms ....................................................... 20

Radio Name ...................................................... 20

Contents         <Radio Control Program for Windows>              Page ii

Com Port ........................................................ 21

Baud Rate ....................................................... 21

Data Bits ....................................................... 21

Stop Bits ....................................................... 21

Flow Control .................................................... 21

Min Channel ..................................................... 21

Max Channel ..................................................... 21

Min Frequency ................................................... 21

Max Frequency ................................................... 21

Signal Detect ................................................... 21

Protocol ........................................................ 21

Model ........................................................... 22

Address ......................................................... 22

VFO Channel ..................................................... 22

Timeout ......................................................... 22

Enable Extended Commands ........................................ 22

Default settings by protocol ...................................... 22

VFO PANELS .............................................................. 25

VFO Frequency ......................................................... 25

Modulation ............................................................ 25

VFO step value ........................................................ 25

Digit Scrolling ....................................................... 25

Trunked Freq .......................................................... 26

VFO Active ............................................................ 26

Aux Controls .......................................................... 26

Attenuator .......................................................... 26

Notch|Tone Enable ................................................... 26

AmSync|Recorder ..................................................... 26

PreAmp .............................................................. 26

Tone|Notch Filter value ............................................. 26

AGC ................................................................. 26

Ant ................................................................. 26

Noise Blanker ....................................................... 27

Bandwidth ........................................................... 27

Tone 2 .............................................................. 27

MEMORY CHANNEL PANEL .................................................... 28

Channel Number ........................................................ 28

Valid ................................................................. 28

Delay ................................................................. 28

Trunked Freq .......................................................... 28

Rsm ................................................................... 28

Frequency ............................................................. 28

Mode .................................................................. 28

Count ................................................................. 29

Time .................................................................. 29

Signal ................................................................ 29

Time Stamp ............................................................ 29

Service ............................................................... 29

Aux Controls .......................................................... 29

Attenuator .......................................................... 29

Notch|Tone Enable ................................................... 29

AmSync .............................................................. 29

PreAmp .............................................................. 30

Notch|Tone  Value ................................................... 30

AGC ................................................................. 30

Ant ................................................................. 30

Noise Blanker ....................................................... 30

Bandwidth ........................................................... 30

Tone 2 .............................................................. 30

Menu .................................................................. 30

Database Menu ....................................................... 30

Edit menu ........................................................... 30

SCAN BANKS PANEL ........................................................ 32

Contents         <Radio Control Program for Windows>             Page iii

Channel number ........................................................ 32

Valid ................................................................. 32

Start ................................................................. 32

Stop .................................................................. 32

Service ............................................................... 32

Menu .................................................................. 32

Database menu ....................................................... 32

Edit menu ........................................................... 32

SEARCH BANKS PANEL ...................................................... 34

Channel number ........................................................ 34

Valid ................................................................. 34

Start ................................................................. 34

Stop .................................................................. 34

Incr .................................................................. 34

Mode .................................................................. 34

Service ............................................................... 34

Menu .................................................................. 35

Database menu ....................................................... 35

Edit menu ........................................................... 35

MESSAGE PANEL ........................................................... 36

SPECTRUM PANEL .......................................................... 37

Information unique for various radios ................................... 38

ICOM .................................................................. 38

KENWOOD ............................................................... 38

OPTO .................................................................. 38

AOR3000a .............................................................. 38

AOR5000 ............................................................... 38

AOR8000 ............................................................... 38

BEARCAT ............................................................... 39

Functions available for each radio .................................... 39

SPECIAL KEYS ............................................................ 41

ENTER ................................................................. 41

Tab Keys .............................................................. 41

DATA FILE FORMATS ....................................................... 42

LOGFILE format ........................................................ 42

FREQLIST.DBF .......................................................... 42

data file format ...................................................... 42

RS232 INTERFACE ......................................................... 50

PROGRAM LIMITS AND RANGES ............................................... 51

Contents         <Radio Control Program for Windows>              Page iv

WHAT'S NEW IN THIS RELEASE

Version 0.1

First Beta test release

Version 0.2

Fixed problem with COM port open failure aborting program.

Fixed various ICOM modulation problems.

Changed DIGITs to FONTS instead of BITMAPS to reduce flashing.

Version 0.3

Fixed problem with Program restarting when closed from ICON.

Fixed hangs and errors caused when switching radios.

Fixed functional problems with AOR3000 code.

Added code to process commands in File (including CHAIN)

Version 0.4

Fixed various AOR8000 problems.

Fixed more bugs.

Made NT and Win95 versions separate (due to DLL differences).

Version 0.5

Fixed several problems with writing Frequency files.

Added DEBUG file generation for troubleshooting.

Added support for DRAKE R8a radio.

Added short form/abbreviations for Frequency files.

Updated Radio Option panel code.

Added additional settings for AOR8000 and KENWOOD radios.

Separated display into multiple panels.

Added range selection for LOAD and SET

Added COPY/MOVE/SWAP operations.

Version 0.6

More bug fixes

Support for Bearcat 895xlt added.

Support for AOR500  added.

Fixed several problems that caused program to die.

Enabled Extended options on PARMS panel.

Re-arranged screen layouts to allow for future additional controls.

Changed VFO A/B operation.

Version 0.7

Fixed problem with AOR8000 not scanning properly.

Support for Bearcat 245xlt added.

Added VFO channel selection for Bearcat radios

Fixed problem with RADIO scan mode.

Added TRUNK keyword

Added FONT selection for Frequency Display (Much requested)

Version 0.71

BUG FIX- Numerous Bearcat bugs fixed

Fixed "sticky" FLOW CNTRL selection on OPTIONS panel

Version 0.80

Merged NT & 95/98 into one file.

Added radio locate function to OPTIONS panel.

re-wrote radio routines to remove hang when radio not connected.

Resized panels for bigger fonts.

Added option to hide unused AUX controls.

Version 0.90

Numerous problems fixed.

Removed Windows 3.1 support.

What's New       <Radio Control Program for Windows>               Page 1

Version 0.91

More bug fixes.

Added support for additional ICOM radios.

Changed VFO A/B operations (Radios use 1st VFO always).

Left/Right Mouse buttons now working.

Bandwidth kept in sync with FM modulation type.

Updated SPEED and DELAY routines for better uniformity.

Added SIZE selection for Frequency fonts.

Version 0.92

Fixed numerous bugs.

Added support for Kenwood THF6A.

Added code to Parms Panel to select best defaults.

Added support for additional BEARCAT models

Added Separate windows for VFOA and VFOB.

Window positions are now remembered.

Unrelated AUX controls are now hidden if GLOBAL SHOW AUX not checked.

Added option to bypass DBF load at startup.

CTCSS scanning for radios supporting it.

Logging to file added.

Added switch to disable update of memory channels.

************************************************************

**** A big THANK YOU goes out to all of the folks that  ****

****        Helped test out this version!               ****

************************************************************

What's New       <Radio Control Program for Windows>               Page 2

DISCLAIMERS

IBM is a trademark of IBM corporation.

ICOM is a trademark of Icom incorporated.

KENWOOD is a trademark of Kenwood Corporation.

AOR     is a trademark of AOR inc.

DBASE III is a trademark of Borland International.

RADIO SHACK is a trademark of Tandy Corporation.

BEARCAT  is a trademark of Uniden  Corporation.

The author  takes no responsibility  for any  damages that may occur to

your radio or computer due to the operation of this program.

This  program  is  being  distributed  as  SHAREWARE. This  means  that

voluntary  contributions are the only source of remuneration obtained for

this work. A contribution of $25.00 gives you the following:

1.  Fixes for any bugs.

2.  Schematics and instructions for the interfaces.

3.  A clear conscience.

4.  Removal of Nag Screen

Please  support continued development of RadioCtl  by registering  your

program.

Please mail all contributions, problem reports  and suggestions to

Lydia Rosa

P.O. Box 414

Marlboro, N.Y.

12542

The author  can also be contacted via E-mail at

richardrosa@yahoo.com

The latest versions of RadioCtl can be downloaded from

www.geocities.com/SiliconValley/1552

Thank you for your support.

Disclaimers      <Radio Control Program for Windows>               Page 3

REQUIREMENTS

In order to run this program you will need the following:

*    A 486 or Pentium based computer.

*    At least 8mb of memory (16mb is recommended).

*    about 2mb of free disk space (for the program and document).

*    Windows 95, Windows 98, Windows NT, Windows 2000.

*    A free Serial (RS232) port.

*    one of the following models of radio:

-   ICOM R-7000*

-   KENWOOD R-5000*

-   AOR3000A (not the AOR3000)

-   AOR8000/AOR8200

-   AOR5000

-   BEARCAT BC895xlt*

-   BEARCAT BC245xlt*

-   RadioShack   radio   equipped  with   OPTOSCAN  interface   (from

OptoElectronics).

-   KENWOOD TH-F6A

*NOTE: Although not tested, this program should work with other models of

this manufacturer equipped with a serial port.

For Kenwood and Icom Radios, a  modified interface is required for full

program  functionally  (signal detection).  For  all  other  radios,  the

manufacturer's  interface will work. However,  signal  detection  can  be

speeded up by use of the modified interface for  all of the radios. Plans

for this interface are  made available  free of charge for all registered

users.

Unfortunately, the  AOR3000  does not currently work with this  program.

The  protocol  differences  between the  AOR3000  and  the  AOR3000A  are

different enough to cause incompatibilities with RadioCtl.

Requirements     <Radio Control Program for Windows>               Page 4

INTRODUCTION

The  RADIO REMOTE CONTROL PROGRAM for Windows will allow you to control

selected radios from  any computer serial  port  (RS232)  using  Windows.

This program expands the capabilities of your radio  to 1000 channels and

100 programmable scan or search banks.  In addition, scanning, searching,

and logging  facilities are  also provided.  Some  radios  will  need  an

additional interface to utilize the  signal detection facilities of  this

program.  A circuit diagram  and build  instructions are provided free of

charge to all registered users.

The  program  allows  you to select various baud rates, COM  ports, and

other parameters associated with individual radios. Up to eight different

radio configurations can be defined for easy  switching.  A LOCAL mode is

also provided to run the program without the need for a radio connection.

Frequency  information  can  be loaded  or saved from a  text file.  An

interface to a DBASE type file is also provided  for frequency to service

lookup.

DEFINING SOME TERMS:

There are various  terms and descriptions used throughout this  manual.

Although RadioCtl is fairly easy to use, it helps to have a clear picture

of what the program is doing and  how it  is doing it.  Various terms are

used throughout this document to explain the program operation.

VFO       Stands for Variable Frequency Oscillator.  This is the settings

for one frequency, modulation, and other  parameters  that tune

the radio and set selected functions. Both the RadioCtl program

and  the  radio  being  run  have VFOs.  RadioCtl contains  two

separate VFOS,  one  active and  one standby.  Some radios also

have two VFOs.  However, RadioCtl will select only one of these

to be the active VFO. RadioCtl's active VFO is where parameters

are sent to or received from the Radio's active VFO.

MEMORY    Memory  is where  multiple  VFO  settings are  saved for future

usage  by  the active VFO.  Both RadioCtl  and the  radio  have

memories. RadioCtl has 1000 memories. The number of memories in

the radio varies from model to model. RadioCtl memories may NOT

always reflect the contents of the radio's memories.

CHANNEL   Channel  in this document refers to the currently active memory

number. A Channel number may either be a RadioCtl memory number

or a radio memory number.

BANK      This is a collection of Channels within RadioCtl. There are two

types of banks, SCAN banks and SEARCH banks.  SCAN  banks group

together  memory channels for  scanning.  SEARCH  banks  define

search parameters.  Although some of  the supported radios also

have  bank  definitions,  they  are usually  not  used  by this

program.

PROTOCOL  A  method of communication with  a particular radio. Each radio

type has a unique method  of transferring  information  between

the radio and the computer. The method used  by RadioCtl is set

by  the  user based on the Make and Model of  the  radio  being

controlled.

Introduction     <Radio Control Program for Windows>               Page 5

PROGRAM OVERVIEW:

RadioCtl provides  the user with a flexible way of controlling a radio.

It also provides a uniform interface for different types  of radios. This

is a high-level block diagram of the program:

┌────────┐

|search  |---┐      ┌──────┐

|banks   |   |      |stndby|

└────────┘   |      | VFO  |

|      └──────┘

|         |

|      ┌──┴───┐

┌────────┐   └----->|active|   serial  ┌────────┐

|memories|<---------|VFO   |---------->|  radio |

|        |--------->|      |<----------|        |

└────────┘          └──────┘  ┌------->|        |

^  ^                      | port   └────────┘

|  |                      |

|  |    ┌─────────┐       |

|  └----|channel #|-------┘

|       └─────────┘

┌──────┐

|scan  |

|banks |

└──────┘

Memory  channels  store  information  on frequency, modulation, squelch

count, duration, and other important parameters. The program provides for

a maximum of  1000 memory channels.  Information can be  directly entered

from the control panel, loaded from a text file,  loaded from the radio's

memory, or transferred from the program's VFO.

The VFO represents  a  single  frequency, modulation,  and  other radio

settings. These settings can be sent to the radio, or  stored in a memory

channel.  The VFO  can be set from the control panel direct input, loaded

from a memory channel, loaded from a search  bank, or set from the radio.

Current VFO settings can be swapped with a standby VFO for quick changing

of VFO information.

Scan  banks  provide  the ability  to  group  blocks  of  channels  for

scanning. Scan banks allow the user to define  up to 100 different blocks

of memory channels for selection.

Search  banks  provide  groups  of  frequencies  to  search.  Up to 100

different ranges of frequency, modulation and step value can be defined.

The Channel number provides selection of the program's memory channels.

It can also be used to address the memory channels of the current radio.

Radio control information is sent to and received from the radio by way

of  the  serial port.  The program provides for selection of up  to eight

different radios, each with  a  unique  set of operation characteristics.

The program can also provide simulation of  a  radio. The actual commands

to  the  radio  are defined by a protocol setting. The  program currently

supports protocols for KENWOOD, ICOM, AOR8000, AOR3000, BEARCAT, and OPTO

equipped radios. A LOCAL protocol is also provided which does not require

a radio to be attached.

If you are controlling the R7000, you  will  need  either the ICOM CI-V

interface adapter, or  the modified RS232  interface.  If you wish to use

the signal  detection  feature of this program, you will need to  use the

modified interface.

For the R-5000, you will need the IC-10 option kit. A modification must

be made to the R-5000 if you wish to use the signal detection feature  of

Introduction     <Radio Control Program for Windows>               Page 6

this program. Some  other  KENWOOD  radios  provide  for software  signal

detection. Although RADIOCTL can make use of this, the hardware detection

will increase the speed of scanning.

The  AOR3000,  Bearcat, and  OPTO equipped radios already have a  RS232

interface.  No  modification  is  needed  for  these  radios,  as  signal

detection  is  done in software.  The modification instructions include a

change to the AOR3000 to allow 9600 baud operation (from the default 4800

baud) .

For  the AOR8000, an  interface  is  also  needed.  The  OPTOLINK  from

Optoelectronics has been tested with this  program. This interface allows

the use of hardware signal detection for faster scanning.  This interface

will also work with ICOM radios as well. However, most interfaces for the

AOR8000 should  work with this program.  Although it has not been tested,

this program should work with the AOR8200, as the command set is similar.

The  Kenwood  TH-F6A uses a different  command  set  and interface then

earlier Kenwood radios. A special cable,  PG-4P  (available from Kenwood)

is  used to  connect  the radio  to the serial  port. Unfortunately, this

cable  mutes the speaker, so  monitoring  is  not  possible.  However,  a

special adapter can be built to overcome this limitation.

It is highly  recommended that the modified interface be used for  ICOM

and KENWOOD radios.  Most of the features of this program make use of the

detection   of  an  available  signal  (SQUELCH).  These  radios  do  NOT

currently have squelch  detection  as  part  of the  standard  interface.

Although this  program will work  fine  without  this  signal  detection,

operations such as SCAN and SEARCH will have no usability.

The interface circuit is a inexpensive, easy to build project.  All the

parts  are  readily available at any electronic store. Estimated cost for

each interface is $15. Plans  and instructions  for these interfaces  are

provided to you when you become a registered user.

Introduction     <Radio Control Program for Windows>               Page 7

STARTUP

INSTALLATION:

To install the program, Unzip all the  files into a temporary directory

and run the  program  SETUP from  the WINDOWS  RUN command.  The  install

process will allow  you  to  specify  a directory and copy all the needed

files to the appropriate places.

GETTING STARTED:

Once the install has  completed. select RadioCtl from the START PROGRAMS

menu.

When the program starts, you will be running with the "LOCAL" protocol.

This allows you  to  check out all  the functionality of RadioCtl without

having  to actually connect a real radio. Before using RadioCtl with your

particular  radio,  you  need   to   tell  the  program  what  radio  and

communication port that you are using. You do this by selecting the PARMS

menu, and selecting one of the eight RADIO_PARMS items.

Once  the  RADIO  PARMS  panel is  displayed, you can enter information

pertaining to your specific radio. First select the PROTOCOL based on the

type  of  radio that you have  connected.  When  the PROTOCOL is changed,

RadioCtl will  fill  in settings that should  work  with  the  radio  you

selected. You WILL need to select the Communications port that your radio

is connected to. If know that your radio uses different settings, you can

change them as you wish.  If you are not sure of your communications port

settings, use the LOCATE RADIO  button  to have  RadioCtl attempt to find

your radio.

Once the parms are  set for  your radio, you can  give these settings a

name  to make it  easy for  you  to  identify them. This name can be  any

combination  of  letters  and numbers up to 40 in length.  Press  the  OK

button to set the parms and return you to the main panel.

If all the  settings are  correct, you should be  ready  to control you

radio using RadioCtl.

SOME SIMPLE OPERATIONS:

*   Press the LOAD  button.  This will load your  radio's memory channels

into RadioCtl's memories.

*   Press  SCAN. This  will start scanning the frequencies  that you just

loaded.

*   While  scanning,  press  the  UP or  DN  buttons  next to the CHANNEL

display. This will change the direction of scanning.

*   Press MANUAL button. This stops the scanning operation.

*   Enter a frequency in the Active VFO Frequency Entry field (be sure to

press ENTER to  complete the entry). Your radio should  change to the

frequency you entered.

*   Left click  on  a VFO FREQUENCY  digit.  The  selected  digit  should

increase by one (or set to 0 if it  was  at  9).  If  the  VFO  being

changed is the Active VFO, then  your radio should also change to the

new frequency.

*   Right click on a FREQUENCY digit.  The selected digit should decrease

(or set to 9 if it was at 0) with similar results to the left click.

*   Select CHANNEL UP with either mouse button and hold the button  down.

The channels will  start incrementing and  your radio settings should

also change. Release the button to stop.

Startup          <Radio Control Program for Windows>               Page 8

THE DISPLAY

The  main  panel  shows you the status  of your radio,  as well as  the

status of the  program.  There  are numerous controls, and various status

fields. Controls are toggled by moving the mouse  cursor over the control

and  clicking.  Entry  fields require a ENTER or TAB key to complete  the

entry.  There are also separate windows for Messages, VFO status,  memory

channels, search and scan Banks, and the spectrum display.

You may  close any window (except the main panel) that you do not wish to

view.  To close a panel either click the "X" on the upper right corner or

click on the panel's ICON in the  upper left hand corner and select CLOSE

from the drop-down  menu.  To  open a  window you have previously closed,

use the  VIEW menu item on the main panel.  If  the Main panel is closed,

the program is terminated.

The following display windows are provided:

*   Main Panel

*   Messages

*   VFOA

*   VFOB

*   Memory Channels

*   Scan Banks

*   Search Banks

*   Spectrum

The Display      <Radio Control Program for Windows>               Page 9

MAIN PANEL

The  MAIN PANEL is  the controlling panel for the entire program. From

this panel, you can display other panels,  start and stop operations, and

terminate the program.

CHANNEL:

This  display  shows  the  current  channel number  being used  by  the

program.  Depending on the state of  the COMPUTER/RADIO  switch,  it will

represent  the  internal memory channels in the  program, or the  radio's

memory  channels. The memory channel can  be  changed  using  the  scroll

buttons to  the left of the display, using left  or right mouse button on

a  digit to  increment or decrement, or  entering a channel number in the

entry field under the display.  When  a memory channel number is changed,

and  the COMPUTER/RADIO switch  is  set  to COMPUTER,  the  corresponding

computer memory channel information is set to  the VFO, and to the radio.

If the COMPUTER/RADIO switch is in the RADIO position, the radio's memory

channel is  changed.  If the program is  in MANUAL mode, and the MEM SYNC

switch  is  selected,  changing  the  channel  number  will  display  the

corresponding Memory Channel information.

MEMORY SCROLL BUTTONS:

The memory channel  number  can be incremented or decremented using the

UP/DN  buttons  next to  the Channel  display.  The  mouse button used to

select the UP/DN button can be held for continuous increment or decrement

of FREQUENCY or CHANNEL.  A memory channel  digit  can also be changed by

right or left clicking the mouse over the digit.

OPERATION BUTTONS:

These buttons  perform  various  operations and set the program into of

three scanning modes. MANUAL, SCAN and SEARCH modes determine the primary

operation  of the  program. MANUAL mode is the default.  MANUAL,  SCAN  &

SEARCH  are "Sticky" operations. That  is,  once  selected, the operation

continues until  another  scanning operation is  selected.  All the other

operation buttons return to the scanning  mode that  was in effect before

the operation was selected.

MANUAL MODE:

Of three scanning modes. MANUAL mode is the  default. In this mode, the

program  will  continuously  monitor the  signal strength from the radio.

Depending on the radio, changes in the radio frequency or modulation  may

be logged in the active VFO display.

The  program  will switch to  manual  mode  if  one  of  the  following

conditions is met:

*   The MANUAL button is pressed.

*   SCAN mode detects no valid channels or banks.

*   SEARCH mode detects no valid  banks.

*   A new radio is selected.

*   Any radio or global parameter is changed.

SCAN MODE:

The  SCAN button starts the program  scanning  through selected  memory

channels.  When  a  signal is  detected, scanning will  stop, the  signal

strength  will   be  displayed  in  the  Signal  Meter,  and  information

(duration, squelch count, etc) will be logged in the memory channel.

When the signal stops, scanning  will continue.  Memory  channels  are

normally  incremented.  The direction of  the  scan  can  be  changed  by

Main Panel       <Radio Control Program for Windows>              Page 10

pressing the  CHANNEL  NUMBER scroll button to the  side  of the  channel

display.

Various Global switches and memory channel setting affect the operation

of scan mode. Global switches (on the main panel) overide any settings of

individual  memory  channels.  In   Addition,  PARMS  settings  for   the

individual radio also affect scanning.

Each RadioCtl Memory Channel has a number of  settings that are used to

setup the radio during a SCAN operations. Depending on the radio, some of

the settings may  NOT  be  used  or  set.  Table  3  shows the  functions

available for each radio.

In addition, several switches in the memory channel affect how RadioCtl

processes an individual channel. These switches are VALID, DELAY, TRUNKED

FREQ and RSM.

The  VALID switch is used during scan to identify  channels that should

be scanned. Any memory channel with the VALID switch off will NOT be used

in any scan operation.

If the DELAY switch of a channel is set, and a signal is detected after

setting  the radio with the channel's  settings,  then  the  program will

delay  for a number of seconds after  the  signal  stops  before resuming

scan. This DELAY value is set from the GLOBAL PARMS menu.

This switch identifies  the  memory settings as belonging to a  set  of

Trunked Frequencies.  If the Global Switch SCAN TRUNKED  FREQS is on, and

this switch is also on,  (and the VALID switch is set). This channel will

be included in scans. If  this switch  is  on and the  SCAN TRUNKED FREQS

global switch is  off, then this channel will  NOT be  used  in any  scan

regardless of the VALID switch setting.

If  the RSM  switch  (Memory  Channel)  is  selected,  and a signal  is

detected after  setting the  radio  with  the  channel's  settings,  then

scanning will resume after a set time period, even if the signal is still

present. This time period is set from the GLOBAL PARMS menu.

The Main Panel has a number of Global Switches that affect the way that

scanning is done.

The  MEMORY SYNC  switch  allows the Memory Channel  display to  follow

active channels. If this switch  is selected for SCAN or SEARCH mode, the

displayed Memory Channel  information will change to any  channel where a

signal is detected.  This allows you to automatically display the channel

information whenever the squelch breaks.  If this switch is off, then the

memory channel display will NOT change when a signal is detected.

Note:  If  you are editing  a memory channel  during Scan with the MEMORY

SYNC selected, you may loose  your edit if  the program  switches  memory

channel before the edit is complete.

If  the PRIORITY switch is  selected, The PRIORITY channel entered will

be  scanned  at  a  specific  interval.  If  a  signal is detected on the

priority channel, normal scanning stops, and the program switches  to the

priority  channel. Normal  scanning  will resume when the  signal  is  no

longer  present on  the  PRIORITY  channel.  The  interval  for  priority

checking is set on the GLOBAL PARMS panel.

Note:The  RESUME counter does not function  when stopped on the priority

channel. Scan or Search will resume from a priority channel ONLY when the

signal disappears.

RadioCtl  provides  100 Scan  banks (numbered  0-99)  that can  contain

ranges  of channels to scan.  These banks  allow you to group  a  set  of

channels based on some common criteria.  For example, channels 1-10 might

be  your  local  fire frequencies,  while  21-25  might  be  your  Police

Main Panel       <Radio Control Program for Windows>              Page 11

departments.

In  order  to  use these  banks,  the  BANKS  ON Global Switch  must be

checked.  If this box  is  checked  when  scanning, the  order of channel

selection is determined by the Scan Bank entries.  Bank channel selection

is done with the following logic:

1.  Set Bank number to 0.

2.  Step through Scan Banks until a  bank is  found with the VALID switch

set. Set the current memory channel number to the START value of this

bank.  If no valid bank found,  terminate the Scan (return to  Manual

Mode).

3.  If the  RadioCtl  memory  channel has  the  VALID  switch is set, and

either the  TRUNKED  FREQ switch is  off  or  the  Global Switch SCAN

TRUNKED FREQS is on, then Frequency, Modulation and other information

from the memory channel is use to tune the radio.

4.  If the radio was set, RadioCtl will check  for a signal.  If a signal

is detected, scanning will stop until either the signal goes away, or

Global Switch RESUME is set and the resume period has expired.

5.  If the current channel number is less than the current Scan bank STOP

number,  then  increment  the  current  channel   number.   Otherwise

decrement the current channel number.

6.  If the updated  channel number is outside the START and STOP range of

the current bank number, then increment the bank number and return to

step 2. Otherwise go back to step 3.

The RESUME switch  determines the action  after  the squelch breaks. If

RESUME is  off, scanning  will not continue until  the  signal  ends.  If

RESUME is on, scanning will resume after the  number of seconds specified

by the RESUME value. This value can be set from the PARMS panel.

NOTE:The  Global  RESUME  Switch  forces  a  resume  regardless  of  the

individual memory channel setting.

Normally,  RadioCtl  Memory  channels are updated with signal strength,

time-stamp, and other information when squelch breaks. If the USE RADIO'S

MEMORY switch is selected, then  Frequency  and  Modulation will  also be

updated when a  signal  is  detected.  Checking  this  box  disables this

logging, effectively  write-protecting the RadioCtl memory channels.

When the LOG TO FILE  switch is selected, all squelch breaks are looged

to the  current log file. This file is set with the FILE->CHANGE LOG FILE

NAME menu. If this switch is off, then  squelch breaks are only logged to

the MESSAGES panel.

The SCAN TRUNK  FREQS switch  determines  what happens with frequencies

marked as TRUNKED FREQ. If this box is checked, memory channels marked as

TRUNKED channels will be included in the scan.  Otherwise memory channels

marked as TRUNKED will be bypassed.

The  CTCSS  Enable  switch  determines  how a  SCAN  handles  the CTCSS

settings in a memory channel.  If this box is checked,  the SCAN function

will  honor  the  CTCSS  setting  on  the memory  channel  being scanned.

Otherwise, the CTCSS  setting  will  be  ignored.  If  CTCSS  scanning is

selected, then RadioCtl will stop on a  channel ONLY if there is a signal

AND  the CTCSS  setting for that  channel matches  the  CTCSS tone  being

received   by  the  radio.  Note  that  CTCSS   scanning  results  in   a

SIGNIFICANTLY  slower  scan  rate,  as  additional  information  must  be

exchanged between the computer when checking the signal.

Note: If the radio  does not support this feature,  this control will not

Main Panel       <Radio Control Program for Windows>              Page 12

be visible.

Under the Channel display is a button marked USE COMPUTER'S MEMORIES or

USE RADIO'S  MEMORIES. This is  a toggle button  that  affects where  the

frequency, modulation, and other settings for the  radio will come  from.

If  this  switch  is in  the  COMPUTER state, then the radio is  set from

RadioCtl's  current  memory  channel information.  If  set  in  the RADIO

state, then the radio is set from  whatever information is in the Radio's

memory channel.

In the  RADIO position,  scan is usually much  faster, as there is less

information  being  transmitted  between  the  computer  and  the  radio.

However, information in RadioCtl's memories  will not always reflect what

is kept in the radio's memories. Also, in the RADIO position, the channel

number range is determined by the minimum and maximum channel numbers for

the radio (set in the PARMS panel).

NOTE.The OPTO protocol does not provide for memory channel control. When

using the OPTO protocol, this  switch remains in  the  COMPUTER position,

and cannot be changed.

The  SPEED  CONTROL setting  determines how  fast each channel will  be

scanned. In the event that a scan  is too  fast  to  respond  to  certain

frequencies,  it can be slowed by  adjusting  the SPEED CONTROL.  Setting

the SPEED  CONTROL to a lower value causes  the  computer to wait on each

channel when  there  is no signal present. This gives the  radio time for

the squelch to detect a signal.

SEARCH MODE:

Search Mode allows you to  scan a  range of frequencies for action.  Up

to  100  different ranges can  be  programmed. Each range consists  of  a

starting frequency,  stopping frequency, increment value and  modulation.

In  addition,  searching  can start at the current VFO frequency  and  be

incremented or decremented by the current STEP value.

When a range is searched, the radio is set to the initial frequency and

modulation. If a  signal is detected, the frequency  and  modulation  are

recorded in the next  free RadioCtl  memory channel, and the VALID switch

is set  for  this channel.  If  this frequency already exists in a memory

channel, then COUNT, TIME and  SIGNAL values are  updated. If the  memory

channel  has  the  VALID  switch  off,  then the  signal  is ignored, and

searching  resumes with  the  next  frequency.  If there are no more free

channels available, then the message "SEARCH FULL" is displayed, and  the

information is NOT saved.

Various Global switches and memory channel setting affect the operation

of search  mode. Global switches (on the main panel) overide any settings

of  individual memory  channels. In  Addition,  PARMS  settings  for  the

individual radio also affect searching.

When Search  encounters  a  signal  on a given frequency, the operation

will  check to  see  if this frequency  is  already stored in one of  the

RadioCtl Memories.  If  the  frequency is found, then the VALID switch is

checked for that channel. If the switch is turned off, then the signal is

ignored, and the next frequency is selected.  If the VALID switch  is on,

information for that frequency is updated in the RadioCtl Memory Channel.

When a signal has been detected, and then goes away, RadioCtl will wait

a  short period  before resuming Search if the memory channel that it has

logged information to has the DELAY switch set.  The DELAY time value can

be set from the GLOBAL PARMS menu.

If  the  RSM  (Memory  Channel) switch  is selected,  and  this  Memory

Channel  is used for logging a signal, then the search  will resume after

a set time period, even if the signal is still  present. This time period

is set from the GLOBAL PARMS menu.

Main Panel       <Radio Control Program for Windows>              Page 13

The Main Panel has a number of Global Switches that affect the way that

searching is done.

The MEM SYNC switch allows the Memory Channel display to follow  active

channels.  If this  switch  is  selected,  the  displayed  Memory Channel

information will change to any channel where information is being logged.

This allows you to automatically display the channel information whenever

the  squelch  breaks.  If  this  switch  is off, then  the memory channel

display will NOT change when information is logged.

Note:  If  you are editing a memory channel during Search with the MEMORY

SYNC  selected,  you may loose your edit  if the  program switches memory

channel before the edit is complete.

The BANKS ON switch determines the  logic for the Search operation.  If

this switch is off, the Search operation performs as follows:

1.  Send current active VFO Frequency, Modulation and other  applicable

settings to the radio, and wait a short period for a signal.

2.  If  a signal is detected, locate a memory  channel that  has the same

frequency as the current  VFO, or a memory channel that does not have

any data (frequency = 0  ).  If the DON'T UPDATE MEMORY Global switch

is not set, update information (modulation, signal strength, etc.) to

this memory channel.

3.  If  a  signal is detected, wait until the signal goes away,  or until

RESUME timeout (if Global RESUME switch set).

4.  Update active VFO Frequency by  the  active  VFO  step value.  If VFO

Frequency  is  greater  than the  maximum frequency allowed for  this

radio, then set  VFO frequency  to  the minimum frequency allowed for

this radio. If  VFO  is  being decremented,  and VFO is less than the

minimum frequency allowed for this  radio, then set frequency  to the

maximum frequency allowed.  Maximum  and minimum frequencies are  set

in the PARMS menu for the particular radio.

5.  If  the UP or DN buttons are pressed  (on the  VFO panel  next to the

frequency display) then change the direction of the search.

6.  go back to step 1.

If the BANKS ON switch is on, then the Search operation is as follows:

1.  Set Bank number to 0.

2.  Step  through Search  Banks until a  bank  is  found  with  the VALID

switch set.  If  no  valid  bank  found, terminate search (return  to

Manual Mode).

3.  Set the  current VFO frequency to the starting frequency of the bank,

Set  the current VFO  modulation  to the modulation  specified in the

bank.

4.  Send  current active  VFO Frequency, Modulation and  other applicable

settings to the radio, and wait a short period for a signal.

5.  If  a signal is  detected, locate  a memory channel that has the same

frequency  as the current VFO, or a memory channel that does not have

any data  (frequency = 0 ).  If the DON'T UPDATE MEMORY Global switch

is not set, update information (modulation, signal strength, etc.) to

this memory channel.

6.  If  a signal is detected, wait until the signal goes away,  or  until

RESUME timeout (if Global RESUME or RSM on channel set).

Main Panel       <Radio Control Program for Windows>              Page 14

7.  Update active VFO Frequency by the INCR value for the current  search

bank.  If the new  frequency  is outside the range specified  by  the

START and STOP of the current  search bank, increment the bank number

and go  back to step 2. Otherwise  set  the  current VFO frequency to

the new frequency and go to step 4.

Note: If START frequency less than STOP frequency then frequency will

be  incremented  by  the  value  in  INCR.   Otherwise  it   will  be

decremented.

The  RESUME  switch determines the action after the squelch breaks.  If

RESUME is off, searching  will  not  continue until  the  signal ends. If

RESUME is on, scanning will resume after  the number of seconds specified

by the RESUME value. This value can be set from the PARMS panel.

NOTE:The  Global  RESUME  Switch  forces  a  resume  regardless  of  the

individual memory channel setting.

Normally,  RadioCtl Memory  channels are updated  with signal strength,

time-stamp, and other information when squelch breaks. If a new frequency

is  found  during a search,  an  empty  memory channel is  used to  store

Frequency  and  modulation.  Checking  this box  disables  this  logging,

effectively  write-protecting the RadioCtl memory channels.

When  this  switch  is selected, all  squelch breaks  are logged to the

current log file. This  file is set with the FILE->CHANGE  LOG  FILE NAME

menu. If this switch  is off, then  squelch breaks are only logged to the

MESSAGES panel.

The  PRIORITY switch does NOT have any effect on Search Mode.  Priority

is not supported during a search operation. Likewise, the  COMPUTER/RADIO

switch is also ignored. Search Mode always uses the computer's VFO.

The SPEED CONTROL setting  determines  how fast each frequency  will be

monitored.  In the event that a search is too fast  to respond to certain

frequencies, it can be  slowed by adjusting  the SPEED  CONTROL.  Setting

the  SPEED  CONTROL to a lower value causes the computer  to wait on each

frequency when there is no signal present. This gives the radio time  for

the squelch to detect a signal.

LOAD:

When this operation is selected, all of the radio's memory channels are

loaded  into RadioCtl's Memory  Channels. The load  starts  at  the first

channel entered in the START LOAD/SET field, and continues to the channel

specified in  the  STOP LOAD/SET field.  The contents of computer  memory

channels are  overwritten  with the  new information.  Channels  that are

locked  out will have  the VALID switch reset.  The count,  duration  and

strength  fields  are  reset  to  0.  If  the  radio  provides  for  text

information, that will be loaded into  the  service field.  Any field  or

setting  not  supported by  the radio will be set to  the default  state.

When all channels specified in the START/STOP range have been loaded, the

program will return to the previous scan mode (MANUAL/SCAN/SEARCH).

For a LOCAL radio, a set of random settings will be set for each channel.

Note: The OPTO protocol does not provide for memory channel control. When

using the OPTO protocol, this switch is disabled.

Note: This control will be disabled if the DON'T UPDATE MEMORY  switch is

selected.

Main Panel       <Radio Control Program for Windows>              Page 15

SET: When  this  operation  is selected, the contents  of  the computer

memory channels are  stored  into  the  radio's memory  channels. The set

starts at the  first  channel entered in  the  START  LOAD/SET field, and

continues  to the channel  specified  in  the STOP  LOAD/SET  field.  The

contents  of  radio   memory  channels   are  overwritten  with  the  new

information. A  zero  frequency  will  cause  the  memory  channel  to be

cleared.  For a LOCAL radio, each memory channel will  be  transferred to

the VFO in turn.

When all  channels  specified in the START/STOP range have been  set, the

program will return to the previous scan mode (MANUAL/SCAN/SEARCH).

NOTE.The OPTO protocol does not provide for memory channel control. When

using the OPTO protocol, this switch is disabled.

MEM->VFO:

This button will transfer the  contents of the Memory Channel indicated

by the Channel  display,  to  the VFO. If the COMPUTER/RADIO switch is in

the COMPUTER position, the Computer's memory channel will be transferred.

If  the RADIO  position  is selected, the information will be transferred

from the radio's memory. For a LOCAL  radio, the  input will be a  random

frequency and modulation. This  operation  is the same as a LOAD  for one

channel.

VFO->MEM:

This  button  will  transfer  the  contents  of the  VFO frequency  and

modulation to the Memory Channel indicated by the Channel display. If the

COMPUTER/RADIO switch is in the COMPUTER  position, the Computer's memory

channel will be updated.  Count, Strength, and Duration fields are reset.

The  Service string is also cleared.  If the RADIO position  is selected,

the information will  be transferred to the radio's memory.  For  a LOCAL

radio, no data  is transferred.  This operation  is the same as a SET for

one channel.

Note:  If  the  DON'T  UPDATE   MEMORY  switch   is   selected,  and  the

COMPUTER/RADIO switch is in the COMPUTER position, then this control will

be disabled.

RADIO->SEARCH:

Some  of the supported radios  have search  memories that can be loaded

into the RadioCtl program.  When this button is  pressed,  those memories

are loaded into the corresponding SEARCH BANK channels.

Note: If the currently selected radio does not support this feature, this

control will not be visible.

SEARCH->RADIO:

Some  of the supported radios have search memories that can be set from

the RadioCtl program. When this button is pressed, those memories are set

from the corresponding SEARCH BANK channels.

Note: If the currently selected radio does not support this feature, this

control will not be visible.

START LOAD/SET:

The starting channel  number for a load or  a set operation  is entered

here.

STOP LOAD/SET:

The ending  channel number  for a load or a  set operation  is  entered

here.

SWAP VFO  BUTTON:

RadioCtl  provides  two  sets  of  VFO  settings.  Each  set includes a

frequency,  modulation,  step, attenuator,  notch,  preamp, amsync, tone,

AGC, antenna, noise blanker and bandwidth settings.  The Swap VFO  button

can be used to quickly switch the program between either of the two VFOs.

Main Panel       <Radio Control Program for Windows>              Page 16

Note: For radios  that have  multiple VFOs, RadioCtl  will always use the

first VFO regardless of the program's VFO selection.

SPEED CONTROL:

For SCAN or SEARCH mode, the speed control determines the time between

each  channel or frequency. When the control  is all the way to the left,

the time between each setting is minimal.  If  the slider is moved to the

right, the program  will  delay for  a certain  time  before changing the

setting on  the  radio. During  this time, the  program will monitor  the

radio for a signal.  The SPEED control  allows you to reduce the  SCAN or

SEARCH speed to give the radio more time to respond to a signal.

GLOBAL SWITCHES:

At  the right of the Main  Panel is a  column  of  switches that affect

various  operations.  These  switches  determine  what  memory  is  used,

priority, resume, and other settings.  If the switch has  a "check", then

the switch is ON. Otherwise the switch is off.

MEMORY SYNC:

The  MEMORY  SYNC switch allows  the Memory Channel  display to  follow

active channels. If  this switch is selected for SCAN or SEARCH mode, the

displayed  Memory  Channel information will change to any channel where a

signal is detected. This allows you to automatically  display the channel

information  whenever the squelch  breaks.  If  the program  is in MANUAL

mode, then the memory channel display will change in sync with the memory

channel number.

PRIORITY:

Priority scanning allows  a selected channel to be tested  for a signal

once  every interval. The channel number to select  as a Priority channel

is entered  in the  entry field at  the  bottom  of  the  frame.  If  the

Priority  switch  is  ON,  then the priority channel will  be  used.  The

priority checking interval can  be set using the  PARMS  panel.  Priority

only works during SCAN mode.

BANKS ON:

When in SCAN or SEARCH mode, the program can  make use of banks of scan

or  search information. These  banks allow  the grouping  of channels  or

frequencies for the SCAN mode. The  Banks  On switch allows the scan mode

to  use  the bank associated  with  the  SCAN  mode.  When selected,  the

indicator  in the  status frame  will display what bank number (00-99) is

currently being used.  See  the discussion of  SCAN and SEARCH mode for a

complete overview of bank usage.

RESUME:

The  Resume  switch  will allow  the Scan or Search  to continue when a

signal is present.  This is useful  when  a  Scan  or Search encounters a

constant signal. With Resume on, the signal is ignored after a set amount

of time. This  time  interval is set using the Parms panel. The indicator

in the Status panel shows approximately  how many seconds  are left until

the SCAN/SEARCH resumes.

Note:This  switch is a global setting. If on,  the  RSM  switch for each

memory channel is ignored.

DON'T UPDATE MEMORY:

This switch acts as a write-protect for RadioCtl's memories during SCAN

or  SEARCH  operations. If this box  is  checked,  then  scan  or  search

operations will NOT update RadioCtl memories when the squelch breaks.

Note: Selecting this switch disables the LOAD operation button.

Main Panel       <Radio Control Program for Windows>              Page 17

LOG TO FILE:

When this  switch  is selected, all actions  are logged to a data file.

This  includes  mode  changes and squelch breaks.  A time  stamp is  also

logged with each  event.  This file is  cumulative,  and is not erased by

this program.  The  name of the logging file can be changed with the FILE

menu selection.

LIGHT ON SIGNAL:

If this box is checked,  the  FREQUENCY display will change  to a green

background whenever a signal is detected. If the connected radio supports

this feature, the light on the radio will also light for a signal.

Note: This switch is disabled for the Bearcat 895.

SCAN TRUNK FREQS:

If  this box is checked, then memory channels marked as TRUNKED will be

included as part of  a SCAN operation. If the  box is not  checked, these

channels will be bypassed.

CTCSS ENABLE:

If this box is checked, the  SCAN function will honor the CTCSS setting

on the memory channel being scanned. Otherwise, the CTCSS setting will be

ignored.  This is only available for radios that support  CTCSS scanning.

For unsupported radios, this control will not be displayed.

STATUS CONTROLS:

The center frame on the Main Panel contains several status controls and

displays. The controls allow you to select a specific radio to test,  and

adjust signal  detection.  The status displays show information about the

connected radio, bank selection, resume time and signal strength.

RADIO SELECTION:

Up to  eight different  radios can  be  defined  to the program.  These

radios can  have  different BAUD rates, Com  ports,  protocols, etc. Each

radio is assigned a name. You  can change the name (and other parameters)

for a  given radio  from the Parms menu. Radio Selection is  a  drop-down

selection field. Click on the  right  button  to display the  radio list.

Double click on a radio  to select it. The Baud and Com port used will be

displayed below to the Radio Selection control.

The connection status of the radio is displayed  below the BAUD and COM

status. This field can be double-clicked to  force a reconnection.  NOTE:

When a new radio is selected, or the current radio is reconnected, Manual

mode is forced.

SIGNAL METER:

The signal  is a 12 step indicator corresponding to the signal strength

being detected. Depending on the  radio, the meter may reflect the actual

signal strength, or just the presence of  a signal. The signal level of 9

is used  for  those radios without the ability to read  the actual signal

strength.  For a  LOCAL radio, clicking any of the signal display buttons

will turn on the signal at that strength. For  all radios, the OFF button

will temporarily turn off the signal. If scanning or searching, this will

cause  the radio to resume scan or search.  For  LOCAL operation, the OFF

button is the only way to turn off the signal detection.

When  not  running in LOCAL  mode, the signal  buttons can be  used  to

define a  minimum squelch  trigger. Clicking  on  a button will cause the

program to ignore any  signal  at  or below  that signal  level. When the

squelch minimum is  active, the  signal limit level buttons are displayed

in green.

NOTE:This  feature  is  not  useful for the  ICOM and KENWOOD radios, as

these radios do not have the ability to determine signal strength values.

Main Panel       <Radio Control Program for Windows>              Page 18

MAIN PANEL MENU:

The  menu on the top of the  main panel provides additional  functions.

These functions provide the ability to load and save information to disk,

customize your radio setup, and  select various panels for  viewing. Menu

items  can  be selected by  placing  the mouse cursor over the  item  and

pressing  the right mouse button.  Menu items  can also be  selected from

the keyboard using  the underlined letter.  For the primary menu, the ALT

key  must also be pressed.  For  drop-down menus, only the key itself  is

need to select an item.

FILE MENU:

The File menu provides the ability  to  load or save the current memory

channel, scan banks, and search  banks to or from a text file.  The  file

format for this file is  described in the DATA  FORMAT section.  The scan

information can  be saved  to or loaded from many different files. When a

file is loaded, only those memory channels, scan bank channels, or search

channels that are  included in the file  are changed.  For  a  save,  all

Memory Channels, Scan Banks and Search Banks are written.

A new DBASE  file  can  also  be selected from this panel. This file is

used for service lookup  operations.  The  format  of  this  file is also

discussed in the DATA FORMAT section.

The DEBUG item enables or disables the logging of debug data. This data

is  used  primarily for logging  information for troubleshooting  program

problems. The information is logged to  the file DEBUG.DAT. This file can

grow to be quite large, as an extensive amount of  information is written

to this file.

The File menu also contains the Quit for the program.

VIEW MENU:

This  menu item enables the display of various panels. These panels are

for VFOA, VFOB, Messages, Frequency database, Scan and Search  banks, and

the spectrum display.

CLEAR EVERYTHING:

This operation will clear all Memory Channels, Search Banks, Scan Banks

and the VFO. In addition, the program will be reset to Manual mode.

PARMS:

For the program, and for each radio, there are a set of parameters that

need to be defined. Although the program sets certain  defaults, the user

will most  likely  wish  to  change these to suit  individual  needs  and

tastes.

GLOBAL PARMS:

Various  parameters determine the operation of  the  program.  For  the

most  part, these  parameters  are  set once  when you first install  the

program. This panel allows you to change program operation parameters.

PRIORITY:

This  is  the time (in seconds) between  priority checks. This value is

used during SCAN when PRIORITY is selected.

DELAY:

The number of seconds that the  program will wait on a  channel after a

signal disappears. This value only has affect if  the DELAY switch is set

on the current memory channel.

Main Panel       <Radio Control Program for Windows>              Page 19

RESUME:

If  RESUME  is  selected, this is  the  number of seconds that SCAN  or

SEARCH will continue after a signal is detected.

FONT:

The  frequency,  Step  and Channel Fonts are  initially set to  use the

TRANSITOR type font. This font was selected  to look like a Seven Segment

display  found on most scanners.  However, tastes  vary, and  a different

font for these displays may be desired. This  drop-down box allows you to

select  any  FONT  that is currently  installed in  the  system  for  the

displays. When the font is changed, the sample box (next to the FONT SIZE

selection) will change to reflect the new setting.

Note:The TRANSISTOR font does not always get selected for use by Windows.

If the font does not  seem  to  be available, go  to the Windows  Control

Panel, Select  Fonts,  locate the  font "Transistor Regular",  and double

click on the file. Press the DONE key  to  register  the font.  This font

should now be available in RadioCtl.  When the font  size is changed, the

sample box will change to reflect the new setting.

FONT SIZE:

The size of the Frequency,  Step and Channel Fonts can also be adjusted

to suit taste. Depending  on the font chosen, the size of  the  font  may

need  to be adjusted to allow the  entire digit to display. Any size from

10 to 30 can be selected.

DISPLAY UNUSED AUX:

On  the VFO and MEMORY panels,  Aux values are displayed. These  values

represent functions that may not be available based on the current radio.

If this  box is NOT  checked,  functions  that are not  applicable to the

current radio selected will NOT be displayed on these panels.

FILE SAVE FORMAT:

There are two  formats  that frequency information can be saved in. The

long format logs a  frequency  record that includes all keywords with  no

abbreviation.  When logged  in  this  manner,  each field is aligned with

field  above and below  it. This format is much  easier  for  reading  or

editing with a text  based  editor. However, it  requires more disk space

than the short form. The long form is the program default.

The short format abbreviates keywords and only logs essential  fields.

This format conserves disk space, but is harder  to read or change with a

text editor.

LOAD FREQUENCY DBF ON START:

Normally  when  the  program  starts,  the   DBF   file  that  contains

information for service look-up is  loaded automatically. If this file is

fairly large, it can take a bit of time to load. If you don't wish to use

this file,  or you want to  load it later (using  the  FILE menu) you can

uncheck this box.

RADIO PARMS:

The  program provides settings for up  to eight radios.  Each radio can

be setup with  different operation characteristics.  For  the  most part,

these  parameters  are  set  once  when  you first install  the  program.

However, you may need to update these parameters if you connect different

radios.  Except for Name, Baud rate  and  Com  port, the defaults for the

particular radio types should  only be changed if a non-standard radio is

being connected.

RADIO NAME:

This is the  name that you give the radio. There are no restrictions on

the name, other than it is limited to 30 alpha-numeric  characters.  This

name is used in the Radio Selection control on the main panel. It is  NOT

required that each radio have a unique name,  but it makes  it easier for

the user to select the right radio if they are.

Main Panel       <Radio Control Program for Windows>              Page 20

COM PORT:

This is the com port that your radio is attached to. The program allows

you to select com 1 thru com 4.

BAUD RATE:

The baud setting  for this radio.  Most  radios  can  be  configured to

accept baud  rates  from  1200  to  9600  baud. The program will set  the

default  baud  rate  when  a  new  protocol is  selected.  If  your radio

requires a different baud rate, it can be changed with this control.

DATA BITS:

Most  of the radios supported work with an 8  bit data format.  If  you

have a radio that requires 7 bits, this can be changed.

STOP BITS:

Most of the radios supported use 2 stop bits. If needed, you can change

this to 1 stop bit.

FLOW CONTROL:

Flow  Control  provides  the ability to stop  and start  communications

based  on the  ability of  the  radio  and computer to accept data.  Flow

control requires the  use of additional control  lines  (RTS/DTS).  These

lines are  used as signals to indicate to  the radio  and the computer to

start and stop sending information. When a new protocol is selected, this

box will be set based on the radio type selected.

MIN CHANNEL:

This is the  first memory channel for the radio (0 or 1).

MAX CHANNEL:

This field defines the maximum memory channel in the radio.

MIN FREQUENCY:

This is the minimum frequency that the radio will accept.

MAX FREQUENCY:

This is the maximum frequency that the radio will accept.

SIGNAL DETECT:

There  are three  ways to detect a squelch  break. The first is using a

spare  signal on the  RS-232  port. This signal  detection is part of the

modified  interface, and also provided with OPTO equipped radios. This is

referred to as HARDWARE detection.

The second detection  method involves using a  command  to the radio to

detect  squelch  break. This  is referred to as SOFTWARE  detection. This

method also uses a command to the radio to determine the signal strength.

Not all radios can use software detection.

The third method is a  combination of HARDWARE and SOFTWARE.  With this

technique, the  signal line is  checked  for  squelch break, and a signal

strength command  is issued  ONLY if  the squelch  break  signal has been

detected. This  method is the most efficient for scanning speed. However,

it can only be used with radios that have both available.

Please note that to make use of the  HARDWARE, or HARDWARE and SOFTWARE

detect methods, either the modified interface or the  OPTO interface must

be present.

PROTOCOL:

This field defines  the  type of radio  (protocol). There are currently

nine  types  of radios supported. ICOM, KENWOOD, AOR3000, AOR8000,  OPTO,

DRAKE,  BEARCAT, TH-F6A and  LOCAL. Other  radios  may be  added  in  the

future.  When this value is changed, all other fields (except RADIO  NAME

Main Panel       <Radio Control Program for Windows>              Page 21

and COM PORT) are updated with values that RadioCtl believes are correct.

MODEL:

Some radios  have  small  variations on the  command  set based on  the

model. For those  radios,  the MODEL field will  be displayed.  Use  this

field to identify to RadioCtl the  correct  model  of radio that you  are

using.

ADDRESS:

ICOM and OPTO radios require an address. For the R-7000, the address is

normally  8. If you connect  a different ICOM or OPTO radio, or you  have

more than one ICOM radio connected, you can change this address to match.

In  addition, the computer (RadioCtl)  must also  have  an  address. This

usually defaults to 0, but can be changed.  These parameters will only be

displayed for ICOM or OPTO radios.

VFO CHANNEL: Some radios (most notably  the BEARCAT series)  do NOT

have a separate  VFO. To make up for this, RadioCtl will  use  one of the

radio's  memory  channels as a "virtual"  VFO.  This  will  overwrite the

contents of this channel whenever VFO  data is sent  to  the  radio.  The

default channel is 300, but can be set to any channel from 1 to 300.

TIMEOUT:

All radios  require a delay  between each packet sent.  In general this

delay  should be between 60 and 90 milliseconds.  If you are seeing error

messages  that  indicate overflow or  bad data from the radio, you should

increase this  value.  Setting  this  value too high  will slow  down the

maximum scanning speed. However,  setting this value  too  low will cause

transmission errors.

ENABLE EXTENDED COMMANDS:

For certain radios an extended command set is provided.  These extended

commands  provide additional radio  settings  (such as  Bandwidth,  CTCSS

tones, etc.).  If this switch is  visible, then  the radio selected  will

support  these extended commands.  Note that  enabling extended  commands

will increase the amount of time scan  or  search  takes  per channel, as

more information must be transferred.

DEFAULT SETTINGS BY PROTOCOL:

The following table shows  what defaults are  used  for  the  protocol

selected:

Main Panel       <Radio Control Program for Windows>              Page 22

TABLE 1-Protocol default settings

┌────────────┬───────┬──────┬─────────┬────────┬─────────┬──────┐

│ parm       │ LOCAL │ ICOM │ KENWOOD │ AOR3000│ AOR8000 │ OPTO │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Baud Rate   │ n/a   │9600  │ 4800    │ 4800   │ 9600    │ 9600 │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Data Bits   │ n/a   │8     │ 8       │ 8      │ 8       │ 8    │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Stop Bits   │ n/a   │1     │ 1       │ 2      │ 1       │ 1    │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Flow Cntrl  │ n/a   │Off   │ On      │ On     │ Off     │Off   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Min Channel │ 0     │1     │ 0       │ 0      │ 0       │ 1    │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Max Channel │ 999   │99    │ 99      │399     │ 999     │ 99   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Min Freq    │10hz   │25mhz │10hz     │10hz    │ 50hz    │25mhz │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Max Freq    │1999mhz│999mhz│999mhz   │2035mhz │1899mhz  │999mhz│

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Signal Det  │ n/a   │Hardwr│Hardwr   │Softwr  │Softwr   │Hardwr│

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Radio Addr  │ n/a   │08    │n/a      │n/a     │ n/a     │ 08   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Comptr Addr │ n/a   │00    │n/a      │n/a     │ n/a     │ 00   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Timeout     │ 90    │90    │90       │90      │ 90      │ 90   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│VFO Chan    │ n/a   │n/a   │n/a      │n/a     │ n/a     │ n/a  │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Extended?   │ n/a   │Off   │Off      │n/a     │ Off     │ n/a  │

└────────────┴───────┴──────┴─────────┴────────┴─────────┴──────┘

Main Panel       <Radio Control Program for Windows>              Page 23

TABLE 1- Protocol default settings (continued)

┌────────────┬───────┬──────┬─────────┬────────┬─────────┬──────┐

│ parm       │ DRAKE │ THF6A│ BEARCAT │ AOR5000│         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Baud Rate   │ 9600  │9600  │ 9600    │ 9600   │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Data Bits   │ 8     │8     │ 8       │ 8      │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Stop Bits   │ 1     │1     │ 1       │ 2      │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Flow Cntrl  │ Off   │On    │ Off     │ On     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Min Channel │ 0     │0     │ 1       │ 0      │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Max Channel │439    │411   │ 300     │ 999    │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Min Freq    │10hz   │10    │ 29mhz   │ 10hz   │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Max Freq    │29mhz  │1999mz│ 955mhz  │ 2035mhz│         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Signal Det  │Hardwr │Softwr│Software │Software│         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Radio Addr  │n/a    │n/a   │n/a      │n/a     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Comptr Addr │n/a    │n/a   │n/a      │n/a     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Timeout     │90     │30    │30       │90      │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│VFO Chan    │n/a    │n/a   │300      │n/a     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Extended?   │n/a    │n/a   │n/a      │n/a     │         │      │

└────────────┴───────┴──────┴─────────┴────────┴─────────┴──────┘

Main Panel       <Radio Control Program for Windows>              Page 24

VFO PANELS

There are two  identical VFO displays. Each one shows the status of the

affected VFO, and allows you to change the current settings. Whenever one

VFO is active, the other  is considered to be in standby. This allows you

to make multiple changes the inactive VFO, and then send those changes to

the radio with one button  press.  Changes made to the active VFO will be

sent immediately to the radio.

VFO FREQUENCY:

This  display   shows  the  current  frequency  of  the  program.  This

frequency  may or may  not reflect the state of the  attached radio.  Any

frequency information  sent to the radio is displayed.  The value of this

display  can be changed using  the scroll  buttons to  the  left  of  the

display or by using the mouse buttons when the mouse pointer is  over the

digit displayed. Clicking the  left button will increase a  digit by one.

Clicking the right button will  decrease  a digit by one.  Holding either

button for about 1 second  will  cause  the  action  to repeat until  the

button  is released.  A frequency value can also be entered in the  entry

field  under  the  display. If  a value is entered,  changed  with  mouse

buttons, or scrolled with the VFO scroll bar for the ACTIVE VFO,  the new

value is sent to the radio.

MODULATION:

This  is  a combination display and selection box. It shows the current

modulation used by the program.  This may or may not reflect the state of

the  radio.  A modulation can be set by pressing the drop-down control on

the right of  the box, and double-clicking on  the desired modulation. If

the modulation on the ACTIVE VFO is changes, the corresponding modulation

will be changed on the radio.

NOTE:Some radios may  not support the modulation selected.  In that case,

the appropriate modulation will be selected, and  the display will change

to reflect this.

VFO STEP VALUE:

This  display  shows  the  current step  value that will be used  when

incrementing  or  decrementing  the  VFO.  This  is  done  during  search

operations, and when the UP/DN frequency buttons are pressed.  This value

can be set by entering the desired step value in  the  entry field  below

the Step digits. Alternately,  left  or right mouse clicks can be used to

increment or decrement individual digits.  During  a  bank  search,  this

field will be loaded from the selected bank.

Note: The STEP  value displayed will most  likely  NOT  reflect  any STEP

value set in the current radio.

Note: Setting the step value to 0 will keep SEARCH from working.

DIGIT SCROLLING:

The VFO Step Value and VFO frequency  displays can be changed using the

left  or right  mouse button to  increment or  decrement the  digit.  The

mouse pointer selects the digit to change. The left button will increment

that digit, while the right button will decrement.  If a digit is a 9, an

increment sets the digit to 0.  Likewise, if a digit  is a 0, a decrement

will set the digit to 9. Either  mouse  button can be held for continuous

repeat of  an increment/decrement.  This  allows quick changing of any of

the information in the display. Changes to the Frequency  display will be

reflected to the attached radio.

Note:  Because the program will send  data to the  radio when  a digit is

scrolled, there may be a delay involved between each digit "click".

VFO Panels       <Radio Control Program for Windows>              Page 25

TRUNKED FREQ:

If this button is selected,  this frequency is  identified as belonging

to a trunked group of frequencies. The primary usage of this switch is to

allow setting of  the  TRUNK  switch  in  radios such as the BEARCAT that

support  trunk  tracking.  This  switch is also  used for  bypass  during

scanning operations.

VFO ACTIVE:

The VFO Active indicator  shows if the VFO in question is the currently

active VFO. If it is, the indicator will be Red, and will be indicated by

"VFO Active". If the VFO is NOT active, the indicator will say "Make This

VFO Active". If you click on this indicator for the inactive VFO, it will

make it the  active  VFO.  This  operation is identical  to  the SWAP VFO

button on the main panel.

AUX CONTROLS:

A  number  of  VFO  settings  are  enclosed  in a box identified as AUX

CONTROLS. Depending  on the protocol selected, and the DISPLAY UNUSED AUX

option setting,  these controls may or may not be visible. If the DISPLAY

UNUSED AUX option (set in the GLOBAL OPTIONS menu) is not  selected, only

those controls  that are meaningful  to the radio being used are visible.

In addition, some controls will have different descriptions for different

radios. Please see table 3 for a list of  what controls are available for

what protocol.

ATTENUATOR:

Some radios have a built in attenuator.  This  can be  switched  on  to

reduce  overload from a  strong signal. Pressing this  switch will toggle

the current  attenuator setting  in the  radio. When in SCAN  mode,  this

switch will be set by the currently scanning memory channel's  attenuator

setting.

NOTCH|TONE ENABLE:

For certain radios, a notch filter is available. For these radios, this

switch  enables  or disables the radio's notch filter.  For other radios,

this  switch enables/disables a descrambler tone.  If  supported  by  the

radio, a CTCSS or  descrambler tone may be set using the  Tone  drop-down

box, and enabled by this switch.  See Table 3 for a breakdown of how this

switch is defined for each radio.

AMSYNC|RECORDER:

For certain radios, an AM synchronous detector is available.  For these

radios, this switch enables or disables the radio's preamp function.  For

the BEARCAT  protocol, this  function  enables  or disabled the  RECORDER

function.

PREAMP:

For  certain  radios, a pre-amplifier is available.  For these  radios,

this switch enables or disables the radio's preamp function.

TONE|NOTCH FILTER VALUE:

For certain radios,  a CTCSS  or  descrambler tone  is  available.  For

these radios,  this  dropdown box selects  the  tone value. When the Tone

switch  is enabled,  the  selected Tone value will be sent to  the radio.

For other radios, this switch enables or disables a Notch filter.

AGC:

For  certain  radios, an  Automatic Gain Control is available For these

radios, this dropdown box selects the AGC level.

ANT:

For certain radios, an antenna selection is available For these radios,

this dropdown box selects the particular antenna.

VFO Panels       <Radio Control Program for Windows>              Page 26

NOISE BLANKER:

For  certain radios,  a  noise  blanker function is available For these

radios, this dropdown box selects the noise blanker function.

BANDWIDTH:

For  certain  radios, a bandwidth  function  is  available.  For  these

radios, this dropdown box selects the bandwidth of the radio.

Note: The Bandwidth control is kept in sync with the Modulation selection

when the modulation type is FM.

TONE 2:

For  certain  radios, a  second tone function is available.  For  these

radios, this  dropdown box  selects the value  of that tone.  The  switch

below this control enables or disables this function.

VFO Panels       <Radio Control Program for Windows>              Page 27

MEMORY CHANNEL PANEL

RadioCtl  Memory  channels  contain information  for a  specific  radio

setting.  Each channel contains  information on Frequency, Modulation and

other parameters to be sent to (or received from) the radio. In addition,

information on signal  strength, number of  squelch  breaks  and  service

information is also displayed.

Each program channel displayed has several  fields associated  with it.

Some of these  fields are entry fields that  information can be  entered.

Other  fields  are  toggle switches that  can be turned on and off.  Note

that entry field information  is NOT accepted until  ENTER  is pressed, a

TAB key is pressed, a new entry  field is selected with the mouse, or the

channel display is changed with the scroll bars.

CHANNEL NUMBER:

This  field  determines which  memory  channel  is  being  displayed or

changed.  The  memory  channel can  be  changed  by  direct entry in  the

channel  field,  or using the scroll  bars  to the  left of  the  channel

number.  If entering  the channel number, keep in mind that the data will

NOT be finalized until you press ENTER or a TAB key.

VALID:

The VALID field determines whether or not SCAN  and SEARCH will use the

frequency  currently  entered  into  that channel.  For  SCAN  mode,  the

program will not scan the memory channel  if VLD is OFF.  For  the SEARCH

operation,  a frequency entered in a  memory  record  that  has the VALID

switch off will NOT cause the search to stop and log.

When  a  program memory channel is transferred to the current  radio, the

VALID switch will be  used  to set  or reset  any channel  lockout in the

radio.

DELAY:

The  DELAY  field  is  use during the SCAN  and SEARCH  operations.  If

delay is set ON, scanning will continue DELAY  seconds  after  the signal

ends on that  channel.  If delay  is set OFF, then scanning will continue

immediately after the signal  ends.  The  DELAY time value can be set  on

the PARMS panel.

When a  program memory channel  is transferred to the current  radio, the

DELAY switch will be used to set or reset any channel delay switch in the

radio. Note that not all radios have this feature.

TRUNKED FREQ:

A memory channel can be identified as a trunked channel. This is useful

for  being able  to store frequencies in the radio from  the program, but

having the ability to exclude  them from RadioCtl scans.  In addition, if

the radio supports it, this switch will be set in the radio when a memory

channel is transferred to that radio.

RSM:

The  RSM  field selects the resume function  for  the individual memory

channel. If this switch is  on, and  a signal is detected for this memory

channel in SEARCH  or SCAN mode, SEARCH or SCAN will continue after a set

number of second if the  signal has not gone away. This works the same as

the global RESUME switch except that it only affects the single channel.

FREQUENCY:

The FREQUENCY field is used to record or set the current memory channel

frequency.  Frequency information is entered in Mhz.

MODE:

The  MODE  field  represents the  modulation being used for a  specific

frequency.  This  field  is  a  drop  down selection  box.  To  select  a

modulation, click on the button to the right of the  box. Double click on

the desired  modulation.  Note  that not all modulations will be accepted

by your particular radio.

Memory Channel Panel  <Radio Control Program for Windows>         Page 28

COUNT:

This  field  displays  the  number of times a signal  was detected on a

specific channel. This field cannot be edited.  However, left-clicking on

this field with the mouse cursor will reset the field to 0.

TIME:

This  field displays the  average number of seconds that a  signal  was

detected on  a specific  channel. This  field cannot be edited.  However,

left-clicking on this field with the mouse cursor will reset the field to

0.

SIGNAL:

The signal strength of the last recorded signal is  reported here. This

field cannot be edited.  However, left-clicking  on  this field with  the

mouse cursor will reset the field to 0.

TIME STAMP:

When ever  a squelch break  occurs for this channel, the time stamp  is

updated with the time and date of the occurrence.  This  field  cannot be

edited.  However,  left-clicking on this field with the mouse cursor will

reset the field to 0.

SERVICE:

This field is used to record the  service for a specific channel.  This

is  a  scratch  pad  location to  record  information  about  a  specific

frequency.  If the squelch  breaks on this channel during  SCAN or SEARCH

mode, the program will attempt  to lookup the  service for  the frequency

entered. If the frequency is found in the current frequency database, and

the SERVICE field  is blank, then it will  be filled in with the  service

found in the database.

AUX CONTROLS:

A number of Memory settings  are enclosed in  a box  identified  as AUX

CONTROLS. Depending on the protocol  selected, and the DISPLAY UNUSED AUX

option setting, these controls may or may not be visible. If  the DISPLAY

UNUSED AUX option (set in  the GLOBAL OPTIONS menu) is not selected, only

those controls  that are meaningful to the radio being used  are visible.

In addition, some controls will have different descriptions for different

radios. Please see table 3 for a list of  what controls are available for

what protocol.

ATTENUATOR:

Some  radios  have a  built  in attenuator. This  can be switched on to

reduce overload  from a  strong signal. Clicking this  switch will set or

reset the attenuator setting when the program memory  channel is sent  to

the radio.

NOTCH|TONE ENABLE:

For certain radios, a notch filter is available. For these radios, this

switch  enables or  disables the  radio's  notch filter when this  memory

channel  is   sent  to   the  radio.  For   other   radios,  this  switch

enables/disables  a descrambler tone.  This  switch  may  also enable  or

disable the CTCSS  tone function.  If supported by  the radio, a CTCSS or

descrambler tone may be set using the Tone  drop-down box, and enabled by

this switch. See Table 3 for a breakdown  of  how this switch is  defined

for each radio.

AMSYNC:

For certain radios, an AM synchronous detector is available.  For these

radios, this switch enables or  disables the radio's preamp function when

the  program  memory  channel  is  sent to the  radio.  For  the  BEARCAT

protocol, this function enables or disabled the RECORDER function.

Memory Channel Panel  <Radio Control Program for Windows>         Page 29

PREAMP:

For  certain  radios,  a pre-amplifier is available.  For these radios,

this switch  enables or disables  the radio's preamp  function  when  the

program memory channel is sent to the radio.

NOTCH|TONE  VALUE:

For  certain  radios,  a CTCSS or descrambler tone  is  available.  For

these  radios,  this  dropdown  box  selects the  tone  value.  When  the

Notch|Tone  switch is enabled, the  selected Tone value will be sent when

the program memory channel is sent to the radio.

AGC:

For certain  radios,  an Automatic Gain Control  is available For these

radios,  this  dropdown  box  selects the AGC level  to be  set  when the

program memory channel is sent to the radio.

ANT:

For certain radios, an antenna selection is available For these radios,

this  dropdown box selects the particular antenna to use when the program

memory channel is sent to the radio.

NOISE BLANKER:

For  certain radios, a noise  blanker  function is  available For these

radios, this dropdown box selects the noise blanker function to  use when

the program memory channel is sent to the radio.

BANDWIDTH:

For  certain  radios, a bandwidth  function  is  available.  For  these

radios, this dropdown box selects the bandwidth of the radio.

Note: The Bandwidth control is kept in sync with the Modulation selection

when the modulation type is FM.

TONE 2:

For certain  radios, a second  tone function  is available.  For  these

radios, this  dropdown  box selects the  value of  that tone.  The switch

below this control enables or disables this function.

MENU:

Several menu operations are available for manipulating memory channels.

These operations are selected by clicking on the Menu option, or pressing

the underlined menu letter.

DATABASE MENU:

The Database  menu allows  you  to  manipulate  various aspects  of the

memory channels. A sort of memory  channel by frequency can be done  from

this  panel. With a sorted  frequency  list, the radio  being  controlled

requires  less settling  time for  each frequency  change,  allowing  for

faster signal detection.

The  Service  lookup will  update  all  Service  fields for  any memory

channel with a non-zero frequency. The service  information  is  obtained

from the current DBASE file.

The Clear  option allows you to  clear all memory channels, all service

fields from all memory channels, or all count, time and  signal logs from

all memory channels.

EDIT MENU:

Selecting  the Edit  menu item will display  the  Move/Copy/Swap panel.

This panel is used to manipulate a block of memory channels at a time.

The COPY operation will  copy  a source block of  channels  to  another

location.  The MOVE operation  is the same as copy except that the source

Memory Channel Panel  <Radio Control Program for Windows>         Page 30

channels are cleared.  SWAP will exchange two blocks of channels.

For each operation, a starting and stopping channel is required for the

source block, and a starting channel is required for the destination. The

source  and destination  blocks can overlap.  If  the  number  of  source

channels specified would cause  the  destination to  exceed the number of

available channels, then  the  program  will adjust the source block such

that  the operation  will  end  when reaching  the  last available memory

channel.

Memory Channel Panel  <Radio Control Program for Windows>         Page 31

SCAN BANKS PANEL

Scan  Bank entry  allows  you to define banks of channels for scanning.

RADIOCTL allows you to  assign any range of channels to various banks. Up

to 100 banks (0-99) can be assigned. Banks can overlap, and can be turned

on or off selectively. In order  for  SCAN mode to recognize  banks,  the

Banks On switch  must  be active.  The bank displayed  has several fields

associated with it.

CHANNEL NUMBER:

This is the first field  on the  display. It determines which scan bank

is being displayed or changed.  The range for this channel number is 0 to

99.  The Channel number can be changed  by  direct entry  in  the channel

field, or using the scroll bars to  the left of the  channel  number.  If

entering  the  channel  number, keep  in  mind that the data  will NOT be

entered unless you press ENTER or a TAB key.

VALID:

The  VALID  field determines whether  or  not SCAN  will use this bank.

Scanning banks without the VALID switch will be ignored at all times.

START:

START identifies  the  first memory channel to start  scanning with for

the given bank. When this  bank is active, START will be the first memory

channel  scanned  if  the  memory  channel's  VALID switch is set. If the

memory channel's VALID  switch is NOT set, then the first  memory channel

to be scanned will  be  the next sequential  memory channel with  a VALID

switch.

STOP:

STOP  identifies  the  last  memory channel  to scan when  the  bank is

active.  After  this memory channel is  scanned, scanning  will  continue

with the  memory channel specified by  the START value of the next  valid

SCAN bank.

If the STOP channel is set lower than the START channel,  the SCAN will

decrement channel numbers. If STOP is greater than START, channel numbers

will increment. If STOP and START are equal, the single channel number in

START and END will be scanned.

SERVICE:

This field is provided to enter comments about a given bank.  It is not

used or modified by any program function.

MENU:

Several menu operations are available for manipulating memory channels.

These operations are selected by clicking on the Menu option, or pressing

the underlined menu letter.

DATABASE MENU:

The Database menu allows you  to clear  a single scan bank or all  scan

banks.

EDIT MENU:

Selecting  the  Edit  menu item will display the  Move/Copy/Swap panel.

This panel is used to manipulate a block of scan channels at a time.

The COPY operation  will copy  a source  block  of channels to  another

location.  The MOVE operation is the same as  copy except that the source

channels are cleared.  SWAP will exchange two blocks of banks.

For  each operation, a starting and stopping  channel  required for the

source block, and a starting channel is required for the destination. The

source  and  destination blocks can  overlap.  If the  number  of  source

channels  specified  would  cause  the  destination  to exceed the number

of available channels, then the program will adjust the source block such

Scan Banks Panel  <Radio Control Program for Windows>             Page 32

that  the  operation will end  when  reaching  the  last  available  scan

channel.

Scan Banks Panel  <Radio Control Program for Windows>             Page 33

SEARCH BANKS PANEL

The  search bank panel  allows  you  to  define  banks  of channels for

searching.  RADIOCTL allows you to assign any range of frequencies, along

with a  modulation and step value for searching.  Up to 100 search  banks

(0-99) can  be defined.  In order for SEARCH mode to recognize banks, the

Banks On Global switch must be active.

When searching, the  program will  start with the  first search channel

marked as  valid, set the radio  VFO to the  frequency in the START field

and the modulation  in the  MODE  field,  and check for  a signal.  If no

signal is detected, the program will increment or decrement the frequency

by the  amount  in  the  INC  field,  and repeat  the  process. When  the

frequency in  the STOP field is reached, the program will select the next

valid  search bank and repeat the process. When all valid search channels

have been  processed, the program  returns  to the first search bank. For

more details  on  the SEARCH operation, see the description of the SEARCH

mode.

CHANNEL NUMBER:

The CHANNEL number determines which search bank  is  being displayed or

changed.  The range for  this  channel  number is 0 to  99.  The  Channel

number can be changed by direct entry in the channel field, or using  the

scroll bars to the  left of the channel  number.  If entering the channel

number, keep in  mind that the data will NOT be accepted unless you press

ENTER or a TAB key.

VALID:

The  VALID  field determines whether or  not SEARCH will use this bank.

Search banks without the VALID switch will be ignored at all times.

START:

START identifies the first frequency  to set  for  the  search.  Please

note  that  the program does  NOT check the range of this value. If it is

outside the range of the radio, the radio may  ignore the setting.

STOP:

STOP identifies the  last  frequency to set for the search.  The search

bank ends  when the VFO frequency reaches  the Stop  value.  If  the Stop

value is less than the Start value, the frequency is decremented. If  the

Stop value is greater than the Start value, the frequency is incremented.

Please note that  the program does NOT check  the range of this value. If

it is outside the range of the radio, the radio may  ignore the setting.

INCR:

INCR determines the steps for  the given search bank. The Incr value is

added to (or subtracted from) the VFO frequency for each step. If an Incr

value of 0 is entered, the SEARCH mode will automatically reset the VALID

switch for that bank.

MODE:

The  MODE  field represents the modulation being  used  for the  search

range. The radio will be set to this modulation at  the  beginning of the

search range.  This  field  is  a  drop  down  selection box. To select a

modulation, click on the  button to the right of the box. Double click on

the desired modulation.  Note  that not all modulations  will be accepted

by your particular radio.

SERVICE:

This field is provided to enter comments about a given bank.  It is not

used or modified by any program function.

Search Banks Panel  <Radio Control Program for Windows>           Page 34

MENU:

Several  menu  operations are  available for manipulating  search  bank

channels.  These operations are  selected by clicking on the Menu option,

or pressing the underlined menu letter.

DATABASE MENU:

The  Database  menu  allows  you to  clear a single search  bank or all

search banks.

EDIT MENU:

Selecting  the Edit  menu  item will  display the Move/Copy/Swap panel.

This panel is used to manipulate a block of search channels at a time.

The  COPY  operation will  copy  a source block of  channels to another

location.  The MOVE operation is  the same as copy except that the source

channels are cleared.  SWAP will exchange two blocks of banks.

For  each operation, a  starting and stopping channel  is required  for

the source block, and a starting channel is required for the destination.

The  source and destination blocks can overlap.  If  the number of source

channels  specified would cause the destination to  exceed the  number of

available channels, then the program will  adjust the source  block  such

that  the  operation will end when  reaching  the last  available  search

channel.

Search Banks Panel  <Radio Control Program for Windows>           Page 35

MESSAGE PANEL

The Message Panel displays messages from the program. This may be error

messages, or normal operation  messages.  The time and  date are included

with all messages as an indication of  when the error occurred. Up to  10

messages at a time are displayed. The most recent message is displayed in

red at the top of the panel

If the LOG TO  FILE Global switch is set, all  messages  that appear on

the Message Panel are also logged to the Log file.

Message Panel    <Radio Control Program for Windows>              Page 36

SPECTRUM PANEL

The  spectrum display shows  a graph of frequency vs.  signal strength.

An active  channel will be shown as a red  line  (if the graph is in  the

range of  the active frequency). Frequencies  representing past  activity

will be displayed in green.

In  the  lower  left  hand  corner,  the  starting  frequency  will  be

displayed. The ending frequency of the spectrum  will be displayed in the

lower right hand  corner  of  the display.  The  center frequency of  the

spectrum  is  changed by moving the  horizontal scroll bar.  The  minimum

start frequency is 0 mhz and the maximum stop frequency is 9999.9mhz. The

horz  step  increment  is determined  by  the  spectrum  resolution.  The

resolution is changed by  moving the vertical scroll bar. Resolution goes

from 100khz per pixel to 1khz per pixel in 1khz steps.

Tic marks are displayed for reference. Tic marks are displayed every 10

mhz  when the resolution is above 10khz/pixel.  When  the  resolution  is

10khz or below, the tic marks are displayed every 1mhz.

Spectrum Panel   <Radio Control Program for Windows>              Page 37

INFORMATION UNIQUE FOR VARIOUS RADIOS

Every radio supported is unique in the way it  operates.  The  RadioCtl

program  has  attempted  to  standardize  the way  that  all  radios  are

controlled. However, it  is  not always  possible  to  make  every  radio

perform the  same way. The  following information  is provided to explain

and identify some of the differences.

ICOM:

Several ICOM  models have variations on the command set.  As a result,

you will need to  select  the particular  model  (in the PARMS panel) for

proper operation.

KENWOOD:

OPTO:

OPTO  radios do NOT  have memory channels. Therefore all  the features

relating to storing and recalling memory from the radio do not work.

AOR3000A:

The AOR3000a does not  have  the ability to clear memory  channels from

the computer. If a  0  frequency is encountered during any operation, the

information will NOT be sent to the radio.

AOR5000:

FM  and  FMw  modulation  provide  the  same  modulation   when   set.

Narrow/Wide modulation is provided by the setting of the BandWidth.

AOR8000:

The  AOR8000 is organized into  20 banks of 50 channels.  Each  bank is

identified as a lower or upper case letter. RADIOCTL maps these  channels

as follows:

TABLE 2- AOR to RadioCtl mapping

┌─────────┬──────────────────┬─────────────┬─────────────┐

│ radio   │   computer       │    radio    │   computer  │

├─────────┼──────────────────┼─────────────┼─────────────┤

│A00-A49  │ 000 - 049        │    a00-a49  │ 050 - 099   │

├─────────┼──────────────────┼─────────────┼─────────────┤

│B00-B49  │ 100 - 149        │    b00-b49  │ 150 - 199   │

├─────────┼──────────────────┼─────────────┼─────────────┤

│C00-C49  │ 200 - 249        │    c00-c49  │ 250 - 299   │

├─────────┼──────────────────┼─────────────┼─────────────┤

│D00-D49  │ 300 - 349        │    d00-d49  │ 350 - 399   │

├─────────┼──────────────────┼─────────────┼─────────────┤

│E00-E49  │ 400 - 449        │    e00-e49  │ 450 - 499   │

├─────────┼──────────────────┼─────────────┼─────────────┤

│F00-F49  │ 500 - 549        │    f00-f49  │ 550 - 599   │

├─────────┼──────────────────┼─────────────┼─────────────┤

│G00-G49  │ 600 - 649        │    g00-g49  │ 650 - 699   │

├─────────┼──────────────────┼─────────────┼─────────────┤

│H00-H49  │ 700 - 749        │    h00-h49  │ 750 - 799   │

├─────────┼──────────────────┼─────────────┼─────────────┤

│I00-I49  │ 800 - 849        │    i00-i49  │ 850 - 899   │

├─────────┼──────────────────┼─────────────┼─────────────┤

│J00-J49  │ 900 - 849        │    j00-j49  │ 950 - 999   │

└─────────┴──────────────────┴─────────────┴─────────────┘

The AOR8000 provides  for  the  installation  of  a decoder for scrambled

communications. If this decoder is installed, the TONE ENABLE switch will

turn the decoder  on  and off.  The  TONE select drop-down box allows the

selection of a decoding tone.

Radio Specific Info  <Radio Control Program for Windows>          Page 38

BEARCAT:

Because the BEARCAT does NOT have a VFO, RadioCtl  makes use of one of

the radio's memory channels to function  as a VFO. The default channel is

300, but is changeable.

BEARCAT  radios  do  NOT have  the ability to  change modulation.  The

modulation within the radio is selected based on the frequency range, and

is not user selectable.

RadioCtl currently  cannot setup Trunk banks. Future releases may include

this support. However, memory channels CAN be  identified as belonging to

a trunked  frequency set. Once these frequencies have been written to the

radio, all  that is  required  is to select  the trunk system type (using

TRUNK key on  the  radio).  The RadioCtl  program  reduces  the amount of

manual data entry for a trunked system.

FUNCTIONS AVAILABLE FOR EACH RADIO:

The following table shows what functions are available for each of the

protocols

TABLE 3 - Functions available by radio

┌────────────┬───────┬──────┬─────────┬────────┬─────────┬──────┐

│ function   │ LOCAL │ ICOM │ KENWOOD │ AOR3000│ AOR8000 │ OPTO │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Tone|Notch  │ Tone  │ No   │ Notch   │ No     │ Tone    │ Tone │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│AM Sync     │ Yes   │ No   │ No      │ No     │ No      │ No   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Preamp      │ Yes   │ No   │ No      │ No     │ No      │ No   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Attenuator  │ Yes   │ Yes  │ Yes     │ No     │ Yes     │ Yes  │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Tone        │ Yes   │ No   │ No      │ No     │ Yes     │ Yes  │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│AGC         │ Yes   │ Yes  │ Yes     │ No     │ No      │ No   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Antenna     │ Yes   │ Yes  │ Yes     │ No     │ No      │ No   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│NoiseBlank  │ Yes   │ No   │ No      │ No     │ No      │ No   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Bandwidth   │ Yes   │ Yes  │ No      │ No     │ No      │ No   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Tone 2      │ Yes   │ No   │ No      │ No     │ No      │ No   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Trunked Freq│ Yes   │ No   │ No      │ No     │ NO      │ No   │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Light on Sig│ Yes   │ Yes  │ Yes     │ Yes    │ Yes     │ Yes  │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│            │       │      │         │        │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│            │       │      │         │        │         │      │

└────────────┴───────┴──────┴─────────┴────────┴─────────┴──────┘

Radio Specific Info  <Radio Control Program for Windows>          Page 39

TABLE 3 - Functions available by radio (continued)

┌────────────┬───────┬──────┬─────────┬────────┬─────────┬──────┐

│ function   │ DRAKE │ THF6A│ BEARCAT │ AOR5000│         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Tone|Notch  │ Notch │ TONE │ CTCSS   │ Tone   │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│AM Sync     │ Yes   │ No   │ Recorder│Recorder│         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Preamp      │ Yes   │ No   │ No      │ No     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Attenuator  │ Yes   │ Yes  │ Yes*    │ Yes    │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Tone        │ No    │ Yes  │ Yes     │ Yes    │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│AGC         │ Yes   │ No   │ No      │ No     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Antenna     │ Yes   │ No   │ No      │ No     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│NoiseBlank  │ Yes   │ No   │ No      │ No     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Bandwidth   │ Yes   │ No   │ No      │ No     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Tone 2      │ No    │ CTCSS│ No      │ No     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Trunked Freq│ No    │ No   │ Yes*    │ No     │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│Light On Sig│ Yes   │ Yes  │ Yes*    │ Yes    │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│            │       │      │         │        │         │      │

├────────────┼───────┼──────┼─────────┼────────┼─────────┼──────┤

│            │       │      │         │        │         │      │

└────────────┴───────┴──────┴─────────┴────────┴─────────┴──────┘

*model dependent

Radio Specific Info  <Radio Control Program for Windows>          Page 40

SPECIAL KEYS

ENTER:

The ENTER key  finalizes  input to the program. For data  entry fields,

the information is stored  into  the  computer's  memory, and the  cursor

advances to the next field.

TAB KEYS:

The tab  keys  will  move the cursor to the  next  field. A forward  or

reverse tab key will complete data entry of the current field.

Special Keys     <Radio Control Program for Windows>              Page 41

DATA FILE FORMATS

There are several files that this program makes use of. These files are

used to store and retrieve data used by the program.

LOGFILE FORMAT:

When LOGGING  is  enabled, certain transactions are logged to  a file.

The name of this file is set by  the user. The  initial  setting for this

filename is RADIOCTL.LOG.

For a given transaction, the following information is logged:

*   Timestamp- The Date and Time of the transaction.

*   The scanning mode (SCAN/SEARCH/MANUAL).

*   The VFO frequency.

*   The VFO modulation.

*   The current memory channel.

*   The signal strength at the time of transaction.

*   The length of the transaction.

*   The service information for the current memory channel.

FREQLIST.DBF:

This is a standard DBASE  III+ type  data file. It contains frequencies

and allocations for  the radio  spectrum. There are three fields that are

used by this program.

field name     allocation       description

-----------------------------------------------------------

FREQUENCY      decimal 10.5     holds the frequency in Mhz

DESCRIPT       character 50     usage of the frequency

ACTION         logical          flags the frequency in use

If the ACTION field is  not present, the program will use  the DESCRIPT

field  to determine ACTION. A  blank DESCRIPT field signals  an  inactive

channel.

The  length of the DESCRIPT field  can be changed, if so desired.  This

program only uses the first 80 chars of this field.

Any  other  fields can be  added  or  deleted  from  this  file without

affecting the operation of the program.  However, if either the FREQUENCY

or DESCRIPT field is not present in the database, the DBASE function will

not operate.

DATA FILE FORMAT:

The SAVE command  creates  a file with the current settings  of  memory

channels, and  various scan  parameters.  The  file created is  an  ASCII

readable  file. Each record is terminated by a CR/LF sequence. One record

represents one frequency channel, one  scan bank channel, one search bank

channel, one  command, or  a comment.  Which type if record is determined

by the first non-blank  character on the line.  The following table shows

the identifier for the various types of records:

Data File Formats  <Radio Control Program for Windows>            Page 42

TABLE 4- Record IDs

┌──────────────┬─────────────────────────────┐

│ identifier   │  type of record             │

├──────────────┼─────────────────────────────┤

│ number       │ frequency channel info      │

├──────────────┼─────────────────────────────┤

│ @            │ scan bank information       │

├──────────────┼─────────────────────────────┤

│ #            │ search bank information     │

├──────────────┼─────────────────────────────┤

│ keyword      │ command to program          │

├──────────────┼─────────────────────────────┤

│ *            │ comment                     │

└──────────────┴─────────────────────────────┘

Each record contains one  or more fields  delimited  by blanks.  The file

can be altered (or created)  using any standard text editor.  The maximum

record  length that the  program can handle is  200  characters.  If  the

comment record flag is detected,  all information following  the  '*'  is

ignored on  that line.  When  the program  reads  the  file, any  missing

required parameter, parameter range error, or  unrecognized  keyword will

terminate the file read at that line.

Data File Formats  <Radio Control Program for Windows>            Page 43

The format for a frequency channel entry is as follows:

TABLE 5- Frequency Channel Entry Format

┌─────┬───────────┬────────────────┬────────────────────────────┐

│field│ name      │ value range    │ description                │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   1 │ CHANNEL   │ 0-9999         │channel number to set       │

│     │           │                │ required-positional        │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   2 │ FREQUENCY │ 0-999.9999     │frequency in Mhz            │

│     │           │                │ required-positional        │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   3 │ MODE      │ 3 chars        │valid modulation for radio  │

│     │           │                │ required-positional        │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ COUNT     │ 0-999999       │count of squelch breaks     │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ DURATION  │ 0-999999       │total duration of squelch   │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ SIGNAL    │ Sxx            │Signal strength value (xx)  │

│     │           │ xx=00 to 15    │ optional                   │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ VALID     │ VALID/NOTVALID │indicator of channel valid  │

│     │           │ VLD            │                            │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ ATTEN     │ ANTEN/NOANTEN  │indicator of attenuation    │

│     │           │ ATT            │                            │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ RESUME    │ RESUME/NORESUME│indicator of resume         │

│     │           │ RSM            │                            │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ TSTAMP    │ TSTAMP/TS value│squelch break time stamp    │

│     │           │ TSTAMP/TS 0    │time value is UNIX time     │

│     │           │                │(number of seconds since    │

│     │           │                │ 1970) 0 = no stamp.        │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ TONE      │ PL 0 0 0       │Tone values. All three      │

│     │           │                │numbers must be specified   │

│     │           │                │and between 0 and 15.       │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ AGC       │ AGCO/AGCS/AGCF │AGC OFF/SLOW/FAST           │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ BNDWDTH   │ BW0/BW1/BW2/   │Bandwidth 0-6               │

│     │           │ BW3/BW4/BW5/BW6│ see BANDWIDTH table        │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ NOISE     │ NBO/NBN/NBW    │Noise Blanking OFF/NAR/WIDE │

│     │ BLANKING  │                │                            │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ NOTCH     │ NTCH           │Notch filter on (default    │

│     │           │                │ is off)                    │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ PREAMP    │ PAMP           │Preamp on (default is off)  │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ SYNC      │ SYNC           │SyncDet on (default is off) │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ TRUNK     │ TR             │Frequency is Trunked        │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ ANTENNA   │ A1/A2/AV       │Antenna 1/2/VHF             │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ DELAY     │ DELAY/NODELAY  │indicator of channel delay  │

│     │           │ DLY            │                            │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   * │ SERVICE   │ 80 chars       │service for channel         │

└─────┴───────────┴────────────────┴────────────────────────────┘

*optional fields

Data File Formats  <Radio Control Program for Windows>            Page 44

The  first three fields MUST  be specified and be in the  proper order.

Optional  fields do NOT  have  to  be specified.  However, the  following

positional restrictions apply:,

*   If COUNT  is  specified,  it  MUST  follow  the  MODE  field.  If not

specified, the COUNT value for the channel is set to 0.

*   If DURATION is specified,  COUNT must also be specified, and DURATION

must follow the COUNT.  If not specified, the DURATION value  for the

channel is set to 0.

*   Duration value MUST be specified if  any of the following fields  are

specified:

SIGNAL,  VALID,  ANTEN, RESUME, TSTAMP, TONE, DELAY, SERVICE.  AGC,

PREAMP, BANDWIDTH,NOISE BLANKING, NOTCH TRUNK or SYNC.

*   If SIGNAL value  is not specified, then SIGNAL value  for the channel

is set to 0.

*   If VALID  is not specified, the channel will be marked VALID.

*   If ANTEN, RESUME  or DELAY is not specified, the  respective switches

will be reset for the channel.

*   If DELAY is specified, it MUST precede the service field.

*   If  SERVICE  is  specified,  DELAY  must   also  be  specified.   All

characters (up to 80) following the DELAY value and up to the  end of

the line  (CR/LF) are taken  as a SERVICE string.  Leading blanks and

any special ASCII characters are removed.

BANDWIDTH keyword  sets a  numeric  index. If  the radio allows  for  a

bandwidth setting, the  index will be used as per the table below. If the

radio does not support BANDWIDTH, then the value is ignored.

TABLE 6- Bandwidth settings

┌─────┬───────────┬────────────────┐

│index│ DRAKE-R8  │ AOR500         │

├─────┼───────────┼────────────────┤

│   0 │  0.5khz   │ 0.5khz         │

├─────┼───────────┼────────────────┤

│   1 │  1.8khz   │ 1.3khz         │

├─────┼───────────┼────────────────┤

│   2 │  2.3khz   │ 6khz           │

├─────┼───────────┼────────────────┤

│   3 │  4.0khz   │ 15khz          │

├─────┼───────────┼────────────────┤

│   4 │  6.0khz   │ 30khz          │

├─────┼───────────┼────────────────┤

│   5 │  6.0khz   │ 110khz         │

├─────┼───────────┼────────────────┤

│   6 │  6.0khz   │ 220khz         │

└─────┴───────────┴────────────────┘

The TONE (PL) field is provided for future enhancement. At present, the

AOR8000 uses  the first  number  as a tone number  for decoding scrambled

communications. The other two numbers are not  currently used, but should

be set to 0 for future compatibility.

Data File Formats  <Radio Control Program for Windows>            Page 45

The format for a scan bank entry is as follows:

TABLE 7 - Scan Bank record format

┌─────┬───────────┬────────────────┬────────────────────────────┐

│field│  name     │ value range    │ description                │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   1 │ BANKID    │ @              │@ identifies a scan bank    │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   2 │ BANK      │ 1-100          │bank number                 │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   3 │ START     │ 0- MAX         │starting channel for bank   │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   4 │ END       │ 0- MAX         │ending   channel for bank   │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   5*│ SCAN      │ SCAN/NOSCAN    │valid bank                  │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   6*│ DESCRIPT  │ 80 chars       │description of bank         │

└─────┴───────────┴────────────────┴────────────────────────────┘

*optional fields

If NOSCAN is not specified, the bank is marked as valid.

Data File Formats  <Radio Control Program for Windows>            Page 46

The format for a search bank entry is as follows:

TABLE 8 - Search Bank record format

┌─────┬───────────┬────────────────┬────────────────────────────┐

│field│  name     │ value range    │ description                │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   1 │ BANKID    │ #              │@ identifies a search bank  │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   2 │ BANK      │ 1-100          │bank number                 │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   3 │ START     │ 0- 9999.99999  │starting frequency for bank │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   4 │ END       │ 0- 9999.99999  │ending   frequency for bank │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   5 │ INC       │ 0- 9999.99999  │frequency increment         │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   5 │ MODE      │AM,FM,FMN,etc.  │modulation for bank         │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   5*│ SEARCH    │ SEARCH/NOSEARCH│valid bank                  │

├─────┼───────────┼────────────────┼────────────────────────────┤

│   6*│ DESCRIPT  │ 80 chars       │description of bank         │

└─────┴───────────┴────────────────┴────────────────────────────┘

*optional fields

If NOSEARCH is not specified, the bank is marked as valid.

Data File Formats  <Radio Control Program for Windows>            Page 47

Certain  commands can be  entered  in a text file for execution.  These

commands provide the ability to execute certain operations automatically,

without having to select them from the various panel controls.

TABLE 9 - Recognized Commands in Frequency File

┌────────────────┬────────────────┬────────────────────────────┐

│command         │ option(s)      │ description                │

├────────────────┼────────────────┼────────────────────────────┤

│BANK            │ ON|OFF         │BANK mode on or off.        │

│                │                │Equivalent to pressing BANK │

│                │                │switch.                     │

├────────────────┼────────────────┼────────────────────────────┤

│CHAIN           │ filespec       │start reading another text  │

│                │                │file. Maximum CHAIN depth   │

│                │                │is 3 files.                 │

├────────────────┼────────────────┼────────────────────────────┤

│CHANNEL         │ 0 to 9999      │set channel number.         │

│                │                │Equivalent to setting       │

│                │                │CHANNEL display value.      │

├────────────────┼────────────────┼────────────────────────────┤

│CLEAR           │    -           │Clear all memory channels   │

│                │    ALL         │Clear all memory channels   │

│                │    COUNTS      │Clear all squelch counts    │

│                │    SERVICE     │Clear all service strings   │

│                │    SIGNAL      │Clear all strength readings │

│                │    TIME        │Clear all squelch times     │

├────────────────┼────────────────┼────────────────────────────┤

│DBNAME          │ filespec       │change DBASE filename for   │

│                │                │service lookups.            │

├────────────────┼────────────────┼────────────────────────────┤

│DELAY           │    xxxx        │Set DELAY value to xxxx.    │

├────────────────┼────────────────┼────────────────────────────┤

│F               │  frequency     │Set VFO frequency           │

├────────────────┼────────────────┼────────────────────────────┤

│HALT            │     -          │Force MANUAL mode           │

├────────────────┼────────────────┼────────────────────────────┤

│LOAD            │     -          │load memory from radio      │

├────────────────┼────────────────┼────────────────────────────┤

│M               │   modulation   │set VFO modulation          │

└────────────────┴────────────────┴────────────────────────────┘

Data File Formats  <Radio Control Program for Windows>            Page 48

TABLE 9 - Recognized Commands in Frequency File (continued)

┌────────────────┬────────────────┬────────────────────────────┐

│MANUAL          │     -          │Force MANUAL mode           │

├────────────────┼────────────────┼────────────────────────────┤

│MEMORY          │     -          │use radio's memory          │

├────────────────┼────────────────┼────────────────────────────┤

│PDELAY          │   0 to 9999    │set priority interval       │

├────────────────┼────────────────┼────────────────────────────┤

│QUIT            │     -          │exit RADIOCTL               │

├────────────────┼────────────────┼────────────────────────────┤

│PRIORITY        │     ON         │set priority switch on      │

│                │     OFF        │set priority switch off     │

│                │   0 to 999     │set priority channel        │

├────────────────┼────────────────┼────────────────────────────┤

│RESUME          │     ON         │set resume   switch on      │

│                │     OFF        │set resume   switch off     │

│                │   0 to 9999    │set resume  interval        │

├────────────────┼────────────────┼────────────────────────────┤

│SCAN            │     -          │Start SCAN mode             │

├────────────────┼────────────────┼────────────────────────────┤

│SEARCH          │     -          │Start SEARCH mode           │

├────────────────┼────────────────┼────────────────────────────┤

│SORT            │     -          │Sort memory channels        │

├────────────────┼────────────────┼────────────────────────────┤

│VFO             │     -          │use computer's memory       │

└────────────────┴────────────────┴────────────────────────────┘

Note:Certain commands are processed on a  deferred basis. In other words,

they are not processed until the entire file (and any chained files)  are

read. Because of this, only one of these commands  (the last one read in)

will be processed. As  a  result, only one of  these  commands should  be

present in the file. The commands are:

CHANNEL, F, HALT, LOAD, M, MANUAL, SCAN, and SEARCH.

The F and M commands CAN co-exist in the same file.

Any record that is not recognized as a Frequency, Scan bank, Search bank,

command or comment will be ignored.

Data File Formats  <Radio Control Program for Windows>            Page 49

RS232 INTERFACE

This program is designed  to use the serial communications port (RS232)

of  the  IBM computer. In  order to  interface a radio  to  this port, an

interface adapter  is needed designed for the specific radio. Each  radio

manufacturer  makes  such  an  adapter.  However, none  of  the  adapters

supplied  from the manufacturer  provide for squelch  detection.  Without

this  detection,  the signal sensing functions of this  program  will not

work.

For some  of  the  radios  supported, a circuit  and  instructions  are

available.  This  circuit provides the squelch detection  needed by  this

program.  The interface  circuits  are  easy to  build,  and  require few

parts.  All of  the  parts  can be obtained cheaply  at  any  electronics

supply  store.  The  schematics  and instructions  are  provided  to  all

registered users.

RS232 Interface  <Radio Control Program for Windows>              Page 50

PROGRAM LIMITS AND RANGES

MEMORY REQUIRED:       4 meg (Windows Minimum). 8 Meg+ suggested

PROCESSOR REQUIRED:    80386 or Higher

WINDOWS LEVEL REQUIRED:Windows 95, Windows 98 or Windows NT

MAXIMUM MEMORY CHANNELS    1000

MAXIMUM SCAN BANKS          100

MAXIMUM SEARCH BANKS        100

FREQUENCY RANGE           .01khz to 9999.99999mhz*

MINIMUM FREQUENCY STEP     10hz*

MAXIMUM FREQUENCY STEP     9999.99999mhz

*may be limited by radio model.

Program Limits   <Radio Control Program for Windows>              Page 51

INDEX

A                                       L

Address of Radio 22                     Light on Signal 18

AOR3000 4                               Load 15

AOR3000A 3                              Local Protocol 8

Log To File 17

B

M

bank (defined) 5

Banks On 17                             Main Panel Menu 18

BEARCAT 3                               Manual Mode 10

Mem->VFO 16

memory (defined) 5

Memory Channel Panel 27

C                              Memory Channel

CHANNEL number 28

Channel 10                              Memory Scroll Buttons 10

channel (defined) 5                     Memory Sync 17

CTCSS Enable 18                         Memory

AGC 30

AmSync 29

Ant 30

D                                 Attenuator 29

Aux Controls 29

DBASE 3                                    Bandwidth 30

Digit Scrolling 25                         Count 29

Don't Update Memory 17                     delay 28

Frequency 28

Menu 30

Database 30

E                                    Edit 30

Modulation 28

Extended Commands 22                       Noise Blanker 30

Notch Enable 29

Notch Value 30

PreAmp 29

G                                 RSM 28

Service 29

Global Switches 17                         Signal 29

Time 29

Time Stamp 29

Tone 2 30

I                                 Tone Enable 29

Tone Value 30

ICOM 3                                     Trunked Freq 28

installation 8                             valid 28

interface 4, 6                          Menu

Parms Radio

Clear Everything 19

File 19

K                                 Parms 19

Global 19, Font, Delay, Display

KENWOOD 3                                        Unused Aux, File Save

Format, Font Size, Load

Frequency DBF on Start,

Priority, Resume

Radio 20, Address, Baud Rate,

Com Port, Data Bits,

Extended Commands, Flow

Control, Max Channel, Max

Frequency, Min Frequency,

Index               <Radio Control Program for Windows>             Page 52

Model, Protocol, Radio Name,   Scan Trunk Freqs 18

Signal Detect, Stop Bits,      Search  Mode

Timeout, VFO Channel              Global Switches 14

View 19                              Search Bank

Message Panel 35                           Channel number 34

Modulation 28                              Incr 34

Menu 34

Database 35

Edit 35

O                                 Mode 34

Service 34

Operation Buttons 10                       Start 34

MANUAL 10                               Stop-freq 34

SCAN 10                                 Valid 34

SEARCH 13                            Search Banks Panel 33

OPTO protocol 15, 16                    Search Mode 13

BANKS ON 14

DELAY 13

DON'T UPDATE MEMORY 15

P                                 MEM SYNC 14

Memory Channel 13

Priority 17                                RESUME 15

protocol (defined) 5                       RSM 13

Protocol Defaults 22                       SPEED 15

VALID 13

Search->Radio 16

Set 15

R                              shareware 3

Signal Detection 21

Radio Selection 18                      Signal Meter 18

Radio->Search 16                        Special Keys

Resume 17                                  ENTER 41

RS232 6                                    TAB 41

Spectrum Panel 36

Speed Control 17

Start Load-Set 16

S                              Status Controls 18

Stop  Load-Set 16

Scan Bank                               Swap VFO  Button 16

Channel number 32

Menu 32

Database 32

Edit 32                                    V

Service 32

Start 32                             Valid 15

Stop 32                              VFO (defined) 5

Valid 32                             VFO Frequency 25

Scan Banks Panel 31                     VFO Modulation 25

Scan Mode 10                            VFO step value 25

BANKS ON 11                          VFO->Mem 16

CTCSS Enable 12                      VFO

DELAY 11                                AGC 26

DON'T UPDATE MEMORY 12                  AmSync 26

Global Switches 11                      Ant 26

LOG TO FILE 12, 15                      Attenuator 26

Memory Channel 11                       Aux Controls 26

MEMORY SYNC 11                          Bandwidth 27

PRIORITY 11                             Noise Blanker 26

RESUME 12                               Notch Enable 26

RSM 11                                  Notch Filter value 26

SCAN TRUNK FREQS 12                     PreAmp 26

SPEED 13                                Recorder 26

TRUNKED FREQ 11                         Tone 2 27

Use COMPUTER's Memories 13              Tone Enable 26

Use RADIO's Memories 13                 Tone value 26

VALID 11                                Trunked Freq 25

Index               <Radio Control Program for Windows>             Page 53

VFO Active 26

Virtual VFO 22

Index               <Radio Control Program for Windows>             Page 54