ELX 215 : Microprocessor Systems

1.	a) Convert the following numbers to binary:

45					4CA9h

[1 mark]

b) Convert the following numbers to decimal:

10111012				5Ah

[1 mark]

c) Convert the following numbers to hex:

89					101100011101b

[1 mark]

2. Fill in the blanks with the specified contents, in hex or binary:

mov	ax, 1234h

mov	al, -1 		ax =

inc	aX 			ax =

mov	ax, 1234h

inc	aL ;			ax =

mov	al, 80h

cbw	;			ax =

mov	al, 77h

cbw	;			ax =

mov	ax, -1

mov	bx, 3

mul	bx ;			ax = 		, dx =

mov	ax, -1 ;

imul	bx ;			ax = 		, dx =

[8 marks]

3. Fill in the blanks with the specified contents, in hex or binary:

mov	ax, 1200h

dec	aX ;			ax =

mov	ax, 1200h

dec	aL ;			ax =

mov	al, FEh

cbw	;			ax =

mov	al, 5Eh

cbw	;			ax =

mov	ax, -1

mov	bx, 5

imul	bx ;			ax = 		, dx =

mov	ax, 3 ;

mul	bx			ax = 		, dx =

mov	ax, 3

imul	bx ;			ax = 		, dx =

mov	ax, 27

cwd

mov	ax, 5

idiv	bx ;			ax = 		, dx =

mov	ax, -2

cwd

mov	bx, 2

idiv	bx ;			ax = 		, dx =

mov	ax, -2

cwd

mov	ax, 2

div	bx ;			ax = 		, dx =

[8 marks]

4. Write a complete assembly program which does the following: Read a single character from the keyboard; compare the character read to the to the character 'y'; if they are equal, display 'YES', otherwise display 'MAYBE'.

[6 marks]

5. Translate the following arithmetic and logic statements into assembly code code. All variables are assumed to be signed words

a)	if (A + 4 < B) then X = X / 2;

b)	if (A <= B) then B = 2 else A = A - 1

c)	if (A < B) or (C == 0) then X = 12

d)	if (A > 14) then A = 0;

e)	if (X <= Y) then A = X else A = Y;

f)	if (A >= B + 3) or (X = 100) then M = 2 * M

[5 marks]

6. List six (6) distinct 64-bit multipurpose registers.

[3 marks]

7. Draw the conceptual view of a typical Pentium 4 microprocessor.

[4 marks]

8. Compare and contrast the program visible registers and program invisible registers.

[4 marks]

9. Write a complete program that fulfills the following criteria:

+ Asks user to key in username

+ Store username to a word size variable usr

+ Asks user to key in password

+ Store password to a doubleword size variable pwd

+ Compare if usr is “segielx”

+ Compare if pwd is “215215”

+ If usr is “segielx” AND pwd is “215215” the print message “Valid user”

+ Otherwise print message “invalid user”

[10 marks]

10. Fill in the contents of BL at the indicated places in the comments in the following code:

mov	bl, 11110000B

and	bl, 11001100B ;	bl =

mov	bl, 11110000B

or	bl, 11001100B ;	bl =

mov	bl, 11110000B

xor	bl, 11001100B ;	bl =

mov	bl, 11110000B

not	bl ;	bl =

mov	bl, 11110000B

shl	bl, 1 ;	bl =

mov	bl, 11110000B

rol	bl, 1 ;	bl =

mov	bl, 11110000B

shr	bl, 1 ;	bl =

mov	bl, 11110000B

sar	bl, 1 ;	bl =

[9 marks]

11. What will the contents of the register ax be after executing each of the following instruction sequences?

mov	ax, 5

shl	ax, 1				ax = _____________________

mov	ax, 10011100101b;	binary

and	ax, 11001001110b		ax = _____________________

[5 marks]

12. Suppose that A is an array of 20 words.  Explain in your own words, what does the following code do?

sub	ax, ax

mov	bx, OFFSET A

mov	cx, 20

mov	dx, 1

Aloop:

mov	[bx], ax

add	bx, 2

add	dx, 2

loop	Aloop

[5 marks]

13. Explain the following addressing modes and the relative speed.

- Direct

- Register

- Register relative

- Scaled index

- Immediate

[10 marks]