Technology And Its Uses

In this book I hope to create a manual for the learning Computer Technician – subclass troubleshooter.  Of course as such one can make forays into robotics, programming, graphics, design or even other complex machines.

As we are users of tool it is beneficial to have strong hands.  Not just the right.  And not just strong but dexterous.  Try juggling bottles of water.

Tools.  Many people ponder what tools are needed.  Valuable tools are a slightly magnetic number two Philips screwdriver, needle-nose pliers and a flat screwdriver with a short neck.  That’s it.  You may need a hammer incase the cops break down your door and there is no time to do a proper reformat of your hard drive, but save for that eventuality a hammer is counter productive as it encourages inconvenient and untimely outbursts of technology orientated violence.

First lesson that always pops is the video card slot lesson.  This speech grew popular in the age of the 486.  Most 486 boards have a 5 or more of the old style ISA slots all in a row.  And most of my students on seeing me demonstrate the installation of the above mention VGA Wonder video card state “So that’s the video card slot” as though in a moment of revelation.  The speech is said thusly “There is no single slot that the computer looks in for a video card any slot you can stick the card into will work.”  This speech skips over the problems of dead slots and the dreaded shared slot.  This speech was made redundant by the arrival of the AGP slot a single slot used for video cards but is still a valuable lesson.  This is not hard and it is not so complex as so many fear.

Old hardware.  Hardware can be finicky as it may work only conjunction with such and such parts. Hardware can be flaky and as such cause numerous errors that look like the small-scale software errors that one often finds but on such a scale as to be infuriating (See above reference for hammer use).  Or it can be simply working or dead.  (Note: I do not mention Jumper configuration as it is often best just to leave them as they are save for reference to the proper manual.)

Trouble shooting hardware.  Trouble shooting hardware is a linear action from boot or from the construction of the machine.

Trouble shooting hardware in boot.  From the time when the power button is first pressed to the time when the computer comes to rest and awaits your instruction it will use every part of it’s own essential hardware.

The first thing that can happen is as you turn on the computer the screen flickers yet remains black and a series of beeps emerge from the PC speaker.  3 beeps is a RAM error.  4 beeps is a Video error.  In either case first see if you have such a piece of hardware.  Then try reseating the offending pieces (Take it out and then put it back in) and then try again.  If the error continues you may have to replace the part as it is dead.  (Note:  A video card is a big square thing that the cable from the monitor plugs into and RAM comes in sticks that fit into 3 or 4 small slots in the motherboard.)

For a split second you will see the video card information on the screen it will be replaced by the BIOS information and the RAM count.  The BIOS is a chip containing basic information on your computer it may have to be setup.  During the RAM count instructions for entering the BIOS to change its’ setup will often be displayed.  This may have to be chosen but latter.  The bios will then begin locating and confirming hardware.  It will tell you of any problem it encounters.  You may have to reseat the keyboard.  “Keyboard error [Press F1]” most people run into a few times.  A few even find it funny.  The computer will then refresh the screen showing you all the hardware info the BIOS has including RAM, Processor, and what ever drives you have installed.  It will then try to begin initializing the OS (Operating system) from one of your drives.  The common drive priority is A: Floppy Disk and if there is no disk in the drive it will skip to C: Hard Disk.

If the computer finds no OS then you face several potential problems.  All end up saying that neither of the above mentioned drives is bootable.  The Hard drive may not be properly installed.  Reset and enter the BIOS setup.  If it is password protected and none of the standard passwords work (Sex, god, Windex, secret, password, Dave, pass and xxx) you face the choice of A) Resetting the BIOS or B) throwing out the motherboard.  Resetting the BIOS is often hard and requires either intimate knowledge of the board or the manual.  Enter the standard setup and find what Primary Master is set for.  Set it either for Auto or leave and find the autodetect hard disk option the autodetect will take a while but you will get conclusive results.  The drive will either work now or need internal intervention.  Save and exit if you have managed to install the drive or found it properly installed.

If the drive is installed yet does not boot then most likely no OS information has been installed.  At this point you had better know enough to know what OS you want installed.  DOS is a basic yet old OS you would be better off with a more contemporary OS like Windows NT, Windows whatever year is the most current to this one, or for the more adventurous Linux.

Find your self a CD-ROM boot disk.  You will need a working CD-ROM drive, a 1.44 floppy drive and an OS setup CD.  Slap the Boot disk in you floppy drive and turn the beast on.

I will assume your disk is DOS based so you will see the OS name pop up and a list of drivers being installed.  Type D: and hit enter this will move you to the CD-ROM drive if you hit and error it is because the drive is empty, not installed, or dead.  Type dir and hit enter to view for contents of the root directory on the CD.  There should be a file labeled something like SETUP.EXE it will end with EXE.  Type SETUP and hit enter.  (Yes I know I did write this for the more clever of the untrained but I do have to reach down to those less blessed with the arcane knowledge as well.)

You will then get to install your OS whatever it is.

Yes, a lot of software errors can be fixed by use of fdisk C: mbr and then format C: commands followed by a full reinstallation of software.  But this is a huge bother if you do not have the full resources of a burner, a network connection to file server, tape drive or zip disks to back up your vital data.

Errors beyond this point are highly individual and relatively unique save for continuous unrelated errors that point to viral infestations or flaky hardware.

Trouble shooting hardware in construction.

First you have the parts purchased or scavenged or stolen or given to you lined up in their boxes.

Take the case (The big metal thing) and look at it.  Open it up.  There should be a big box twice the size of a human heart in a corner of the box; this is the power supply.  Power flows from the wall socket through the cable to the power supply and from there to the motherboard and the drives.  There is a big empty place with little specialized holes drilled into protruding metal.  This is were the MB (motherboard) goes.  Mindlessly tap the metal of the case twice.  Make this a habit.  Whenever you stop to think or ponder tap the case thusly.  If you have been walking around and are about to pick-up a piece of internal hardware stop and tap the case.  This will maintain a neutral static charge and spare you some mistakes.

Take the MB (The big square thing with the complex patterns on it) careful to lift by the sides and not to bend it more than is needed.  Take the board to the case and compare the holes in the board to the holes in the case.  From a bag of supports select 3 or 4 of the white plastic ones that have a snap on thing on the top and a sliding piece on the bottom.  Take also one internal case screw (Smaller of the two kinds) a brass support (A male screw thing on the bottom and a female screw thing on the top [Co-workers will often screw them together]) and two felt washers (Little circles of fabric).  The side of the case with the slots in it is the back and the big piece of black plastic that the keyboard plugs into faces the back across from the opening in the case.  Slid one plastic mount in each of the back corners.  Screw the brass mount into the appropriate hole in the back area.  Hold the MB over the area again and see if everything lines up.  If not move the mount so it does.  If a hole in the middle of the board corresponds with a hole in the case put a plastic mount there.  If not cut the snapping part of the mount off and set the mount in the center of the area so the board can rest upon it.  With crazy glue affix the felt washers onto each side of the hole that corresponds to the brass mount.  Snap the MB into place and place a screw into the brass mount.

The MB is now mounted.  Take two of the multicolored cords that come from the power supply. The ones with the heads that make little sense and are flatter and wider than the rest.  They plug into the MB.  Remember Black to Black.

There should be a speaker in the front of the case plug it into the MB where 4 pins stand in a row and are marked “speaker” or “spkr.”

Take a video card and plug it into the MB.  Do not secure it.

Take a power cord and plug it into the back of the power supply and the wall socket.  Plug in a keyboard and a monitor (into the video card.)

Turn on the power and see what happens.

You should make it all the way to a non-system disk error.

If you do receive an error It will be with the components you have installed.  See the troubleshooting during boot section.

If it all works at this point rejoice.  Then find the hard drive(s), floppy drive and CD-ROM drive and install them into the case.  There are large metal boxes in the case that you can mount given drives into.  Use inner case screws to secure them in place.  Use all four on each drive.  Plug the cables from the power supply into the drives.  It should be pretty obvious the way they go.  Now you may need a controller card to install some of the drives into your computer but most new (last 3 years) computers have the controller card built into the MB.  Take some ribbons.  You will know which cables I am talking about when you see them.  The wide ones are for your hard drive(s) and CD-ROM and the skinnier ones are for your floppy drive.  The ribbons have a red coloured edge.  This is called pin one.  Remember pin one towards power (with the exception of Hewlett Packard floppy drives) as you plug the cable into the drive.  Plug each cable from there into the appropriate slot in the MB or controller card.  It should be clearly labeled FLOPPY, PRIMARY and SECONDARY.  Primary refers to your hard drive and secondary your CD-ROM both should be Master any additional drives can be slave.  Never chain the Hard Drive and CD-ROM together it will slow the Hard Drive down to 16 bit speeds.  Plug the ribbons in remembering the red side or pin one goes on the side that has A) the little 1 next to it, B) the little 2 next to it or C) the one closest to the point were power enters the MB.  Floppy cable may have a twist in it.  The end, closest to the twist in the ribbon, plug into the floppy.  If the floppy drive light goes on and does not go off, and the floppy drive does not function then the ribbon is installed incorrectly.  Turn on the beast.

Make sure the drives are acknowledged in the BIOS setup.  For an optimized boot up auto-detect any hard drives (Failing auto-detection you may have to read the drives characteristics off the drive it’s self and enter them manually) and set the CD-ROM to auto.  If your computer boots up properly then you have A) used a used hard drive, B) there is a boot disk in your floppy drive you did not notice or C) your computers bios can boot from CD-ROM and there is a bootable CD in the CD-ROM.  Failing these small miracles toss a boot disk into your floppy and access your hard drive. If it’s there rejoice if not you may have to set the jumpers on the back of the hard drive to Primary Master (Clearly labeled, mostly), reconnect the cables that go to it and from it.  You may have to partition and format the drive.  If the drives shinned their lights and spun as they where accessed during the boot then you have working drives.

Install the serial ports.  The little pieces of junk that came with the MB.  They plug from the MB (clearly marked spots) to the back of the case.  Plug your mouse into the appropriate serial port.

Turn on the computer and see if an error occurred.  If an error occurred you made a mistake with the serial ports.  Correct it.  The glory of computers is the ability to correct ones mistakes.

Install one of your additional cards and turn on the computer.  If an error occurs it conflicts.  Repeat this action till all cards are installed.

Using a CD-ROM boot disk boot up and using an OSCD install an OS onto your hard drive.  You should be clear of most problems save for IRQ conflicts!  You will have to either jigger with your software, change jumper settings, replace or discard parts to make it work right should such devastating problems occur.

This is it for usable hardware solutions that can be written down.  Now for some common sense.  CPU speed is almost unimportant compared to RAM, and video card speeds.  CPU’s can be over clocked to read as being faster but will actually test slower in most cases.  Over clocking also makes the chip run hotter than it should, reducing it’s life span.  Entire generations of hardware regardless of its’ maker can be faulty.  There is a big gap between 200 Meg and 2 gig hard drives that is filled with burnt out pieces of junk.  Do not slow your learning in any regard.  We have all heard of the old guy that decided he knew all he need know and has been caught with one night to learn a new complex OS or loose his job.  Needless to say this story has a sad ending in most cases.  No one can defeat a fresh opponent with so little time.

Servers.  Print servers, proxy servers and file servers.  A proxy stands between a network of computers and the Internet.  It holds a massive Internet cache and can make the Internet seem to go faster and free up bandwidth.  It permits the whole network to surf with one IP address.  This is handy when dealing with @home services and setting up multiple computers for the price of one.  Print servers take in all print commands from the network and print them chronologically.  File servers hold files that can be accessed from any computer in the network.  With more drive space and private access these servers are a must for any office.

To setup a server?  I have never done so and would need a few hours of research (or trying) before I would succeed.  But remember do not hesitate to experiment in matters of software.  Experimenting and failing will help you learn more than all the manuals in the world, even this one.

CD-Burners or CD-Writers, as the manufacturers say, are vital to Piracy, back-up files and mass presentations.  CD’s burn best at 1x or 2x never bother with anything faster it will just ruin the disc.  You will have to experiment with many kinds of burner software before you will find something that works most of the time.  A margin for error is expected.

Remember the rise of technology will make the world a more interesting place to live and that is the point.  We work not for money or esteem but rather make people feel free to accept new and complex technology into their daily lives because we will be there and we will make it work if it falters.