CNTR package version 3.00

A  package  for  discrete/continuous  control  systems

design/simulation. SX/GX compatible.

Written in UserRPL, SysRPL and Machine Language

Library number 1212

By Cesar Augusto Rorato Crusius - 1994

NOTICE.....................................................3

ABOUT......................................................4

LIMITATIONS and BUGS.......................................5

GX COMPATIBILITY.......................................5

BE AWARE...............................................5

Author's English Language Limitations..................5

VERSION DIFFERENCES........................................6

Version 3.00 and older versions........................6

INSTALLATION...............................................7

GETTING STARTED............................................8

ZDOM and SDOM..........................................8

FAST and SAFE..........................................8

Transfer function description..........................9

LIBRARY COMMANDS DESCRIPTION...............................10

CHGSGN.................................................10

FROOT..................................................10

NWTN...................................................11

BISSECT................................................11

S<->Z..................................................11

F<->S..................................................11

SLVG...................................................11

SLVtheta...............................................12

GM.....................................................12

thetaM.................................................12

F<w>...................................................12

SRCH...................................................12

CNVID..................................................13

STDANG.................................................13

ABOUT..................................................13

USER INTERFACE.............................................14

LOCUS directory : frequency domain.....................14

P, Q, ZE and PO...................................15

T.................................................15

KP+Q..............................................15

DATA..............................................15

SRCH, F<w>, oM, GM, SLVG and SLVo.................15

K.................................................15

thetaMAX..........................................15

SDOM directory : time domain...........................16

S->T..............................................16

R(T)..............................................16

STEP..............................................16

T(S)..............................................16

EQ................................................17

MO................................................17

page 1

TSTAB.............................................17

TRISE.............................................17

FRPRC.............................................17

APPLICATION NOTES..........................................18

Passing from S to Z state-vars description.............18

THANKS TO..................................................19

page 2

NOTICE

* The program CNTR 3.00 and the documentation in this

file are provided "as is", and are subject to change

without notice. I give no warranty of any kind with

regarded to the software or documentation, including,

but  not  limited  to,  the  implied  warranties  of

merchantability and fitness for a particular purpose.

Cesar Augusto Rorato Crusius shall not be liable for any

error or for incidental or consequential damages in

connection with the furnishing, performance, or use of

this software and documentation.

* Sale of this material is not allowed without prior

written permission of Cesar Augusto Rorato Crusius.

* Noncommercial distribution allowed, provided that the

whole package is distributed, the original documentation

is preserved unchanged, and any modified version are

cleary marked as such.

page 3

ABOUT

CNTR library is a program for control engineers. It

allows you to do inverse Z and Laplace transforms, to

project compensators and to simulate the system. You can

easily calculate the maximum overshoot, stabilization

time and many other system parameters.

Together with MATH capabilities, this program allows you

to do many tasks that took hours in just a few minutes!

page 4

LIMITATIONS and BUGS

Of course you can find equations that blow up my

program. If you find a *simple* system that my program

cannot deal with, send it to me and I'll see what I can

do.

I used this control package a lot during my graduation

control courses, and it worked very well. But now I

rarely use the programs. So I probably won't find any

more bugs by myself. You must tell me if you find a bug.

GX COMPATIBILITY

This program was tested on David Peterson's GX without

noticeable errors. I cannot, however, guarantee the

compatibility. Anyway, I think GX users can use this

package without problems.

BE AWARE

that TSTAB program may not work correctly if you put a

equation by hand in EQ. But it is almost sure that it

will work correctly for equations generated by the

inverse transforms programs.

Author's English Language Limitations

English is really not my native language. Actually I

speak A LOT of brazilian portuguese :-) There must be

many mistakes on this manual, but don't blame me!

page 5

VERSION DIFFERENCES

Version 3.00 and older versions

The earlier versions of CNTR worked with my almost

unknown :-( PMTH library, an ancient version of MATH.

They  did  not  have  inverse  Z  transforms,  and  some

commands are safer in version 3.00.

page 6

INSTALLATION

First of all, you must have MATH library (version 1.10

or newer) installed on your calculator. Without MATH

library you won't be able to use CNTR (and your memory

will probably blow up).

Now that you have MATH you can install your control

package. The package consists of two files: CNTR.LIB and

SDOM. CNTR.LIB is the library with many useful commands,

and SDOM is a "user-friendly" RPL directory that will

make your project & simulation job much easier. Just

install MATH.LIB, CNTR.LIB and download SDOM to your

HOME directory and you are ready to use the control

package.

page 7

GETTING STARTED

After the installation you must enter in the CONTROL

MODE (CM). But what the hell is the CM ? This package

performs many calculations depending on the current

domain of operation (Z or S domain). But you must know

in what domain you are working. So do the following:

(1) Go to the LIBRARY menu

(2) Go to the CNTR library

(3) Press NXT twice

(4) Press CNVID and you'll se two new annunciators

appear on the top of the screen. One of them

will indicate the current domain. The other will

indicate FAST or SAFE.

ZDOM and SDOM

All the package functions perform calculations depending

on the current domain. Example: the GM (Gain Margin)

command takes two polynomials and calculate the gain

margin CORRECTLY for the current domain. So, if you are

in S mode the program will search in the imaginary axis,

and if you are in Z mode the program will search in the

unity circle. To change between S and Z modes you must

use the library command S<->Z. The current domain is

controled by the user-flag 31 (yes, the same flag that

controls the type of partial fractions in MATH).

FAST and SAFE

FAST and SAFE acts on the way that some programs find

the root of an equation. Example: the gain margin

program  will  try  to  find  the  first  root  of

phase(P/Q)=180 degrees. It will try to find the root by

the following ways (to know what the commands mean, see

the LIBRARY COMMANDS DESCRIPTION):

FAST: CHGSGN { x1 x2 } -> NWTN

SAFE: CHGSGN { x1 x2 } -> make x1=0 -> FROOT -> BISSECT

As we can see, in most cases the program will find the

correct root in FAST mode. For *dirty* equations, where

there are more than one root or Newton method does not

converge, try to run the program in SAFE mode and

chances are you will get the right answer. My suggestion

is: work in the FAST mode. If you think your system is

very strange or that the answer is wrong, try to use

SAFE mode. FAST and SAFE mode is cotrolled by the user-

flag 32.

page 8

Transfer function description

A transfer function description consist of two numerical

polynomials  representing  a  polynomial  fraction.  The

polynomial variable will be S or Z, depending on the

current domain. We shall call P the nominator and Q the

denominator of the fraction. So, if you are in S mode

and your system transfer function is (s-2)/(s+2), P will

be [ 1 -2 ] and Q will be [ 1 2 ].

page 9

LIBRARY COMMANDS DESCRIPTION

In this section a complete description of the library

commands will be given. You can use any of the library

commands to make your own programs in user or Sys RPL.

What I'm trying to say is that I will NOT change the

library ID or the XLIB numbers. The only command that

can have it's XLIB number changed is ABOUT, but STDANG

can be removed anytime...

Commands marked with *SZ* works depending on the current

domain. Commands marked with *FS* works depending on

FAST/SAFE mode.

As said before, the root finders accepts the standard HP

ROOT values instead of the "program" input.

ALL THE FREQUENCIES are in rad/s, and in Z mode all the

frequencies will lie between 0 and pi, i.e., the sample

period will not be considered.

ALL THE ANGLES are given in DEGREES, no matter the

current angle mode.

You'll probably never user the library commands, 'cause

they are not very "user-friendly". I'm sure you will

prefer the SDOM user interface.

CHGSGN

*FS* Try to find an interval where the equation has

opposite signal. It will search only in the POSITIVE

direction of the variable. The first line of the display

will show the variable value, and the second line the

funcion value

Input                        Output

-------------------------------------------------------

2: program                   2: program

1: initial guess (real)      1: interval list {x1 x2}

FROOT

*FS* A GREAT program for engineers! Takes an interval

where the equation has opposite sign and returns a

narrower interval containing the FIRST ROOT of the

original  interval.  I  use  this  program  everywhere

(example: to find the phase margin we must find the

FIRST PLACE where P/Q have unity gain). The algorithm

was developed by me, and need some parameters. If you do

not give the parameters, default values are assumed (16

for Nmax and 1 for Prec%)

page 10

Input                        Output

-------------------------------------------------------

4: program                   2: program

3: interval list             1: interval list

2: Nmax (opt)

1: Prec% (opt)

NWTN

Try to find the root using Newton method. The variable

value is displayed on the screen. The program probably

will never stop (until you press ON) when the root is

zero.

Input                        Output

-------------------------------------------------------

2: program                   1: root

1: initial guess (real/list)

BISSECT

Try to find the root using bissection method. The search

interval  is  displayed  on  the  screen.  The  program

probably will never stop when the root is zero.

Input                        Output

-------------------------------------------------------

2: program                   1: root

1: interval list

S<->Z

Switch  between  S  and  Z  domains.  Current  system

description is not modifyed by this command. Uses user-

flag 31.

F<->S

Change between FAST and SAFE modes. Uses user-flag 32.

SLVG

*FS*SZ* Find the first frequency where the system has

the given gain.

Input                        Output

-------------------------------------------------------

3: Gain                      1: freq

2: P

1: Q

page 11

SLVtheta

*FS*SZ* Find the first frequency where the system has

the given phase.

Input                        Output

-------------------------------------------------------

3: Phase                     1: freq

2: P

1: Q

GM

*FS*SZ* Find the gain margin of the system (NOT in dB).

Input                        Output

-------------------------------------------------------

2: P                         1: gain margin

1: Q

thetaM

*FS*SZ* Find the phase margin of the system.

Input                        Output

-------------------------------------------------------

2: P                         1: phase margin

1: Q

F<w>

*SZ* Returns P(dv)/Q(dv). If you are in POLAR mode you

will see the gain and the phase of the system for the

given  frequency.  The  program  will  automatically

calculate the correct value for the domain variable (dv

-> can be s or z) for the given frequency.

Input                        Output

-------------------------------------------------------

3: freq                      1: P(dv)/Q(dv)

2: P

1: Q

SRCH

*FS* Starting from an initial point and going in the

direction determined by an angle, find the first point

on s/z plane that belongs to the system Root-Locus, and

the necessary gain to put the system in that point.

page 12

Input                        Output

-------------------------------------------------------

4: Initial point             2: Nec. gain

3: Angle                     1: Root Locus point

2: P

1: Q

CNVID

Switch between normal and control video mode. This

command changes the value of the user-flag 30.

STDANG

Give  the  equivalent  angle  between  +-180  or  +-pi

(depending on the current angle mode).

Input                        Output

-------------------------------------------------------

1: Angle                     1: Equivalent angle

ABOUT

Show my name :-)

page 13

USER INTERFACE

If we needed to put P and Q on the stack each time we

need to calculate some system parameter certainly we

would go crazy. So I made a user interface for the

programs. The interface resides on two directories with

the following structure

+------+ Time domain programs. Inverse S and Z

| SDOM | transformations. Time domain parameters

+------+ calculations.

|

|

V

+-------+ Frequency domain programs. User interface

| LOCUS | for the library commands. Root-Locus

+-------+ goodies.

In both directories you must work in CST menu. The VAR

menu will have a lot of variables and programs that you

don't need to use. We will describe first the LOCUS

directory.

LOCUS directory : frequency domain

In the LOCUS directory you will find the necessary tools

to project a compensator C for your system. Your system

must have the following form:

+      +---+  +---+  +-----------+

input --->O----| K |->| C |->| Plant (G) |---+-> output

A -  +---+  +---+  +-----------+   |

|                                  |

+----------------------------------+

You'll soon note that for some commands you can work as

if your system has a non-unity feedback using GH as a

description for the plant.

Another important thing is that the plant AND the

controller  must  be  normalized,  i.e.,  the  leading

coefficients of the polynomials must be one. Actually,

they really don't NEED to be one, but I STRONGLY

recommend that you use normalized plant and controller,

leaving all the system gains to the block K.

In  LOCUS,  the  plant  and  the  controller  will  join

together to form the polynomial descriptions P and Q. If

you are working in Z mode, you'll need to specify the

page 14

sample  period  in  order  to  calculate  the  inverse

transform of the system.

P, Q, ZE and PO

P and Q are polynomials describing your system GC. The

variable can be S or Z, depending on the current domain.

ZE and PO are the list of the GC zeros and poles

respectively. To update the values you MUST use, from

CST menu, the LEFT ARROW KEY in the same manner as you

use to store a common variable. This procedure will

assure that the values of the other variables will be

updated too.

T

This is only for Z domain. The variable must contain the

sample period.

KP+Q

Calculates the step response of the system for a given

gain. The gain is the block K on the figure already

shown. The input of the command is the gain value on

level  1.  KP+Q  will  perform  the  following  tasks

automatically: calculate the system transfer function

for the given gain, add the S or Z step transform to the

function, calculate the partial fractions and find the

inverse transform of the answer. KP+Q will switch to

SDOM directory and store the answer in EQ.

DATA

Calculate the essential data for drawing the Root-Locus

of the system. The data includes the asymptotes angles

and intersection point and the I/O points with their

respective K.

SRCH, F<w>, oM, GM, SLVG and SLVo

Interfaces for the same name library commands. The

difference is that you don't need to put P and Q on the

stack anymore.

K

Find the roots of the closed-loop system when K is equal

to the value on level 1.

thetaMAX

Only for S domain. Find an estimation for the maximum

angle for the dominant pair of complex poles if you want

page 15

a maximum overshoot of (level 1)%. The angle is measured

anti-clockwise from the NEGATIVE real axis.

SDOM directory : time domain

All the commands for inverse Laplace transforms work for

inverse Z transforms too. But for inverse Z transforms

you need one more parameter: the sample period. The

sample period must be the FIRST ARGUMENT (the higher

stack level).

S->T

Performs the inverse Z/Laplace transform of a fraction.

Q must have only one zero (or a pair of complex zeros).

You can do the inverse transform of P/(Q^n). If you

don't specify n, it is assumed to be 1.

Input                        Output

-------------------------------------------------------

4: Sample period (Z only)    1: Inverse transform

3: P

2: Q

1: n (optional)

R(T)

Calculate the time response of a fraction. The inputs

are the same as for MATH function FRPRC. The difference

between R(T) and S->T is that S->T calculates the

inverse trafo only for one fraction. R(T) calls FRPRC

and execute S->T for each fraction, adding the results.

The final answer is stored in the EQ variable.

Input                        Output

-------------------------------------------------------

3: Sample period (Z only)    1:

2: P

1: Q in FRPRC style

STEP

Calculate the step response of a transfer function

(open-loop system). The inputs are the same as for R(T).

The result is stored in the EQ variable

T(S)

If your H(s/z) is not equal to one, then you may use

this program to get the system transfer function. No

pole/zero simplifications are made

page 16

Input                        Output

-------------------------------------------------------

4: Num(G)                    2: Num(T)

3: Den(G)                    1: Den(T)

2: Num(H)

1: Den(H)

EQ

The variable where literal solutions are stored for

later use.

MO

Find  the  maximum  overshoot  of  the  system  response

equation stored in variable EQ. Take no inputs.

TSTAB

Find the stabilization time of the system response

equation stored in variable EQ. You must put on level 1

the number of time constants you want to use to find the

stabilization time.

Input                        Output

-------------------------------------------------------

1: # of time constants       1: stab time

TRISE

Find the rise time of the system response equation

stored in EQ. The rise time is the time that the system

takes  to  go  from  (level  2)*final  value  to  (level

1)*final value.

Input                        Output

-------------------------------------------------------

2: see above                 1: rise time

1: see above

FRPRC

The MATH command FRPRC

page 17

APPLICATION NOTES

Passing from S to Z state-vars description

If you have a S-dominium state-variable description of

your system and want to transform it to a Z-dominium

description, you know that you must find the phi matrix

and do some calculations. We know that

phi(t)=inverse laplace of (1/(sI-A))

With the control package you can find phi with the

following RPL program stored in SDOM directory:

<< [1 0] SWAP MSUB MINV -> p << << p RTEMP >> PR|L >> >>

Input                        Output

-------------------------------------------------------

1: Acontinuous               1: phi(t)

Now, with phi(t) you can easily find Adiscrete and

Bdiscrete (the C and D matrices remain the same). As we

know, Adiscrete = phi(sample period) and Bdiscrete =

integral(0,sample period,phi(t),t)*Bcontinuous. For more

details, see the book DIGITAL CONTROL SYSTEM - ANALYSIS

AND DESIGN by Phillips & Nagle, page 144.

As an example, find phi(t) for the following A matrix:

[ [ 0 1 ] [ 0 -1 ] ]

The answer will be:

{ { 1 '-EXP(-t)+1' }

{ 0 'EXP(-t)' } }

To find Adiscrete, store the sample period in the

variable 't' and execute, with phi(t) on level 1, the

MATH command A<->L (only for 1.21 or earlier versions of

MATH). If you have an older version of MATH you can

still find Adiscrete executing << ->NUM >> PR|L before

execute A<->L.

page 18

THANKS TO

I want to thank the following people for their support

and suggestions:

Joe Horn

Jurjen bos

Mika Heiskanen (SAD, ML stuff and many things)

David Peterson (help on PMTH and beta-tester of CNTR)

page 19