THE SWAN NEWS

May 1996 - Issue 7

FROM THE MANAGER

For many of you this is probably the first time you have seen the Swan News,

and for many others, this is probably the first issue you have seen for some time!

It should be noted that this issue is in your hands due to the unselfish help

of other Swan enthusiasts, namely Monte/N7OQVand especially Stu/K4BOV.  Stu has recruited the folks at Boddicker Electronics to partially "sponsor" our little publication, and we all owe a debt of gratitude to them.  The next time you

talk to either of them, pass along your thanks.

For those of you who have received this publication before, you will note

that I have not included the list of users.  It turns out we have grown to

such an extent that to include it with the newsletter has become impractical.

We have about 1000 active Swan collectors on our list, so if you want a

list, I would be happy to provide one for you if you send an SASE to WA9AZK @ 3256 East Bell Oaks Circle, Sandy, Utah 84092.  I would appreciate a few

pennies for duplication costs.

And speaking of pennies, if you appreciate this publication and would like to

receive more, a small donation of a few dollars Stu Morrison/K4BOV will help.

(NOTE: 4/99 - Only those wanting an original copy mailed need to consider any donation.  Anyone ever donating in the past, need not donate again.  The e-mail text version is now available to anyone.) Many of you have generously donated to this publication to WA9AZK in the past, and be assured that your donation is on record and being applied toward this and future issues.

******************************************************************************

BEAT THOSE VFO DRIFT WOES

By K4BOV

By mid 1965, anyone comparing ham gear based on price and performance,

easily saw the Swan 350 as an exceptional buy. And not surprisingly, the

Swan 350 soon became the hottest selling transceiver of that period.

Even to this day the model name "SWAN 350" is still the most recognizable of all Swan products.

However, within a year or two of regular use a lack of frequency stability began to appear and gradually worsened to a point of frustration and embarrassment to the user. Although increased attention was directed toward VFO integrity in later Swan equipment, the same problem tended to develop. Most attempts at the user level to thwart this nuisance instability involved changing anything and everything in the VFO frequency determining network. Unfortunately, this effort rarely achieved complete success.

So, why was it that Swan amateur equipment built a reputation of having

poor frequency control while others of the same era did not? Plain and

simple - the Swan VFO needed to produce four (early Swans) or five (late

Swans) selectable widely spaced frequency ranges in order to cover the

five different ham bands with a single conversion system. Other leading

equipment of the day employed a separate crystal converter for each band

which allowed for use of a single range VFO typically covering a 500KHZ

spread of 5.0 to 5.5MHZ.

A single range VFO requires no switching while the multi-range Swan VFO

must be switched from band to band and is therefore ganged to the main

bandswitch via a flexible mechanical link. Under perfect conditions, one

section of the VFO rotary wafer switch (located inside the VFO

compartment) selects the correct VFO range corresponding to the desired

ham band in use. The other remaining section grounds out the

"not-in-use" VFO ranges so as to prevent any unwanted signals from

getting into the scene. Because the switch is actually connected to

components of the tuned L/C circuits, IT becomes a part of the overall

frequency determining network. Remember, these types of oscillators are

very very sensitive to outside forces. Just moving your hand close to an

open VFO compartment will shift the frequency.

Through use and time, various actions result in weakening the positive

contact areas of the VFO switch. Dirt, wear and corrosion on these

surfaces introduce resistance and capacitance changes. And, it doesn't

take much to cause the observable frequency (what you hear coming in and

what the other members of your QSO hear coming out) to jump, warble or

drift-off. Just slightly moving the VFO switch by applying pressure

either left or right on the front panel main bandswitch knob will cause

frequency movement.

What can be done? Well, it's not always easy. If cleaning the VFO switch

does not reduce the problem, a new, unused or otherwise unworn switch of

exact contact layout should be substituted. After disconnecting the old

switch you can prove to yourself that the switch was at fault by

hardwiring one band at the points previously made by the switch

contacts. This simulates a single range VFO. Under this set-up any drift

should be minimal and if evident is merely warm-up related. Should

correction of warm-up drift be considered essential, positive and

negative compensating capacitors are available to "tighten" things up.

By selecting 80 meters for the hardwiring test on the earlier 5.1745MHZ

I.F. transceivers, the 20 meter band is automatically operational as

both bands utilize the same VFO range. Additionally, when connecting 40

meters in this manner, the later 5.5MHZ I.F. equipment can be easily

aligned to operate on 17 meters as both bands use the same VFO range.

(The green scale on late Swan VFO's is already calibrated for 17 meters)

In the event changing a worn VFO switch is not an easy task for the

user, an external VFO is the next best move. Most Swan external VFO's

did not develop any serious drift problem. And, any external VFO

manufactured between the time of the SWAN 240 and the SWAN 700CX can be made to operate any Swan of that period, even the 600T and 600R twins.

Some being directly interchangeable and others requiring one or more

minor wiring changes. Some pruning of the tuned circuit coils and

calibration is also required when using the late 5.5MHZ I.F. designed

VFO with early 5.1745MHZ I.F. transceivers and visa versa. When using an

external VFO with any drifting Swan, the control circuitry of the model

22(B) VFO adapter and similar circuitry built into later VFO's should be

reverse wired for optimum results. That is, you want the external VFO to

be "A" and the internal "B". This means the highly stable external VFO

is transceiving in position one and is the transmit only VFO in position

two. So when operating split frequency, the internal VFO is receiving

and therefore any drifting on that VFO is only noticeable to the user

who can re-adjust the VFO to his own liking as needed without affecting

the transmit frequency.

Crystal control is another option. All Swan crystal controlled external

oscillators can be made to operate any of the equipment discussed. These

units hold up to 10 crystals and employ a vernier control to flex the

crystal frequency over several KHZ for tuning refinements. Since

production of Swan crystal oscillators was much much lower than the

variable oscillators, they are somewhat hard to come by. However, all

Swan VFO boards are convertible to crystal control and can be outfitted

with the same type vernier as a means of tuning the crystal over a 4 or

5 KHZ range. Plus, remember about early Swans, the 80 and 20 meter bands

employ the same VFO range. Thus one crystal would get you on both bands.

For example, the crystal required for operating around 3920KHZ. And we

say "around" because the exact frequency is factored in only when we know

the precise setting of the carrier oscillator and whether normal or

opposite sideband is desired. But, we can get close enough for the

vernier to put us right on.

Calculate as follows: 3.920MHZ + 5.173MHZ carrier oscillator frequency =

9.093MHZ crystal frequency. Looking at that same crystal we also see the

20 meter frequency: 9.093MHZ + 5.173MHZ = 14.266MHZ operating frequency.

When properly maintained and aligned to factory specifications, Swan

audio quality is very difficult to transcend by any manufacturers

amateur equipment past or present. And, it would be a shame to see these

handsome rigs spend any more time in those dark hideaways just because

that pesky drift appeared too tough to tame.

Stu/K4BOV

******************************************************************************

The original manufacturer of Swan crystal filters is:

K & L Quartztek

20 South 48th Avenue

Phoenix, AZ 85043

(602) 272 7944

Other manufacturers were used later.

******************************************************************************

HOW TO MATCH SWAN FINAL AMPLIFIER TUBES

By Dean/WA9AZK

Harry/N7DOE has expressed an interest in seeing this article in the "Swan

News." And, Harry was even gracious enough to type the steps as follows:

1.   Disconnect the power supply from the radio.

2.   Remove the main top cover of the radio.

3.   Remove flat cover over final tubes.

4.   Put (or leave) final tubes to be matched in radio.

5.   Connect radio up to a dummy load.

6.   Connect radio up to power supply. "BE VERY CAREFUL."

7.   Tune radio up as usual into dummy load. (any frequency is OK)

8.   Push PTT and set resting current to say, 100-150 ma (not to critical).

Don't touch setting again until all testing is done.

9.   Disconnect power supply. "THIS IS IMPORTANT."

10.  Disconnect "one" of the two plate connectors hooked to the top of the

final tubes. (note which one it is)

11.  Connect Radio to power supply again.  "BE CAREFUL"

12.  Turn radio on and let warm up. (2 or 3 minutes)

13.  Push PTT and note new resting current.  It should be less than previous

setting.  Write it down.

14.  Disconnect power supply again.  "BE CAREFUL"

15.  Hook up plate connection on final tube where it was previously removed,

and remove the (other) connected that was connected.

16.  Connect radio to power supply ("BE CAREFUL"), turn on radio and let it

warm up again.

17.  Push PTT and note resting current.  Write it down.

18.  Do this for all final tubes.  Matched tubes should have resting currents

within about 10 ma or less of each other for best results.  If they don't,

keep trying other tubes until a match is obtained, or get a matched pair

elsewhere.

(Note:  See K4BOV's article in issue #6 of the "Swan News" for additional

explanation and understanding about matched tubes.

Dean/WA9AZK

******************************************************************************

TOOLS, TOOLS, TOOLS..

Believe it or not, this section of the Swan News is one I generally receive

perhaps the most positive comments about.

I guess we all want to have a better way to work on our old treasures, and

perhaps since I have been in the tool business for over 20 years, it's an

area that interests me quite a bit, too.

But, looking at back issues of the Swan News I see I have given up about all

the tool tips I can think of.  Perhaps you have some to contribute?  There

are; however, a few more I would like to pass along.

One tool I use frequently is one of those promotion type screwdrivers, often

given away as advertisements, on which I place shrink wrap tube over the

metal shaft so when I neutralize finals I don't short out the screwdriver

against the bottom cover on the rig.

And, another "tool" I use frequently is a good quality, small flash light.

While this is not technically a tool, it is sure necessary for the serious

Swan repairer!

Dean/WA9AZK

******************************************************************************

MORE POWER, WHO NEEDS IT?

By  WA9AZK

The other night a recently licensed friend was over checking out his Swan 500

that I had done some work on.  I spent a few minutes explaining to him about

the relationship between power and signal strength.  I reminded him that a

doubling of power represents about 1/2 S-unit (3db to be exact).

Understanding this and still insist upon using a linear amplifier on the back

side of your already powerful Swan, it is a good idea to have ALC (has many

phrases; but, Automatic Level Control will do) hooked up.  The powerful Swan would otherwise overdrive most linear amplifiers.

To add an ALC circuit, simply install a 100Kohm 1/2 watt resistor in series

with pin #1 of the 6EW6 (12BA6 later units) 1st I.F. Amplifier.  Then install a .01 uf capacitor between the output side of the resistor and ground.  An RCA jack is then mounted on the rear with the center conductor connected by wire to the junction of the resistor and capacitor. Hope it helps keep the splatter down!

(Drawing is displayed on original page - If needed send SASE to WA9AZK)

Dean/WA9AZK

******************************************************************************

******************************************************************************

NETS OF INTEREST TO SWAN COLLECTORS

Name               Day     Frequency   Time                Control

State

Swan User        Sun     14.250Mhz   5PM Eastern   WA9AZK      UT

Swan Tech        Wed    14.251Mhz   2300 GMT       K4BOV        MD/NY

Swan Tech        Sat       7.235Mhz   2PM Eastern   K4BOV        MD/NY

Vintage SSB     Sun      14.293Mhz  2000GMT       WB0SNF      NE

Note: You will note a small frequency change for the Wednesday Swan Technical Net.  As of July 1995, we have moved up one Kc from 14250 to 14251. Earlier in the year we had a few contacts with Alan/WA4JTK in Florida who operates a DX calling net on 14247Kc.  From time to time, our check-in

stations, as well as net control, are strong enough to cause interference

to their operation and vise versa, more so to them as we are on their high

side.  Thus, we've agreed to move up one Kc to give them more cushion.

Stu/K4BOV

******************************************************************************

PERIODICAL ARTICLES FOR SWAN RADIOS

(Original copy contains chart displaying Title, Source and Date of

publication for 94 separate Swan related articles on modifications, reviews,

service bulletins. Many of these articles are held in our archives by

Norm/W7RXG.  If you desire this list, (all on one page) send SASE to K4BOV.)

******************************************************************************

SWAN SSB TRANSCEIVER

(Picture of 1961 Swan Single-band Transceiver)

MOBILE - PORTABLE

HOME STATION

Manufactured by Swan Engineering Company

A Revolutionary New Design

Swan Engineering provides single sideband

communication at a surprisingly low cost.  The

one-band design gives exceptional, high quality

performance in all respects on the chosen band.

The following models are available:

Freq. Range           Sideband

SW-175

3.8 - 4.0 mc.                 Lower

SW-140

7.2 - 7.3 mc.                 Lower

SW-120                                  Net weight 11 1/2 pounds

14.2 - 14.35 mc.             Upper

SW-115

21.25 - 21.45 mc.           Upper

$275 Net Price, from authorized dealers only

Matching 12 Volt Power Supply $99.50

Rugged High Quality Construction

*  Simplified circuitry provides an unusually high degree of reliability.

*  180 watts PEP input to 6DQ5 Power Amplifier.

*  High Frequency crystal lattice filter; 3 Kc. nominal bandwidth,

used for both transmit and receive.

*  Unwanted sideband down approximately 40 db. Carrier suppression

approximately 50 db.

*  Transmits automatically on receiving frequency.

*  Exceptional mechanical, electrical and thermal stability.  Frequency is

practically unaffected by voltage or temperature variations, or by

vibration when driving over rough road.

*   Receiver sensitivity less than 1 micro-volt at 50 ohm input.

*   13 1/4 inches wide, 5 3/8 inches high, 11 inches deep.

*   Smooth audio response from 300 to 3,000 cycle provides

excellent voice quality for both transmitting and receiving.

*   Control system designed for greatest ease of mobile operation.

Front panel controls include:  Main Tuning, Volume, Carrier Balance,

Microphone Gain, Exciter Tune, P.A. Tune, P.A. Load, T-R Switch,

Supply On-Off Switch, and Tune Switch.

*   Main Tuning control is firm and smooth, with 16:1 tuning ratio.

Calibrated in 2 Kc. increments.

*   Transceiver produces approximately 25 watts carrier output on AM

by simply adjusting the Carrier Balance control.  Receives AM signals

very satisfactorily.

*   3-Circuit microphone jack provides for Push-to-Talk operation.

*   Mounting bracket and power plug included.

POWER SUPPLY REQUIREMENTS

*   275 Volts DC, nominal, at 100ma., receive and transmit

*   600 Volts DC, nominal, at 25-250 ma., transmit only.

*   - 90 Volts DC, negative bias, at 6 ma., receive and transmit.

*   12.6 Volts AC or DC at 3.45 amperes, for filaments.

MATCHING DC POWER SUPPLY

Swan Model SW-12A

Provides full power to transceiver from a 12-13.5 volt automotive

system.

Net weight 7 lbs.; shipping weight 9 lbs.  Complete with mounting

hardware,

cables and installation instructions.   $99.50

SIZE:  4 inches by 4 3/8 inches by 6 1/2 inches.

SWAN Engineering Co.  Benson, Arizona