THE SWAN NEWS

December 1998. . . . . Issue 9

FROM THE EDITOR:

Happy holidays to all our Swan radio collector friends.  We hope this

newsletter finds you all in good health and enjoying your holiday season.

Again, we want to thank all those who have been so helpful keeping our fellow

Swan enthusiasts on the air.  A special thanks to Stu, K4BOV for all his help with the nets, the newsletter and all of the many other contributions he has made

on our behalf.

Since there is a limited amount of subject matter that can be dealt with

regarding the care and feeding of our radios (old and new), we will include

solutions to some of the more common and most encountered problems associated with our particular type of radio, and general amateur practices that can improve our radio experiences.

Therefore, you will find not only "Swan specific" solutions and suggestions

in this issue, you will also find ideas about things you can do around the

shack or in the mobile to keep things "happy."  One example of that will deal

with antennas and grounding systems.  A subject I think we have all dealt

with, with varying degrees of satisfaction.  You will also find a very

informative article by Stu, K4BOV about helping yourselves in the event you

must troubleshoot your rig(s).

Hope you enjoy this issue, and again, happy holidays to everyone.

******************************************************************************

TIP:  WATCH OUT FOR PIN 6 ON YOUR 6GK6 DRIVER TUBE.

Way back in issue #1 of the "Swan News"  I wrote a brief article on the

importance of disconnecting any wires from pin #6 of the 6GK6 Driver tube socket.  In the early days, Swan used this tube socket connection as a sort of terminal strip that made its way to ground.  Well, it turns out that later versions of the 6GK6 tube tied that pin (internally) to the screen grid.  Not so good!  If your pin 6 is tied to ground, and you replace a driver tube with one of these newer style tubes (only about 25 years old), you will take out a 100 ohm resistor, an RF choke and the new tube!

Therefore, each time you acquire a "new" rig, or work on an old one or a

friend's rig, take the 30 seconds it takes to snip any connection to pin 6

and isolate it totally.  Okay?

Dean Lang, WA9AZK

******************************************************************************

KNOW THY SWAN,  Part II.  by Stu Morrison,  K4BOV

Previously, we discussed the importance of keeping on hand a complete record

of circuit parameters that exist when your radio is operating properly.  And, we

indicated that, after tube checking and visual inspections for breaks, burns,

etc, the most basic nonhazardous approach to troubleshooting was resistance

measuring.

Further, a resistance chart must be prepared by the owner because such aids

where not included in most Swan manuals.

Now we will focus on the next step which involves voltage measurements.  All

Swan manuals did include a voltage chart; however, the accuracy of the values

could not be fully relied upon due to many factors such as the variation in

power supplies and capacities, and the many production changes to equipment

that resulted in completely different circuit paths.

It is known that voltage charts were not always updated to accommodate later

runs of the same model radio.  Additionally, the effects of accessory and

modification loading was seldom addressed.  With many of these radios being

over 30 years old, there are few that have escaped the wrath of repairs,

modifications, etc.; and thus, a new voltage chart must be prepared.  It can

be patterned after the manual version; but, additional check points should be

added.  For the purists, this would mean a space on the chart for the VOX and

VFO socket and 12 pin Jones plug.  It's a good idea to make up several copies

of the blank voltage chart for any possible down the road tests.  Always

affix the model and serial number of the transceiver and power supply

somewhere on the associated chart - even the date of test is good for record

keeping.

With all clerical preparations complete, the next consideration is with

safety and accuracy.  Studying voltage levels in vacuum tube radio circuits

can be dangerous.  Therefore, extreme caution is mandatory during the voltage

measuring process.  For some, it will be necessary to review safety

precautions connected with the handling of open electronic equipment and use

of multi-meters.  If one is the least bit apprehensive about a face-to-face

encounter with the voltages awaiting, it would be wise to seek the assistance

of someone experienced in electronics troubleshooting, as the information

here would not be sufficient to provide adequate guidance on important safety

measures.

The intent here is to bring to light the need for having the circuit voltages

or "vital signs" so to speak, of your properly working radio down on paper in

the event you need to compare "good" radio with "broke" radio.  Finally, a few pointers.  The plate voltage to the Power Amplifier (finals) stage is applied at all times in Swan transceivers when the power is turned on regardless of whether in

receive or transmit mode.  Never remove the PA cage cover with power on.

Think of it as a "rattlesnake in a cage."  Of course, there are exceptions to

any rule - most skilled and long time technicians are careful enough to work

without the protective cover.  But for now, we are measuring, not working -

so, keep the lid on it!

Next, is accuracy.  That's the whole idea, to refine the values provided in

original Swan voltage charts to match those unique to the current radio

readings.  This means meter accuracy has to be without more than say 5% error.

Without precision calibration equipment, some Swan transceiver circuits can

be employed as standards.  For instance, the early Swans employed voltage

regulator tubes to keep carrier oscillator tubes stable.  The OA2 in

Swan-350, 400, 500 and early 500C's and 350C's for example, can be used to

set your meters at 150VDC.  The VFO zener diode that provides the regulated

-10VDC for early Swan VFO's and    -12VDC for later units can be relied upon for calibrating the lower voltage range of voltmeters.  Make sure which diode you are dealing with though, as many of these have been interchanged or

replaced by other values.  The Swan VFO does not care whether -10VDC or

-12VDC is in place regardless of radio vintage.  In fact, anything from -9VDC

to -15VDC will work fine - it's the voltage regulation that counts.

It's good practice to make all measurements a second time on another blank

sheet; then, comparing the two.  Since there nearly 100 possible check

points, it's not hard for even the best of us to make an error.

Happy Holidays,

Stu/K4BOV

******************************************************************************

P.S.  Stu wants all of you to know that he will be happy to help you with

your technical questions.  You can FAX him at (607) 936 6925 or E-mail: Radyojo@Angelfire.com Be sure to be specific about your problem and

provide model and serial number.  Also, indicate what accessories you may have

hooked up, etc.

******************************************************************************

SOMETHING FROM THE VAULT

or

A Suggested Reprint From Past Issue #3

SO YOU WANT TO GO OFF BALANCE

Something that comes up from time to time has to do with the way Swan

designed various circuits with regard to off-setting the carrier balance when

the radio is placed into the TUNE position.  I have found that the 350's and

the 500's (not C's or CX's) required the operator to "inject" some carrier by

off-setting the carrier balance if any power is to be generated in the TUNE

position.  This is inconsistent with the majority of the other Swan radios

and sometimes is confusing.  It recently happened to me.

It is easy to forget to null out the carrier after this is done, too.  So,

what can you do to make this more consistent with the other rigs?  Simple.

Install a wire from the REC/TUNE wafer switch (usually the only unused

terminal) to pin 9 if you have a 7360 or pin 1 if you have a 6JH8 balanced

modulator tube.  This will "take one of the deflection plates to ground" and

fully off-set the carrier balance when you throw the old baby into TUNE.

This might help eliminate those reports of "I hear carrier on your signal."

If you operate CW, your rig will put out ONLY full power which may be a bit

hard on the finals and create other problems, but if you are like me (a phone

freak), this will probably be a good solution.  The best approach to this was

what was done on the 700CX with different switch positions for CW and TUNE.  There are many schematics available to refer to for this design should you have questions.

Dean/WA9AZK

******************************************************************************

MORE ON GROUNDING

As indicated, lets talk about some good general rules for our stations.  The

topic of grounding comes up from time to time, and it is certainly one of the

most important aspects of equipment installation.

Norm (W7RXG) and I were having a discussion about this topic the other

evening and Norm pointed out a couple key points about grounding.

Think of ground as a place, a large receptacle, for electrons to flow and

disburse.  Also, in the event of RF energy, ground holds a lot of the same

role.

Since, when we transmit, we subject ourselves, our surroundings, and our

equipment to relatively strong electrical fields.  Improperly protected from these

fields, all sorts of strange things can occur.  The most consistent one of

these is audio distortion.  This usually occurs because the strong RF field

enters in any number of places in our communications system.  It could be

introduced into one of the stages of our transmitter, through our

microphones, back through our coax, or even through the electrical system

that provides power to our radios.  (115VAC lines). So, how does grounding

prevent these problems from occurring, and what is the role of grounding?

Grounding plays more than one role.  First, by having a good low resistance

ground for all your equipment (including your antenna), you provide a better,

less resistant path for excess electrical energy to be dispelled into the

ground.  I'm talking about lightning.

This brings up a comment I have so often heard over the years.  "A well

grounded antenna system is more likely to be struck by lightning, so why

improve the chances of a strike by grounding?"  Probably true, but look at the alternative!  Think of it this way:  If I drive without insurance, I will be more careful and, therefore, I will be less likely to have an accident.  Not a good choice.

The other thing grounding does is "bleed" off RF.  If your rig and associated

equipment is properly grounded, you will send the powerful RF, that so much

likes to get into everything, on a trip to hell.  So if you have distortion

in your audio, or get "bit" by that old D104, then look to a better ground

for your solution.

What is a better ground?  Here there are many opinions, but one thing can't

be debated; the bigger (such as in wire gauge, pipe size, etc.) the better.

When I say "big", I mean less resistance.  So, don't be afraid to use a great

big piece of major gauge wire for your ground, and the further away from the

ground, the more important this becomes.

It is probably true that multiple grounds are a good idea, too.  What do I

mean?  Think of it this way.  If your rig is throwing off a lot of RF and

your ground is sending that RF to a cold water plumbing system within your

house, chances are everything that is electronic is hooked to that same

ground.  what if the piping system is a wonderful conductor?  And copper pipe

is a wonderful conductor.  And what if the ground portion of that system

isn't perfect, but the circuit is?  Get the picture?  Not only that, most

telephone systems are also connected to the plumbing system.

So, theoretically at least, a separate ground for your radio, your antenna,

the phone and the electrical system in your house would be less likely to

"cross-feed."

I have actually experienced that situation and by relocating a ground

connection, I eliminated any annoying hum in my phone system.  Presumably my

regular electrical system was feeding back into the phone system through the

plumbing system.  Once relocated, the problem disappeared.  And, how many of us have experienced mic bite because we weren't properly grounded?

Do you have any grounding/RFI experiences you can share with us?  Let me know.

Dean/WA9AZK

******************************************************************************

A FEW WORDS ABOUT THE INTERNET

Well, you just can't get away from it . . .The Internet.  Often I think hams

feel the Internet is a negative thing for our hobby because it somehow

competes with it. I guess that can be true, but I think properly used, the

Internet is a great addition to our hobby.  I know I have found it to be very

useful and a great help to fellow hams all over the world.  The other day I

downloaded the instruction manual for my new (used) A-3 Cushcraft antenna.  I

also received Stu's wonderful article published in this issue over the

Internet.

Why not give it a try if you haven't already?

Dean/WA9AZK

******************************************************************************

SILENT SWANS

I sure wish I didn't have to include the following information about some of

our fellow Swan enthusiasts who are no longer with us; but, I know many of you would  want to know where these great guys are.  Wish I were providing better news.

Lost to the Ages

W3WAO        Charles Jose           Towson, MD

KE2NO          Henry Herrlich          Corning, NY

N1BVS          Ken Erickson           Bethel, CT

W2YL            Joel Young              Elmira, NY

K4IEE            Mel Clark                Lynchburg, VA

W8QYW        Homer Crozad         Geneva, OH

KA2VQX        Joseph Smith          Painted Post, NY

W2VW          Joseph Meyer          Elmira, NY

NB3C            Fred Carr                 Myrtle Beach, SC

******************************************************************************

Current Swan Net activity schedules and other Vintage Radio Nets can be found

at: <A HREF="http://www.geocities.com/latemod97">Swan Radio Network

</A> or http://www.geocities.com/latemod97 and

******************************************************************************

A FEW OTHER SITES FOR SWAN EYES......

Our own Gary, VE4YH - see his Virtual Swan Museum at:

www.pcs.mb.ca/~standard/

Take a walk through AADE's Swan Radio Room at:

www.aade.com/hampedia/swan/swan.htm

The Star-Gazette newspaper in Elmira, NY and serving Corning, NY, was the

first Gannett news agency in the USA.  Find the news and weather conditions for the QTH of K4BOV at

www.star-gazette.com

******************************************************************************

SWAN RADIO TRIVIA...........a must for your personal files.

Swan 350 Front Panel:

Watch out!  Front panels from late 1967 Swan 350's are not interchangeable

with earlier panels and vice versa.  The PA Plate and Course Load controls are

mounted higher on the early units.  If a non-matching panel MUST be installed, new shaft holes will need to be drilled in the panel.

Swan 400 - Where'd they go?

The Swan 400 was Swan's first 5 band amateur transceiver.  At the time it

went into production, the Swan 350 was already on the drawing board.  The 400 sold well until 6 months later when the 350 was released.  The rage at the time was for transceivers that employed built in VFO's; thus, the 350 quickly out paced the 400, as an external VFO was required for the latter.

Even though the 400 had a slightly longer production period, only about 1200

units were built compared to 9000 of the popular 350.  At the end of 1967

both models were discontinued to make room for the new 350C and 500C

transceivers.  But, the 400 was to good of a radio to retire completely.  With a few minor changes in order to accommodate nontechnical operators and operation outside the ham bands, the 400 found service with military, industrial and commercial customers.

Sales to those groups continued for over 6 years as the 400, initially dubbed the 400E; then, the 400F; 400G; and, finally the 400H, proudly served in silence.

While the commercial 400's went completely unnoticed by the amateur ranks, it

is still equally unknown today, that the original 400 design had actually

followed right through to the Swan HF700S - the last tube radio Swan produced.  So, where'd the commercial 400's go?

Well, they are starting to turn up in flea markets, government auctions,

military surplus, etc.  These transceivers need only a Swan 117X or similar

power supply - no speaker needed, as the 400 has a built in speaker.  Specifications include continuous frequency coverage 2 through 24Mhz and limited tune-up requirements.  In reality, this unit will cover 160 through 12 meters without any modification.  Any Swan external VFO can be retuned to provide the necessary injection frequencies.  Unfortunately, commercial use means commercial treatment.  Therefore, it is difficult to come across any of these transceivers that have retained their original pleasing appearance.

At any rate, should any of these radios become a part of your inventory, the service manuals are still available from Brock Publishing.

73,

Stu/K4BOV

******************************************************************************

Current Swan Net Activity Schedules are as follows:

Swan User          Sunday          14.250      5PM eastern      WA9AZK   UT

Dean

W7RXG     UT   Norm

WA5BDR  NM   Jim

Swan Tech          Wednesday    14.251     2300GMT           K4BOV     NY

Stu

Swan Tech          Saturday           7.235     2-4PM eastern    K4BOV    NY

Stu & K1QQ      NJ    Kent

Other vintage equipment nets:

Collins Tech         Tuesday          3.958     8-9PM eastern    W3ST

PA   Dave

Hallicrafters          Saturday         7.280     1700GMT           WB8DML

OH   Jim

Hallicrafters          Sunday          14.293    1800GMT

Vintage SSB        Sunday          14.293     2000                   WB0SNF  NE

Andy

Heathkit               Sunday          14.293     2130GMT           WB6LRG

CA  Don

Periodic updates and further information can be found at:

http://www.angelfire.com/ny2/hamradio

http://www.geocities.com/latemod97

******************************************************************************

Swan Net Notes:  With the dramatic increase in Swan User Net session

check-ins, net control will now call for check-ins by time value needs,

mobile operation and by QTH area.

Also, with the advent of computer operations and e-mail, we are not actively

solicitating donations for the Swan News periodical.  We will try doing it on

the revenue from the publications and products available through our web

pages.

Feel free to forward the Swan News to those you believe would enjoy it's

content.

73,

Swan User Group