Well men here are the Carrier Break Landing procedures.  These are the actual one’s use in T-45 Goshawk FCLP and CARQUALS.  It can be adapted to land-based approaches by offsetting the runway heading by 10 degrees when entering the BREAK.  It is vital that you practice this procedure, because this is how we will land in meetings and squad coop and so on (NOT LAND HOW YOUR COMFORTABLE).  If anything it puts all real challenge in landing, instead of the straight in boring CASE 3 recoveries, I mean come on too damn easy if you ask me. -Taz

Carrier Break Landing

- Marshall

- Command Marshall Altitude (Standard 8,000 feet ASL; DME

and proper RADIAL)

- Trail Formation (1 mile separation)

- Hook DOWN

- Airspeed 250 KTS.

- FUEL DUMP (as necessary to get below 42,000 lbs.)

- NAV lights ON (real life no anti-collision though)

- Formation Lights ON (Day-DIM; Night-BRT)

- Mark PUSH time on line-up card (make sure you have TIME

selected on your HIS

- Coming to Break

- Descend to 800 ASL in a LEFT turn, and then reverse in RIGHT turn

once 45 degrees offset of carrier BRC (Basic Recovery Course)

- Line-up one-mile starboard of carrier

- Set HUD to BARO (Barometric Altitude)

- Master Arm SAFE

- ENG page on left MPD and HSI on right MPD

- Airspeed 300 KTS. (400 KTS for Hornets)

- Parade Formation (Close as comfortable)

- Break (Cross bow or on seventeen second interval)

- FLOP: Smart roll to 60-90 degree AOB (Angle of Bank)

- CHOP: Retard throttle back to IDLE

- PULL: PULL 3-4Gs (depending on airspeed; EX. 300 KTS= 3G pull)

- Decrease PULL as speed comes down (should be at 2Gs around 200

KTS.)

- DROP: Lower landing gear as airspeed DROPS below 200 KTS.

- Speed Brakes EXTEND; Flaps FULL

- Level off on the downwind heading (shoot for 1 mile abeam

starboard of carrier)

- Downwind

- Add POWER to stop settle and attain ON-SPEED AOA

- Start descent to 600 feet while maintaining ON-SPEED AOA

- Complete landing checklist (Use CHK page for reference if needed)

- 180 Position

- 1 mile abeam LSO platform (1.0 –1.4 miles is okay)

- Call ABEAM (“Callsign, Abeam, Fuel State”)

- Roll RIGHT to 27-29 degrees AOB (add power AFTER roll

stabilizes)

- Start descent at -300 fpm

- This is an INSTRUMENT TURN; no need to look at the “boat”

- 90 Position

- 450 ASL at this position (be HIGH above LOW of this altitude)

- Increase the rate of descent to –600 to-700 fpm

- Adjust AOB to roll out on centerline

- Groove

- 200 feet ASL

- Reduce POWER to stop from going flat

- Maintain ON-SPEED AOA and –700 fpm descent

- MEATBALL, LINE-UP, AOA

- Keep Scan up until touchdown

- Touchdown

- Throttle FULL POWER

- Speed Brakes RETRACT

- Stick FULL AFT

- Bolter or Touch n’ Go

- Wingdip RIGHT to BRC

- 600 ASL then level off

- Once stabilized turn LEFT at 27-29 degrees AOB

to DOWNWIND heading

- Maintain ON-SPEED AOA through out turn