Week 6

June 12

Start Time: 9:30	End Time: 1:30

- Planned Experiments

- Filled tip boxes

- Isolated RNA

RNA Extraction with Rneasy Kit

- Spin samples in RNA later at 14000 rpm for 2 min to pellet in a microcentrifuge

- Discard supernatant completely

- Add 350μl Buffer RLT to homogenise and vortex 1 min to mix *RLT must have β-ME added! Stable for 1 month. # OR (but generally not) Add 600μl Buffer RLT if sample particularly hard to lyse or >20mg, vortex.

- Add 350μl of 70% ETOH, mix by pipetting

- Pipette sample into supplied column on 2ml tube

- Spin for 15s at ≥ 10,000

- Wash with 350μl RW1 buffer

- Spin 10,000 rpm for 15s, discard flowthrough

- Add 10μl DNase + 70μl buffer RDD (mixed) directly to membrane. Incubate for 15min at room temperature.

- Add 350μl RW1 Buffer to column

- Spin at 10,000 rpm for 15s; discard flowthrough

- Add 700μl RW1 buffer to column and spin 10,000rpm for 15s; discard tube and flowthrough.

- Transfer column to new 2ml collection tube (supplied)

- Pipet 500μl RPE buffer to column, spin 15s @ 10,000rpm

- Add another 500μl RPE buffer to column

- Spin 2min at max speed

- Rinse with 40μl RNase free water = elution step. Can use new 1.5 collection tube for this step.

- Spin at 10,000rpm for 1min

- Made up more DNase

June 13

Start Time: 10:15	End time: 1:30

- Quantified RNA in bioanalyser

| - Prepare Gel | - Filtered Gel (good for 1month)- place 400μl gel into filter column on a microcentrifuge tube (RNase free) - Spin at 40,000rpm for 10min - Remove filter and label tube with date - Make Gel-dye mix: Add 130μl gel to a new RNase free tube - Add 2μl blue RNA dye concentrate to gel - Label mix with date (good for 1 week) |
| --- | --- |
| - Clean Electrode | - Add 350μl RNase Zap to RNase Zap chip from kit - Put filled chip in machine, close gently (machine is on, not analysing) - Leave for 1min, then open machine and remove chip - Fill H2O chip with 350μl RNase free water - Put chip in machine, gently close - Leave for 30s, open machine and remove chip - Leave lid open for 30s to dry, then close. |
| - Prepare Samples | - Bring gel-dye mix and nanomarker to room temperature - Heat 1.5ml samples and ladder at 70oC for 2 min - Put samples on ice immediately |
| - Prime Chip | - Add 9μl Gel-dye mix to G well - Place new chip on priming station, close until a “click” is heard - Attach syringe (pulled up to 1ml) to loading station - Depress syringe and leave for 30s (will lock when depressed) - Open release (on syringe) and make sure syringe rises to >0.3ml/1s - *Use chip within 30min! |
| - Adding Samples | - Add 9μl gel-dye mix to top 2 G wells - Add 5μl nanomarker to all wells EXCEPT 3 G’s - Add 1μl/well samples to sample wells - Add 1μl ladder to ladder well - Add 1μl nanomarker to unused sample wells - Vortex plate at 2400rpm for 1min (Tape to secure!) |
| - Running Samples | - Place chip in machine - Click Agilent icon - Go to assay; select type of nucleic acid to analyse: Total RNA - Start analysis - Label wells by going to Data and Assay - Click Apply - When finished (much later) save as: date/suresh/My Documents |
| - Cleaning Up | - Remove chip immediately after analysis finishes - Clean electrodes with 350μl RNase free water in H2O chip - Leave in machine for 30s. - Remove chip and leave machine open for 30s - Close, exit software, clean area |
| *Record RNA concentrations for Reverse Transcription |  |

RNA Concentrations:

| Tube | 1(JR Her2) | 2(JR dl) | 3(JR –ve) | 4(JAS Her2) | 5 (JAS –ve) |
| --- | --- | --- | --- | --- | --- |
| Quantity: | 57 | 25 | 25 | 16 | 33 |

June 14

Start time: 9:10	End time: 6:40

- Reverse Transcription

| - Find quantity of RNA of each sample and standard (ie. Bioanalysis) |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Samples/std: Take 50ng RNA in 12μl or less into a new tube, diluting if necessary |  |  |  |  |
| - Add a volume of RNase free water to bring volume up to 12μl |  |  |  |  |
| - Make master mix (total volume = 8μl/sample or std + extra for pipeting errors) |  |  |  |  |
|  | 2.0μl | 10x Buffer RT | X 5reactions + 1extra = | 12μl |
|  | 2.0μl | dNTP mix | X 5reactions + 1extra = | 12μl |
|  | 2.0μl | Oligo-dT primer | X 5reactions + 1extra = | 12μl |
|  | 1.0μl | RNase Inhibitor | X 5reactions + 1extra = | 6μl |
|  | 1.0μl | Sensiscript Reverse transcriptase | X 5reactions + 1extra = | 6μl |
|  | Vortex samples briefly and spin at 10,000 rpm for 2s to collect liquid |  |  |  |
| - Add 8μl mastermix to each sample/standard well (20μl reaction volume) |  |  |  |  |
| - Incubate for 60min at 37oC (hotplate with water in wells) |  |  |  |  |
| - Incubate for 5min at 93oC to inactivate RT (make short cDNA fragments) |  |  |  |  |
| - Cool on ice, then place in fridge or run Taqman qRT-PCR |  |  |  |  |

RNA concentrations and Mix concentrations:

| Tube | 1(JR Her2) | 2(JR dl) | 3(JR –ve) | 4(JAS Her2) | 5 (JAS –ve) |
| --- | --- | --- | --- | --- | --- |
| Master Mix | 8μl | 8μl | 8μl | 8μl | 8μl |
| RNA stock | 0.9μl | 2.0μl | 2.0μl | 3.1μl | 1.5μl |
| H2O | 11.1μl | 10.0μl | 10.0μl | 8.9μl | 10.5μl |
| Total Rxn volume: | 20.0μl | 20.0μl | 20.0μl | 20.0μl | 20.0μl |

- SKBr3 cells are A2 negative- so that explains a lot of the confusing data, and why there was not a lot of lysis in the Chromium Release assay specifically related to the Her 2 treatment.

Plate Map:

| S1 | S1 | S2 | S2 | S3 | S3 | S4 | S4 | S5 | S5 | S6 | S6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jr her 1/5 | Jr her 1/5 | JR dl 1/5 | JR dl 1/5 | JR – 1/5 | JR – 1/5 | JAS her 1/5 | JAS her 1/5 | Jas – 1/5 | Jas – 1/5 | NTC | NTC |
| GAPDH^ |  |  |  |  |  |  |  |  |  |  |  |
|  | INF V |  |  |  |  |  |  |  |  |  |  |
| S1 | S1 | S2 | S2 | S3 | S3 | S4 | S4 | S5 | S5 | S6 | S6 |
| Jr her 1/5 | Jr her 1/5 | JR dl 1/5 | JR dl 1/5 | JR – 1/5 | JR – 1/5 | JAS her 1/5 | JAS her 1/5 | Jas – 1/5 | Jas – 1/5 | NTC | NTC |

Taqman qRT-PCR (real time) Proceedure

| Standards | - Take standard sample from Reverse Transcription - Dilute the 20μl sample by 2 = 40μl - Took 20μl for S1 - Set up serial dilutions with H2O; all standards 80μl except S1 (20μl) and S6 (100μl) S2 = 20μl sample (original) + 80μl H2O (5x dilution) S3 = 20μl from S2 + 80μl H2O (50x dilution) S4 = 20μl from S3 + 80μl H2O (250x dilution) S5 = 20μl from S4 + 80μl H2O (1250x dilution) S6 = 20μl from S5 + 80μl H2O (6250x dilution) *standards can be frozen at –20oC until ready to continue. |
| --- | --- |
| Plate Prep (96 well clear optical plate) | - Make master mix: Manufactured mix. Makes ~45 wells - 12.5μl/primer-probe type +1.25μl primer/probe + 281.25μl RNase free H2O/well - Put 20μl master mix in each standard and sample well - Put 5μl of sample or standard or RNase free water in each well according to a map made previously** key to knowing where samples are and what they are! Esp. in analysis! |
| Machine | - Turn on machine. - Select SS2.1* New software! - File- new - Instrument- open/close - Start new assay- 96 well plate, total quantification, blank template - Save As: date…suresh BEFORE starting - Startrun for ~1hour - Analyse (Green tirangle), click results file tab (on right) - Export as A:/…/suresh to take away |

| PCR MM | 312.5μl | 312.5μl | 12.5x25wells |
| --- | --- | --- | --- |
| 20x probe and primer | 31.25μl | 31.25μl | 1.25x25 wells |
| RNase free H20 | 156.25μl | 156.25μl | 6.25x25 wells |
|  | GAP dh | INF γ |  |

- Ran as 9600 emulation.