telnet commands

Command Option Variable Function or Action Taken

COMMAND/OPTION/VARIABLE /FUNCTION OR ACTION

Help                                                                              Returns the list of available commands.

Ping

IP Address                      Indicates whether a device is active.

Reset                                                                             Terminates the Telnet session and resets the Gateway.

Task               ID #       Resets an individual task. The Task ID # is displayed

through the Show Task command.

Logout                                                                          Terminates the Telnet session with the Gateway.

Logfile

Ena                                      Enables the logging mechanism on the Gateway and

starts logging transactions

Dis                                         Stops logging and disables the logging mechanism on

the Gateway.

Show Boot

Version                                 Returns the boot code version of the Gateway.

TFTP            File         Returns the name of the application file the  Gateway

will request from the  Server. If  not defined (blank

field), the Gateway will request:

/jbmgatew/jbmgw50/jbmgw50.bin

IP                Returns the IP address of the TFTP Server which

holds the application file for  the  GateWay Server

IP                Returns the IP address of the JBM unit used when

retrieving an application from the TFTP Server.

Mask                                      Returns the IP subnet mask used by the Gateway.

Gate             IP                  Returns the Gateways default router IP address.

App              Load             Returns the value of the "Load Application" flag. If

Enabled is returned,  the Gateway will reload an

application after reset. If Disabled is returned,  the

Gateway will NOT reload an application after set.

Show

Task                                          Returns a list of current tasks and their ID numbers.

Stat             PortType     Returns the current status of the requested port. The

, Port types are: RS232 X25, Frame, Appl, Multi, TCP,

UDP.

Memory      Hex Addr xxxx   Allows memory dump of any area in the

Gateway. The default length is 16 bytes. (xxxx -

Optional Length in bytes)

Gate              IP                   Returns the Gateways default router IP address.

App               Version         Returns the current Gateway Application version

number.

TFTP           File               Returns the name of the configuration file the

Gateway will request from the TFTP Server. The

default file is: /jbmgatew/jbmgw50/jbmgw50.cfg

IP                    Returns the IP address of the TFTP Server which

holds the configuration file for  the Gateway.

Server           IP                   Returns the IP address of the JBM unit used when

retrieving an configuration from the TFTP  Server.

Mask               Returns the IP subnet mask used by the Gateway.

Conf             Load               Returns the value of the "Load Configuration" flag. If

Enabled is returned, the  Gateway will attempt to

reload an configuration after reset. If Disabled is

returned,  the Gateway will NOT reload a

configuration after reset.

Load Dump     Returns a memory dump of the configuration load.

ChangeBoot

TFTP           File                  Configures the name of the application file that the

Gateway will request from the TFTP Server. If not

specified, the Gateway will

request:/jbmgatew/jbmgw50/jbmgw50.bin. For

example: >CHANGE BOOT TFTP

FILE /jbmgatew/str/jbmgw50.bin

IP                  Configures the IP address of the TFTP Server used

by the Gateway to request an application file. For

example: >CHANGE BOOT TFTP IP 155.10.10.10

Server          IP                    Configures the IP address used by the Gateway when

retrieving an application from the TFTP  Server. For

example: >CHANGE BOOT SERVER IP 182.10.20.105

Mask              Configures the IP subnet mask used by the Gateway.

For example: >CHANGE BOOT SERVER MASK

255.255.0.0

App               Load          Configures the value of the "Load  Application" flag.

If Enabled is defined, the Gateway will reload an

application after reset. If Disabled is defined, the

Gateway  will not  reload an application after reset.

For example: >CHANGE BOOT APP LOAD ENABLE

Gate              IP                   Configures the IP address of the default router used

by the Gateway. For example: > CHANGE BOOT

GATE IP 192.10.10.10

Change

TFTP            File               Configures the name of the configuration file that the

Gateway will request from the  TFTP Server. If  not

specified the Gateway will request :

/jbmgatew/jbmgw50/jbmgw50.cfg For example: >

CHANGE TFTP FILE /jbmgatew/str/jbmgw .cfg

IP               Configures the IP address of the TFTP Server used by

the Gateway to request an  configuration file. For

example: >CHANGE TFTP IP 155.10.10.10

Server          IP                  Configures the IP address of the JBM unit used when

retrieving a configuration from  the TFTP Server. For

example: >CHANGE SERVER IP 182.10.20.105

Mask                                       Configures the IP subnet mask used by the Gateway.

For example: >CHANGE  SERVER MASK 255.255.0.0

Gate             IP             Configures the IP address of the default router used  by

the Gateway. For example: >CHANGE GATE IP

192.10.10.10

Conf            Load            Configures the value of the "Load Configuration"  flag.

If Enabled is defined, the  Gateway will reload  a

configuration after reset. If Disabled is defined,  the

Gateway will not reload an configuration after  reset.

For example: >CHANGE CONF LOAD ENABLE

Set

Port #             Up              Starts a Port.

Down        Stops a Port.

Subaddress #       Up             Starts a Subaddress.

Down         Stops a Subaddress