# Study of a Proposed Tsunami Detection Device

Authored by Chun hua Zheng

With Emily Warmann

Professor  Douglas Hart

Tuesday 1-5pm

October 17, 2000

Massachusetts Institute of Technology

Department of Mechanical Engineering

Contents:

1.	 Abstract ............................................................................................................

2. 	Introduction   ............................................................................................................

3. 	Experimental Study  ................................................................................

4. 	Theory and Analysis   ..................................................................................

5. 	Results and Discussion   ..............................................................................................

6. 	Conclusions and Recommendations   ...........................................................................

- References   ...........................................................................

# 1. Abstract

### The devastating and oftentimes unpredictable effects of tsunamis have prompted the need for tsunami research and data acquisition.  Current battery-operated wave information recorders lack tsunami detection mechanisms that will trigger the recorder on and off with the passing of a tsunami wave.  The proposed tsunami detection device utilizes fluid low-pass filters to detect tsunamis by their characteristic range of frequencies.  This project’s goal is to determine the feasibility of installing such a detection device in the open ocean.  In determining the feasibility of the system, a fluid circuit model of the device was created and its validity tested.  Using the satisfactory model to size the detection device, it was concluded that the proposed detection device would not be feasible due to the extreme size of parts required.

2. Introduction

Tsunamis are seismic waves that propagate over water; although rather harmless in the open ocean, upon reaching land, tsunamis can cause massive flooding and leave devastating effects on shoreline areas.  At present there exist efforts to study the environmental impact of tsunamis on remote shoreline regions.  Battery operated recorders record the amplitude and period of these deep sea seismic waves.  As it is desirable to prolong the operating life of the batteries, the recorders should be turned on only at the onset of a tsunami.  A tsunami detection device operating a float switch is needed to trigger the recorder on and off at the appropriate moments of a tsunami passing.

A tsunami detection device needs to accurately distinguish between the different deep sea disturbances such as ripples, chops, sea swells, surf beats and tides.  Since tsunami waves have a distinct period (on the order of ten of minutes) it is theoretically possible to build a sensor that will respond only to frequencies within this range.  The use of two fluid low pass filters, one to detect tides (disturbances with periods on the order of hours) and another to detect both tides and tsunamis (periods greater than several minutes) will allow for the identification of a passing tsunami.  Acting as a band pass filter, it will detect and respond only to those frequencies characteristic of tsunamis.

In detecting the efficacy of the tsunami detection device, a fluid circuit model representation of the device was created.  The validity of the model was assessed and upon getting satisfactory results, the model was used to size a tsunami detection device for use in the open sea.

The following report describes, first, the apparatus and procedures used in the conduction of the experiments and the analysis of data.  In the Theoretical Analysis section, the underlying theory behind the fluid circuit model and the governing equations are explained.  In the Results and Discussion section, theoretical and experimental results are compared and analyzed.  The last section, Conclusion and Recommendations highlights the important findings of the project and makes recommendations for future work.

# 3. Apparatus and Procedure

The tsunami detection device is comprised of two main sections (Figure 3a), the wave generator on the right and the wave detector on the left.  The wave generator consists of a weighted cylinder that is driven up and down by a variable speed motor, generating pressure waves that propagate through the system.  The pressure at the base of the wave generator cylinder is measured by a pressure sensor and sent to the computer.

Figure 3a :  Tsunami Detection Apparatus

The wave detector side of the device consists of three separate sets of horizontal and vertical tubes that are connected to the wave generator. At the base of each vertical tube is a pressure sensor whose voltage outputs are sent to the computer for analysis.  The different dimensions of the sets are shown in Figure 3b.

Figure 3b :  Tube Geometry

Input Pressure

Pwave(t)

Output tube pressure

Patm

Inner diameter, d

Inner diameter, D

L

h

| Tubes | Horizontal Inner Diameter, D | Vertical Inner Diameter, d | Horizontal Length |
| --- | --- | --- | --- |
| 0 | 0.0254 m | 0.0254 m | 1.75 m |
| 1 | 0.0127 m | 0.0254 m | 1.75 m |
| 2 | 0.0095 m | 0.0508 m | 1.75 m |

Data was collected from nine runs of different generator frequencies by the “Waveform” software program. One separate run was used to calibrate the voltage readings with water level measurements that were taken by hand.  The time versus voltage plots of the detector tubes and generator tube were filtered using a Matlab scrip that plotted the average of every five data points.  Results from the calibration were used to correlate voltage data to pressures with the proper offsets.

Bode plots relating frequency to pressure outputs were made using the experiment results and compared to bode plots from the theoretical model.

# 4. Theoretical Analysis

One can often model fluid systems by the use of idealized electrical and mechanical elements with pressure being analogous to voltage or force and with flow rate being analogous to current or velocity.  Based upon the proposed tsunami detection device, a theoretical model or the system using the fluid circuit analogy was devised (Figure 4a).

The model sought to account for all fluid elements in the detection device.  The wave generator is modeled as a pressure source (Pin).  The horizontal tubes are modeled as fluid inductance (I) and resistance (R) connected in series.  The series model accounts for the fact that the same flow rate, Q, travels through the fluid inductor and resistor.  The water in the vertical tubes is modeled as a fluid capacitance (C).

Figure 4a : Fluid Circuit Model

Patm

Capacitance

Inductance

Resistance

P in

P out

The formulae for the elementary fluid elements are:

(Eqn 4.1)                               (Eqn 4.2)                           (Eqn 4.3)

I=ρlAh                             C=Avρg                            R=128μlπD4

density

l = length of tube

h = cross sectional area of horizontal tube

Av = cross sectional area of vertical tube

g = acceleration due to gravity

= absolute viscosity of the fluid  [ n sec/m2 or lb sec/in2]

D = inside diameter of capillary

The value for fluid resistance is based on the model of steady state incompressible flow through a long capillary tube.  The steady state assumption neglects entrance effects and the large length to diameter ratio of the horizontal tubes (69 for tube 0, 138 for tube 1, and 184 for tube 2) were used to justify the use of the capillary tube assumption.

The laminar flow model was based on calculations of the Reynolds number.  For Reynolds’ values less than 2300, the laminar flow model is valid.

(Eqn 4.4)

Re=ρvDμ

Where :

v = velocity

Elemental equations for the ideal elements are used in conjunction with continuity and compatibility equations to arrive at a pair of state equations describing system behavior.  In standard form, the state equations are:

(Eqn 4.5)

Pc

Where:

Pc = pressure across capacitor

QI = flow rate through inductor

Ps = pressure source

The resulting transfer function relating output pressure to input pressure is:

(Eqn 4.6)

H(s)=PoutPin=1ICs2+RCs+1

Where

Pout = pressure across capacitor

Pin = pressure source

The magnitude:

(Eqn 4.7)

H(jw)=11−ICw2−RCw

The phase lag:

(Eqn 4.8)

Φ(jw)=tan−1−RCw1−ICw2

Damping ratio:

(Eqn 4.9)

ζ=RCIC2

Resonant frequency:

(Eqn 4.10)

wr=1−R2C3IIC

Magnitude of resonant peak:

(Eqn 4.11)

Mr=12ζ1−ζ2

The transfer functions for all three detection tubes were plotted and experimental results were compared with the theoretical results.

5. Results and Discussion

The frequency responses of the tsunami detection system as predicted by the fluid circuit model is as seen in Figure 5a.

Figure 5a:  Bode Plot of Theoretical Model

### Distinct resonance peaks can be seen in tubes zero and one but not in tube 2.  The flow in tube 2 is over-damped (z >1), while the under-damped tubes zero and one peak at their respective resonance frequencies.  Calculated values for important bode plot characteristics of the three detection tubes can be seen in Figure 5b.

Figure 5b: Damping Ratio, Resonance Peak and Resonance Frequency

| Tube | Damping Ratio,  | Resonance Peak | Resonance Frequency |
| --- | --- | --- | --- |
| 0 | 0.00184 | 271.653 | 2.365 |
| 1 | 0.05890 | 8.50389 | 1.1747 |
| 2 | 1.98559 | ### None | .44 |

The damping ratio, resonance frequency and peak values were calculated using the following resistance, inductance and capacitance values of the tubes:

Figure 5c: Resistance, Inductance and Capacitance Values

| ### Tube | Resistance | Inductance | Capacitance |
| --- | --- | --- | --- |
| 0 | 168126 | 3448802 | 5.18019E-08 |
| 1 | 2690014 | 13795207 | 5.18019E-08 |
| 2 | 8501771 | 24524812 | 2.07208E-07 |

Experimental data values were plotted onto the theoretical model bode plots in Figure 5d.  The resonance peak of experimental data for tube 0 can be seen clearly and the cutoff frequency corresponds within .5 rad/sec.  However, the peak magnitude at resonance is much smaller in the experimental data. This is due to the fact that the theoretical damping value was lower than that experienced by the actual device.

Figure 5d: Theoretical and Experimental Bode Plots

Tube 0

Tube 1

Tube 2

Along the same line of reasoning, resonance peaks are not noticeable in the responses of either tube 1 or tube 2 because actual damping is higher than what was predicted by the model.  The damping ratio increases with an increase in either of three elementary fluid elements, but is affected most by values for capacitance and resistance.

Most likely, deviations in the theoretical model arose from use of the steady state laminar flow assumption in calculating resistances.  The steady-state assumption may be inadequate as the flow experiences entrance effects, especially at the higher frequencies.  The laminar model breaks down when the Reynolds value exceeds 2300.  The Reynolds’ number calculation for tube 2 exceeds 2300 at high flow rates, meaning that the flow cannot be modeled as laminar. (Figure 5e)

Figure 5e: Reynolds Values

|  | Tube 0 | Tube 1 | Tube 2 |
| --- | --- | --- | --- |
| Reynolds No. | 2584 * v | 2584 * v | 5169 * v |

A better model would take into account the friction factor of the tubing and would use a different equation for resistance to account for turbulent flow.  A different fluid circuit may also be used to model the device.  One such method would place the resistance and inductance in parallel instead of in series. (Figure 5f) Another method would be to plug up the tops of the detector tubes during experimentation and introduce an “air spring” between the capacitor and atmosphere.

Figure 5f: Fluid Circuit with Inductance and Resistance in Parallel

Patm

Capacitance

Inductance

Resistance

P in

P out

Figure 5g: Fluid Circuit with an Air Spring

Patm

Capacitance

Inductance

Resistance

P in

P out

In sizing the device for use in real life tsunami detection in the open sea, the equation for natural frequency is used.

(Eqn. 5.1)

wn=1IC=DhDvgl

Knowing that the frequency of a tsunami wave is on the order of .001, the equation can be rearranged as:

(Eqn. 5.2)

Dh=.0001Dvl

Selecting values for the horizontal diameter (Dh) and vertical diameter (Dv) of the tubes, the required length of the horizontal tubing is calculated.

Figure 5f:  Tube Sizing

| Diameterh (m) | Diameterv (m) | length (m) |
| --- | --- | --- |
| 1 | 1 | 100000000 |
| 1 | 10 | 1000000 |
| 1 | 100 | 10000 |
| 0.1 | 100 | 100 |
| 0.01 | 100 | 1 |
| 0.001 | 5 | 4 |

From Figure 5f, it can be seen that the implementation of this setup in a tsunami detection device is unfeasible due to the extreme sizes of the tubes.

6. Conclusion and Recommendations

The theoretical and experimental results from this study point to the conclusion that a the proposed tsunami detection device would be physically unfeasible to build and implement due to the extreme size of tubing required.  The fluid circuit model used to analyze the detection system is not without error as it did not account for excess damping in the tubes.  A better model would use a correction factor to account for the friction of the tubing surfaces as well as entrance effects.

In terms of the actual tsunami detection device, it would be desirable for the gage system to be enclosed, using distilled water so as to decrease corrosion effects.  It is important also to account for the range of error of tsunami reporting, for example, accounting for overlapping disturbances such as tsunami waves riding on tidal waves.  Although not tested in this study, the introduction of pulses to the system would change the character of detection.

# 7. References

Adams, William Mansfield.  Tsunamis in the Pacific Ocean.  East-West Center Press, Honolulu, 1969.

Ogata, Katsuhiko.  Modern Control Engineering.  Prentice Hall, NJ, 1997.

Rowell, Derek and David N. Wormley.  System Dynamics.  Prentice Hall, NJ, 1997.