# chapter one

## INTRODUCTION AND BACKGROUND

### Introduction – the issues before the Committee

- This inquiry was initiated to investigate reports that chemical fumes, particularly containing Tricresyl phosphate (TCP), have contaminated and continue to contaminate, cockpits and passenger cabins of the BAe 146 model aircraft operating in Australia, affecting the capacity of pilots and cabin crew to safely operate the aircraft.

- The Committee particularly investigated whether TCP, which is a known toxin if inhaled, and other chemicals toxic to humans, have entered, and continue to enter BAe 146 aircraft cabin air. The impact on the health of flight crew and passengers as a result of possible exposure to fumes in the cabin air, was also a principal concern of the Committee’s inquiry.

### Current Applicable Australian Regulatory Requirements – Flying and Airworthiness

#### Civil Aviation Regulations (CARs)

- The Committee initially notes that several current regulations (Civil Aviation Regulations – CARs) made pursuant to the Civil Aviation Act 1988 require pilots to be in a suitable state of health for flying an aircraft and therefore acknowledges the regulatory link between crew health and air safety. The following Civil Aviation Regulations on crew health are considered relevant:

- CAR 2 (major defect)

…. as in relation to an aircraft, means a defect of such a kind that it may effect the safety of the aircraft or cause the aircraft to become a danger to person or property.

- CAR 48.0 (Flight time limitations).

1.4: Notwithstanding anything contained in these orders, a flight crew member shall not fly, and an operator shall not require that person to fly if either the flight crew members is suffering from, or considering the circumstances of the particular flight to be undertaken, is likely to suffer from fatigue or illness which may affect judgement or performance to the extent that safety may be impaired;

- Civil Aviation Advisory Publication (CAAP) 51-1 (O) advises

(c) smoke, toxic or noxious fumes inside the aircraft is considered a major defect.

- With regard to any relationship between cabin air quality on the BAe 146 and air safety, the Committee is also aware of the following Federal Aviation Regulations which are incorporated into Australian CAR’s governing cabin air quality.

#### Federal Aviation Regulations (FARs)

- FAR 23.831 (Ventilation and heating)

(a) Under normal operating conditions and in the event of any probable failure conditions of any system which would adversely affect the ventilation air, the ventilation system must be designed to provide a sufficient amount of uncontaminated air to enable the crew members to perform their duties without undue discomfort or fatigue and to provide reasonable passenger comfort.

(b) Crew and passenger compartment air must be free from harmful or hazardous concentrations of gases or vapours.

(c) There must be provisions made to ensure that the conditions prescribed in paragraph (b) of this section are met after reasonably probable failures or malfunctioning of the ventilating, heating, pressurisation or other systems and equipment.

- The Committee also observes that the link between pilot health and air safety is explicitly acknowledged by the Civil Aviation Safety Authority (CASA), in a newsletter dated November/December 1999 and titled ‘Fit to Fly’ which counsels pilots on the impact that minor health problems can have on their capacity to fly.

#### Health and Safety Issues

- In addition to crew health, and aircraft airworthiness, the issue of fume contaminants should also be considered a safety issue with regard to the ability of cabin crew to properly supervise the evacuation of an aircraft and the ability of passengers to take part in an evacuation.

- Written submissions to this inquiry considered by the Committee, both public and confidential, provide evidence of more than 700 recorded incidents in the last 15 years where fumes have been reported to have entered the cabin and contaminating the cabin air on BAe 146 aircraft operating in Australian airspace. This evidence was provided by aircraft operators and by various unions and associations representing flight crew.

- While the total number of reported incidents varies, a summary of fume reports provided by Ansett Australia and the Flight Attendants Association of Australia shows the figure of 700 incidents to be a conservative estimate of fume occurrences since the BAe began operating in Australia.

- Evidence was also provided of a successful application for employees compensation in the Compensation Court of New South Wales for the aggravation of a pre-existing illness caused to a flight attendant, Ms Alysia Chew, due to exposure to fumes during an incident on a BAe 146.

- Several other successful applications for workers’ compensation for illness attributed to fumes on the BAe 146 have also been drawn to the Committee’s attention. Currently two civil actions are being pursued for common law damages for illness allegedly resulting from exposure to fumes on a BAe 146.

- The Committee was informed that when this issue became a matter of general public concern, some 140 Ansett flight crew held medical certificates exempting them from flying on the BAe 146. Some of these crew requested exemption as a precautionary measure and were not suffering the effects of exposure to fumes on the aircraft. At the time of this report, Ansett has advised the Committee that nearly 80 per cent of previously exempt flight attendants have returned to flying on the BAe 146 ‘without any significant issues being raised or ill effects reported’.

- The Committee received approximately 20 individual submissions describing symptoms experienced by crew members and attributed to oil fumes leaking into the aircraft cabin. The Committee notes that of 31 public submissions made to the inquiry, a significant number argued that contamination of cabin air on BAe 146 aircraft was a continuing problem warranting further action and investigation.

- These submissions described in detail symptoms crew members experienced as a result of exposure to cabin air in the aircraft. These submissions also set out how these exposures had affected their health and the processes they followed in dealing with their employers concerning the health problems they experienced. A number of other submissions argued there was not a continuing problem warranting further action and investigation. The remaining submissions did not advocate the implementation of any further action.

- The Committee also received evidence from the operating airlines describing in detail the steps taken to address the problem, including compulsory reporting systems and extensive modifications to the BAe 146 air circulation system.

- As noted, the Committee is particularly concerned to identify whether the BAe 146 presents a link between effects on occupational health of flight crew and the safe operation of the aircraft. This has emerged as a difficult and controversial issue.

- The Committee received considerable evidence criticising aspects of the regulatory regime for the aircraft and focusing on issues that should be taken up by regulators, such as:

- oil leaks and exposure to oil fumes;

- responses to crew complaints;

- testing procedures for cabin air; and

- modifications measures necessary to remedy fume contamination.

- The Flight Attendants Association of Australia (FAAA) told the inquiry in evidence that:

There has been a significant exercise in semantic tap-dancing by the regulatory authority, CASA, over whether this is a health issue or a safety issue as though there is some need for distinction between the two. The flight attendants on board the aircraft are on board for this reason: there is a regulatory requirement that, to ensure the evacuation of all passengers in under 90 seconds through half the available exits, cabin crew are required to be there. Flight attendants are there for safety. If flight attendants are having to be carted off aircraft in wheelchairs and placed onto oxygen during descent, the health of these flight attendants has been affected to the extent where the safety of the flight and of those passengers has been compromised. Consequently, the issues of health and safety are not separate, but are inextricably intertwined.

- However, a letter to the Committee from the Australian Transport Safety Bureau (ATSB) responding in part to the issue of fumes in relation to neurological impairment, set out the view of the Bureau on this issue:

This is in the field of occupational medicine and should not be confused with Aviation Safety unless there is immediate incapacitation of flight crew. If the latter occurred, then the ATSB, and for that matter CASA, would become involved.

Long term incapacitation as a result of exposure in the workplace is covered by all State and Commonwealth legislation as an Occupational Health and Safety (OH&S) issue and is appropriately addressed as such.

- Opinion is divided on this issue within the ATSB. Mr Brett Leyshon of the ATSB supported the view of the flight attendants in relation to safety when he told the inquiry in evidence:

The crew are not simply there to direct passengers to seats and to serve meals. They serve an important safety function throughout the flight, even a normal flight. Removing those removes a layer of safety to the passengers in the cabin.

- Potential links between air safety and health effects resulting from exposure to fumes on the BAe 146 are discussed in Chapters 4 and 5 of this report.

- The Committee has had regard to those confidential submissions made during the inquiry by individuals currently pursuing claims for compensation through appropriate means and in the appropriate tribunals. The submissions allege exposure to fumes on the BAe 146 to be the source of the illness and incapacity to continue work and of damage to health, careers or both.

See CASA website, www.casa.gov.au, Legislation and Regulations, Civil Aviation Advisory Publications, 51-1.

This regulation is an United States of America Federal Aviation Regulation (FAR) applicable as an Australian CAR under international regulatory harmonisation arrangements and, accordingly, apply to Australian registered aircraft.

Flight Safety Australia, Nov-Dec 1999, pp 33-34

Flight attendant information kit, Ansett Australia. (Estimate of on average of 1 in every 131 flights is affected by fume occurrences); see also submission 24, Flight Attendants’ Association of Australia.

Alysia Chew v Eastwest Airlines and Ansett Australia Ltd, Compensation Court of New South Wales, (Matter no 19652/1995)

Letter dated 22 September 2000, Ansett Australia to the Committee, supplementary material.

Letter dated 10 October 2000, Ansett Australia to the Committee, supplementary material.

Ms Judy Cullinane submitted a detailed submission and additional documents setting out details of her illness and her experiences in dealing with Ansett, Submission 17, Ms Judy Cullinane; see also Submission 10, Deborah Carter; and Confidential submissions C20 and C19.

FAAA, Evidence, 2 February 2000, p 155

Correspondence from ATSB to the Committee dated 10 April 2000, p 1

ATSB, Evidence, 13 March 2000, p 196