# chapter three

## SYMPTOMS OF ILLNESS AND POSSIBLE SOURCES

- The central issue in this inquiry is whether health effects result from exposure to oil fumes in aircraft cabin air. Current medical science and technology available for measuring and analysing the clinical effects of exposure to minute combinations of chemicals are both relatively new. The Committee received submissions from several medical and occupational health professionals supporting claims by flight crew that exposure to fumes on BAe 146 aircraft resulted in deterioration of their health.

- The Committee is also aware that the Industrial Court of New South Wales has acknowledged that exposure to fumes on a BAe 146 exacerbated a pre-existing illness suffered by former Ansett flight attendant Alysia Chew.

- One medical professional, Dr Robert Loblay, gave evidence to the Committee arguing that there are no health effects as a result of exposure to fumes. Unfortunately, his evidence consisted largely of attacking the personal and professional integrity and status of other witnesses. Dr Loblay did not supply a written submission to the inquiry.

- The majority of the professional witnesses to the inquiry highlighted an absence of clinical testing of flight crew and passengers immediately after their exposure to fumes. In the absence of equipment sensitive enough to detect all potential chemical components present in human tissue following a fume exposure incident, it appears difficult to measure the health consequences of fume exposure.

### Examples of symptoms

- There was a commonality within the symptoms reported by affected flight crew exposed to fumes on aircraft which can be summarised as follows:

- dizziness;

- nausea;

- vomiting;

- headaches;

- head pressure;

- numbness;

- tingling;

- irritations to eyes, nose and throat;

- breathing difficulties;

- vision difficulties;

- fatigue;

- weakness;

- cognitive dysfunction;

- concentration difficulties;

- disorientation confusion;

- chemical sensitivities; and

- neurobehavioural difficulties.

- Associate Professor Chris Winder from the University of New South Wales set out in his submission both short term and long term symptoms exhibited by affected people he had interviewed who had flown on the BAe 146.

- Dr Winder advised that symptoms from single or short term exposures, include:

- neurotoxic symptoms: blurred or tunnel vision, nystagmus, disorientation, shaking and tremors, loss of balance and vertigo, seizures, loss of consciousness, parathesias;

- psychotoxic symptoms: memory impairment, headache, light-headedness, dizziness, confusion and feeling intoxicated;

- gastro-intestinal symptoms: nausea, vomiting;

- respiratory symptoms. cough, breathing difficulties including shortness of breath, tightness in chest, respiratory failure requiring oxygen;

- cardiovascular symptoms: increased heart rate and palpitations; and

- irritation of eyes, nose and upper airways.

- Dr Winder also said symptoms from long term low-level exposure or residual symptoms from exposure events, include:

- neurotoxic symptoms: numbness (fingers, lips, limbs), parathesias;

- psychotoxic symptoms: memory impairment, forgetfulness, lack of co-ordination, severe headaches, dizziness, sleep disorders;

- gastro-intestinal symptoms: salivation, nausea, vomiting, diarrhoea;

- respiratory symptoms: breathing difficulties (shortness of breath), tightness in chest, respiratory failure, susceptibility to upper respiratory tract infections;

- cardiovascular symptoms: chest pain, increased heart rate and palpitations;

- skin symptoms: skin itching and rashes, skin blisters (on uncovered body parts), hair loss;

- irritation of eyes, nose and upper airways;

- sensitivity: signs of immunosupression, chemical sensitivity leading to acquired or multiple chemical sensitivity; and

- general: weakness and fatigue (leading to chronic fatigue), exhaustion, hot flashes, joint pain, muscle weakness and pain.

- Evidence to the Committee presented from pilots, cabin crew and their medical advisors that these generalised symptoms are common to those who have developed symptoms after being exposed to fumes while flying in the BAe 146.

- However, Dr Robert Loblay of the University of Sydney told the inquiry:

The Australian Institute of Health and Welfare has done population surveys for many years now showing that these non-specific symptoms are present at any one time in 10 per cent of the population. If you survey the population two years later, it is still 10 per cent but they are different people.

- Dr Loblay went on to state:

Tunnel vision is not a symptom of neurotoxicity. It is not an uncommon symptom in people with acute anxiety and hyperventilation.

I am not denying that there have not been problems with air quality and with fumes and so on in the BAe 146. That is absolutely clear from the evidence the expert panel was presented with. But when a belief system spreads in a population that a particular work or other environment is dangerous, then people come to attribute these common symptoms to their environment. It is a common phenomenon in this area …. People’s beliefs often lead them to mistakenly attribute common symptoms or anxiety symptoms to toxic exposure when they are in an environment where they believe there are toxic chemicals. This is a phenomenon that has been demonstrated in healthy individuals in experimental circumstances as well. So I do not necessarily believe that most of those symptoms that are described are symptoms of chemical neurotoxicity in the way that it has been claimed …

- The Committee notes that reported incidents of health problems among flight crew arising from claimed fume exposure on BAe 146 aircraft indicate a higher rate of problems than the 10 per cent in the general population quoted by Dr Loblay.

- Dr Loblay advised:

I accept that when there are leaks and fumes come into the cabin people do experience irritant symptoms - irritation of the eye, nose and throat. Many people experience headache and nausea when they are exposed to unpleasant fumes and smells. That is a pretty common phenomenon. I have no difficulty with that. What I have difficulty with is the claim that flying in these aeroplanes regularly and being exposed to the usual cabin air, leaving aside those single episodes where there are significant leaks, is in any way responsible for long-term adverse health effects or even short-term adverse effects.

### Possible enhanced effect on symptoms from flying

- It has been put to the Committee that symptoms of toxicity from fumes are not only caused by chemical exposure but may be “… exacerbated by the hypoxia of cabin pressure, other chemical exposures (such as carbon monoxide), temperature, humidity, workload or pre-existing health conditions.” This phenomenon is described by some in the medical profession as “aerotoxic syndrome” and is now a specialist area for medical and occupational health research.

### Possible causes and sources of illnesses in the BAe 146

- Associate Professor Winder listed the following occurrences as possible sources of fumes and invisible smoke in a BAe 146 cabin:

- oil leaks to the air conditioning system;

- smoke from combustion/pyrolysis events;

- contamination following pack burn outs;

- exposures during times when contaminated engines/APU are being used; and

- residual contamination.

- As noted in the description in Chapter 1, the BAe 146 contains an auxiliary power unit (APU) which primarily supplies compressed air for ground operation of the air conditioning system and is also used during take off and landing. According to Dr Winder; “Both the engines and APU have been implicated as sources of the fumes/mists that have entered the flight deck and cabin, although the engines are considered the main source of the problem.”

#### Engine oil and oil seals

- The main engine oil used in the BAe 146 as well as in other jet aircraft in Australia is Mobil Jet Oil II, a synthetic phosphate ester product manufactured by Mobil USA and marketed in Australia by Mobil Australia. The Committee understands that in various formulations, Mobil Jet Oil II has been in use in the world aviation industry for more than 38 years. During 1998 1.4 billion passengers were carried on jets using this oil.

- Mobil Oil told the inquiry:

Mobil has roughly 51 per cent of a world market for turbine oils. Jet oil II is certainly the main grade and would account for over 90 per cent of that. … about 45 per cent of turbine engines worldwide would run on jet oil II.

- Several submissions from crew/and medical professionals maintained that the cause of fumes on the aircraft was burnt oil lubricants leaking from engines into the BAe 146’s cabin air system:

Excessive oil leakage from oil seals allows smoke and lubricating oil components to enter the cabin. Oil seals are used to ensure that engine oil does not mix with the air system. Oil is passing through the engine seals into the compressor bleed air system and therefore contaminating air used for the environmental control system (ECS). The engine bearing conditions are in some cases further allowing oil to pass the engine seal system and therefore enabling oil to enter the cabin bleed air system, being the source of cabin air-conditioning/heating and pressurisation involves the following problems:

-residual oil leaks from engines/ APU into cabin air conditioning system - complete bearing / seal failure;

-residual oil leaks remaining from intense exposures after air con pack burn (engines run very hot to produce very high air con pack temperature so as to push any residual oil contaminants through the system)

- The AFAP also maintained that the problem in the BAe 146 involves the design of its engines and air conditioning system along with problems involving oil seals and filters.

- The AFAP claimed that:

The BAe 146 appears to have a higher proportion than normal of oil leakage into the aircraft air conditioning system. The engine bearing/seal system is allowing excessive oil to leak into the aircraft bleed air system,

- Dr Chris van Netten of the University of British Columbia commented that:

The engines used by the BAe 146 aircraft appear to have an inherent problem with leaking oils seals, specifically in the compressor section of the engine at bearing locations 1 and 9.

Bleed air, used for pressurisation of the aircraft, from the compressor stage of the engine can become contaminated with engine oil constituents. The temperature of the bleed air can be in excess of 500' Celsius. At this temperature any oil constituents will pyrolize resulting in smoke formation. The presence of smoke in the cabin has been well documented in these aircraft.

- and

…One is not exposed to carbon monoxide alone but a cocktail of pyrolised and original oil components. These combinations of exposures have not been studied and it would be wise to pay close attention, as this Committee is presently doing, to the complaints and symptoms of flight crew members.

- In its submission on this issue, CASA submitted that, early in its service, the BAe 146 did experience relatively higher engine malfunction rates than other similar sized aircraft:

… however the engine reliability rates are now comparable to all other transport category aircraft of a similar size. Early engine problems included poor reliability of engine bearing oil seals, which resulted in engine oil mist being present in the air which is bled from the engines for cabin air conditioning. Tests have shown, however, that even with engine bearing oil seals missing close to where the bleed air is ducted from the engine to feed the air conditioning packs, oil residues do not pass through to the cabin environment.

#### Pack burns

- A pack burn is a process under which, before the first flight by an aircraft on a given day, the first officer manually adjusted the aircraft air conditioning outlet temperature to a very high setting in order to vaporise residual oil traces in the air conditioning ducts of the BAe 146. As the procedure was normally carried out in the morning before aircraft operations commenced, cabin crew were often exposed to large amounts of smoky residue flushed from the ducting.

- The Flight Attendants’ Association of Australia gave this additional information concerning pack burns:

Pack burn offs were introduced in March 1997 as a BAe 146 Odour Inquiry Committee initiative. The intention was to remove the engine oil gathered in the sump, near the cabin air inlet, during overnight stops. It was demonstrated that pack burn-offs were counter-productive because they loaded the filters with carbon and bi-products from the burnt oil and the loaded filters were then unable to remove the contaminants from the air destined to air condition the cabin and flight deck. Pack burn offs were discontinued as a routine procedure in mid 1998.

The process of pack-burn offs was used regularly on BAe 146 aircraft as it was believed that it cleansed the air conditioning systems, and thus reduced odour occurrences.

After crew reported odours a pack burn would routinely be ordered during the turn-around or before start up for the next day’s duty.

- and,

Allied Signal, the APU manufacturers, warned of the danger of pack burns in their Richard Fox Report of November 1997.

“Total contaminant levels, in the supply air to the cabin exceed 50% of the current Safety Standard Limit (NIOSH, ACCIH) during pack burn outs.

Compounds present include formaldehyde, tetro-hydrofuran, and cumene. These compounds are recognised as causing skin, respiratory and eye irritation, as well as nausea and narcosis, if present in excessive levels. The majority of currently detected compounds do not have established exposure limits.”

Fox also states that “ this exposure can continue for some time after the completion of this procedure”.

- The FAAA drew attention to a 1997 Ansett notice to cabin crew on BAe 146 aircraft directing them not to remain on board during pack burns. Ansett also issued a notice to its engineering section to discontinue pack burn procedures, “… in line with the recommendations of the Fox Report”.

### Difficulty in finding the source of fumes

- A confidential submission to the inquiry set out the difficulties involved in precisely locating the source of fumes on board the BAe 146:

The result of this air condition design, the output temperature and pack contamination problems is that it is almost impossible to accurately locate the original source of an oil leak. In the event of more than one engine/APU leak combination, identification becomes almost impossible.

### Issue of toxicity in relation to exposure to fumes

- Many of the submissions from flight crew and medical witnesses to the inquiry asserted that members of some flight crews had suffered health effects from exposure to fumes due to toxic ingredients in the oil fumes which leaked into the aircraft cabin and were inhaled. The issue of toxicity is controversial and difficult to effectively measure. The ingredient of Mobil Jet Oil II identified as a possible source of neurotoxic effects is Tricresyl Phosphate, classified by the National Industrial Chemicals Notification and Assessment Scheme (NINAS), as toxic.

- The National Industrial Chemicals Notification and Assessment Scheme was established in 1990 under the Industrial Chemicals (Notification and Assessment) Act 1989 (Commonwealth). NICNAS is a statutory scheme with staff and support services provided by the National Occupational Health and Safety Commissions. The objection of the NICNAS program is to establish the scientific basis for safe chemical use by assessing industrial chemicals for occupational, public health and/or environmental effects. NICNAS’ submission to this inquiry states:

There are numerous case reports of human poisoning with TCP as a result of ingestion of adulterated or contaminated beverages, foods or drugs. In some cases transient gastro-intestinal symptoms such as nausea, vomiting and diarrhoea have occurred shortly after the ingestion, whereas the neurological symptoms are characteristically delayed and persistent. Initially, there are pain and paraesthesia in the lower extremities, with a mild impairment of cutaneous sensations and, at times, of vibratory sense. Muscle weakness may progress to paralysis of the lower extremities with or without an involvement of the upper extremities. Recovery can be extremely slow and extend over a number of months or years”.

- NICNAS’ submission also stated that the oil’s manufacturer, Mobil, acknowledges possible toxic components but considers the performance of the oil is certainly an important factor in its continual use:

The neurotoxicity of jet engine oil containing TCP has been reviewed in a recent paper from Mobil Business Resources Corporation and Mobil Technology Company (Mackerer et al., 1999). The paper states that although it has been known for many years that TCP contains neurotoxic components, lubricant formulators have been reluctant to replace the additive because of it’s excellent performance in critical applications.

- Dr Chris van Netten told the inquiry:

We have many different compounds which really have not been analysed yet… It appears, therefore, Mobil Oil has a rough idea of what the composition is of their oils but does not have a clear picture of the different isomers  that might be present.

This is very important when one is dealing, for instance, with a mixture of closely related compounds. I understand Chris Winder has discussed these with you in detail, so I do not want to go into a large amount of detail here, but we have many of these compounds. … I think it is very important for us to know what is in these oils because if we do not know what all these isomers are we cannot really state anything regarding their inhalation exposures and their toxicity.

- Submissions were also made suggesting that the combination of chemical components in fumes leaking into the aircraft could have as yet unknown toxic effects. One submission to the inquiry raised a possible connection between genetic/chromosomal damage and exposure to oil fumes, although the Committee is not able to verify assertions of this nature.

#### Toxic Exposure

- Between 1997 and 1999 the company Genetic Consulting and Testing Pty Ltd (GCAT) carried out blood tests on five people who flew on BAe 146. The results of these tests, contained in a submission to this inquiry, found that two of those tested showed evidence of having been exposed to “clastogenic and/or aneuploidogenic chemicals.” The report went on to state:

The finding of notable disturbances in three people from the same environment is compelling evidence that there was significant toxic exposure.

The absence of findings in the other two persons could be explained either by sampling error (we did not happen to detect abnormal cells because of the relatively small number of cells sampled) or that the two people did not have chromosome abnormalities. In our experience of repeat analyses … these results are most likely to mean that these two people do not have chromosome abnormalities. The finding of chromosome abnormalities is influenced by three factors. (1) The exposure (2) The person's genetic make-up and (3) The person's diet at the time of exposure; some foods are known to be protective.

- GCAT’s report concluded; “The chromosomes analyses performed on these 5 persons show evidence of exposure to significant levels of chemical toxins, sufficient to cause grave, short and long term health consequences.”

#### Tricresyl phosphate (TCP)

- The inquiry was told that jet engine lubricants can typically contain up to 3 per cent tricresyl phosphate as an antiwear agent.

- Dr van Netten stressed in his submission that his research indicated:

… all engine oils tested to-date contain, among many other compounds, tricresyl phosphate (TCP) isomers. Tricresyl phosphates have been associated with neurotoxic properties.

- Associate Professor Winder told the inquiry:

I believe that tricresyl phosphate is the particular chemical that causes the neurological problems that staff on planes have been exposed to. But I consider that in many of these exposures there is also a hydrocarbon component context, and it is possible that the hydrocarbon exposure may either exacerbate the effect or assist in increased absorption. I do not think it is necessarily one chemical. It may be one chemical, but it is possible that it could be exposure to other chemicals as well.

#### Aerotoxic syndrome

- In evidence to the inquiry Associate Professor Winder also asserted that symptoms reported by individuals after exposure to fumes on the BAe 146 were sufficiently consistent to indicate the development of a “discrete occupational health condition”. This condition was described as ‘aerotoxic syndrome’. Professor Winder said:

Aerotoxic syndrome is a syndrome which is associated with aircrew exposure at altitude to atmospheric contaminants from engine oil or other aircraft fluids temporarily juxtaposed by the development of a consistent symptomology of irritancy, chemical sensitivity and neurotoxicity.

- and

This syndrome may be reversible following brief exposures, but pictures are emerging of a chronic syndrome following significant exposures.

Ansett in particular questioned this assertion.

### Mobil Jet Oil II and the issue of toxicity

- While the majority of submissions to the inquiry from medical/occupational health professionals point to components of Mobil Jet Oil II as a source of health problems, some witnesses dispute the oil as a source of toxicity. Dr David Lewis, Chief Medical Officer with Ansett, told the inquiry in evidence:

Last year 1.4 billion people flew on aircraft lubricated by this oil. You take that over the last 35 years and this must be the longest clinical trial for any chemical in the history of medicine. With the 30 cases claimed by Balouet, that would make it a one in 800 million chance of developing aerotoxic syndrome.

- The National Industrial Chemicals and Assessment Scheme (NICNAS) has placed Mobil Jet Oil II on a list of chemicals for review and assessment. NICNAS has informed the Committee that Mobil Jet Oil II may be selected as a priority for review and assessment - subject to given direction from the government - outside bodies and other factors.

- NICNAS submitted a document titled ‘Mobil Jet Oil II Overview of Available Scientific Background Information’ to the Committee setting out information concerning the chemical ingredients in Mobil Jet Oil II. In the document, NICNAS points to the following information:

According to Material Safety Data Sheets (MSDS) provided by the Australian Federation of Air Pilots and Mobil Oil Australia Ltd, Mobil Jet Oil II contains >90 per cent synthetic esters and <10 per cent additives and /or other ingredients 3% (or 1-5%) TCP and 1% (or 1-5%) PAN. A MSDS from 1992 also lists 2-naphthalenamine, N-phenyl (CAS No 135-88-6) as an ingredient. This chemical, also known as phenyl-beta-naphthylamine (PBN) is listed in the Australian Inventory of Chemical Substances, too.

Tricresyl phosphate (TCP)

When heated to decomposition, it can emit highly toxic fumes of phosphorous oxides (HSDB 1999). …The critical effects of TCP include delayed neuropathy ascribed to the TOCP isomer and reproductive toxicity.

Neuropathy may occur after both single and repeated exposure to TOCP and is similar in its mechanism of action and manifestations to the delayed nerve damage induced by other organophosphates. Clinical signs of paralysis typically appear after a latency period of 1-4 weeks. Histologically, there are degenerative changes in the axons which gradually spread towards the cell body. The lesions are attributed to the metabolite saligenin cyclic ortho-tolyl phoshate, which irreversibly inhibits a subset of nervous system esterases called neuropathy target esterases (NTE). …

The neurotoxicity of jet engine oil containing TCP has been reviewed in a recent paper from Mobil Business Resources Corporation and Mobil Technology Company (Mackerer et all 1999). The paper states that although it has been known for many years that TCP contains neurotoxic components, lubricant formulators have been reluctant to replace the additive because of its excellent performance in critical applications. …

In reproductive toxicity studies in rates and mice, TOCP has been shown to cause histopathological damage to the testes and ovaries, morphological changes in sperm, decreased fertility in both sexes and decreased litter size and viability, against without a clear cut no observed effect level.

Human health effects

There are numerous case reports of human poisoning with TCP as result of ingestion of adulterated or contaminated beverages, foods or drugs (IPCS, 1990). In some cases transient gastro-intestinal symptoms such as nausea, vomiting and diarrhoea have occurred shortly after the ingestion, whereas the neurological symptoms are characteristically delayed and persistent. Initially, there are pain and parasthesia in the lower extremities with a mild impairment of cutaneous sensations and, at times, of vibratory sense. Muscle weakness may progress to paralysis of the lower extremities, with or without an involvement of the upper extremities. Recovery can be extremely slow and extend over a period of months or years. …

Hazard of the product as a whole

In the available MSDS it is stated that the “Worksafe classification” of Mobil Jet Oil II is ‘not hazardous by Worksafe criteria’.

- In its submission, Mobil Oil Australia Ltd responded to the NICNAS document by stating:

We are concerned that the lack of context for these toxicological profiles may result in the Secretariat [of this Committee] (or members of the public who otherwise review the document) concluding that the product displays certain health and safety risks, when in fact it does not. When the product is viewed as a whole, MJO is not a hazardous material as classified pursuant to the National Occupational Health and Safety Commission (NOHSC) Work Safe criteria … Also, scientific studies, including the recent work by Macker et. al. … demonstrate that exposure to jet oils does not pose a significant risk to human health.

- The Mobil Oil submission went on to set out a number of “specific concerns with respect to the presentation of information in the Overview” document prepared by NICNAS, namely the presentation of a ‘misleading picture of potential health and safety hazards associated with Mobil Jet Oil II”.

- Later in its submission the company stated:

… we believe the Overview could lead to undue public concern due to the statement regarding the presence of potential carcinogens in the product. In fact, these constituents are present as impurities only at trace levels and below the level at which an adverse health effect could occur.

- In a supplementary submission to this inquiry Mobil noted that:

We do not believe that Mobil jet turbine oils pose any significant toxicological risk to individuals accidentally exposed to aerosols or vapours in aircraft cabins. Such exposures are not what we would refer to as "normal use” but the cabin levels that can be reached during such exposures are comprehended by our internal and published risk assessments and are considered safe. These assessments are based on Mobil toxicology testing as well as the extensive toxicology database found in the published literature.

- In response to specific claims that Mobil Oil II is toxic the company stated:

… based on the toxicological data developed over the years, Mobil Jet Oil II is a non-hazardous product based on the NOHSC document "Approved Criteria for Classifying Hazardous Substances”. …

With regard to phenyl-napthylamine, Mobil Jet Oil II contains approximately 1% of the alpha isomer. Testing has confirmed that this concentration did not cause sensitization in animals or humans. The concentration of the carcinogenic beta isomer and beta naphthylamine which might be present as impurities is negligible to non-existent in Mobil let Oil II….

There has been much speculation that Mobil Jet Oil II may be the cause of the alleged adverse health effects. This is unsupported by the scientific evidence … The clinical symptoms reported are not the same as those that have been historically seen for TCP. The reported symptoms appear to closely match those for exposure to carbon monoxide (CO). These effects can be exacerbated by low oxygen levels and high carbon dioxide levels in the aircraft cabin. Possible sources for carbon monoxide include the ambient cabin air and, in the case of a malfunction of the aircraft mechanical systems, thermally degraded hydraulic oil and turbine oil that might enter the aircraft cabin. These oils may break down at very high temperatures and liberate carbon monoxide on contact with hot metal surfaces. Under these extreme conditions, carbon monoxide would be produced from virtually any oil and independent of any additives, including TCP, that might be present.

- During his oral evidence to the Committee, Mr Julian Plummer, Manger of Aviation Lubricant Sales with Mobil Australia made the following comment:

Mobil do not consider accidental exposure to oil vapours in an aircraft cabin to be ‘normal use’, but the levels that can be reached are comprehended by our internal and published risk assessments and are considered safe. …

The tricresyl phosphate (TCP) additive used in jet oils provides the lubricant with improved anti-wear and load carrying capability. Its properties are unique, and no replacement has been identified which can meet the stringent performance requirements of a modern jet engine oil. Our submission details that the TCP used in jet oil II is low toxicity, about 25 to 60 times less neurotoxic than TCPs used in the 1950s. …

Our risk assessment details that it is not possible to receive a harmful dose by inhalation at the threshold limit value of five milligrams per cubic metre, which would be visible mist. It is also not possible to receive a harmful dose from accidental skin contact, and there is no record of a jet oil formulated with modern conventional TCP causing human toxicity. …

Tricresyl phosphate is present in jet oil at approximately three per cent, which is around 30,000 parts per million. The neurotoxic components are orthoisomers which are only a small proportion of the TCP and are present at roughly 140 parts per million in the jet oil. Jet oil is neurotoxic if you drink it, and we have established doses for both a toxic one-off dose or a toxic ongoing dose, which is a smaller amount that you would have to ingest each day. Based on normal things, these would be impossible to achieve. …

The studies show that it is possible to breathe a mist. We are now talking about a mist of the oil which contains three per cent, whereas I suspect the 0.1 milligram per cubic metre that was mentioned as the NOHSC requirement – the maximum – was just TCP. I am now talking oil containing three per cent TCP. It is possible to breathe a mist at five milligrams per cubic metre, which is the accepted maximum workplace level for lubricating oils, five days a week, eight hours a day, in an ongoing sense without absorbing a toxic dose through inhalation. For dermal contact, we have established that it is possible to cover your entire body surface with the liquid for six hours and not absorb a toxic dose through the skin. Our prime warnings are against ingestion of a product. It has always been believed that more pure forms of TCP would not do the same lubricating job. It relied on the variety of molecules in there to perform the function.

- In contrast to these comments, Dr Jean Christophe Balouet told the inquiry on 13 March 2000:

… evidence presented to this committee suggests that covering the entire surface of the body with oil would not be hazardous. This may be the case for a mineral oil but not for a synthetic oil containing toxic ingredients.

- On 24 February 2000, Mobil Oil Australia replied to a series of questions put to the company by Associate Professor Winder. The following are excerpts from the response by Mobil Oil to Dr Winder’s inquiry, a copy of which was supplied to the inquiry:

From the historical literature, the reported symptoms from exposure to TCP consist of transient gastrointestinal complaints followed some days or weeks later by a progressively developing "dying back" neuropathy starting in the feet and migrating upward toward the hips; in some cases the hands are affected and paralysis migrates upward toward the elbows. This neuropathy is often referred to as organophosphate induced delayed neuropathy (OPIDN). There have been upwards of 60,000 human poisonings from TCP with remarkable similar symptomatology and neuropathology. We do not believe that other human toxic effects are produced by TCP whether the exposure is acute, subacute or chronic. …

In summary, we do not believe that any of the symptoms, reported by individuals claiming to have been exposed to mists or odours of Mobil Jet Oil 11, were caused by exposure to the oil or any of its components. Neurological effects claimed to occur from low-level chronic exposure, or cumulative effects from multiple exposures, are strictly anecdotal and are not supported by concurrent documentation of exposure or of biochemical, or pathological effects known to be produced in humans by TCP. In the absence of proven exposure and recognisable toxicologic sequelae known to be related to TCP, the allegations appear to be simply unfounded speculation. …

We believe that the toxicity of Jet Oil 11 would not be altered by reduced pressure or oxygen level - however, this would not necessarily be true of pyrolysis or combustion products of the oil. …

The more frequent symptoms, i.e. disorientation, blurred vision, impaired memory, altered coordination, nausea, loss of balance, headache, dizziness, increased heart rate, loss of consciousness, shortness of breath.... are consistent with hypoxia. 1 suggest that hypoxia might result from one or more of the following: low oxygen level, presence of carbon monoxide, elevated oxygen demand possibly resulting from increased muscular activity and/or hyperventilation possibly aggravated by high carbon dioxide levels and stress from lack of sleep.

Mobil's turbine oils are designed to meet appropriate standards for engine performance, safety and product stewardship. Our risk assessments define the conditions under which we consider Jet Oil 11 to be of negligible risk through inhalation, dermal and ingestion exposure. That information has been provided to our customers and is available publicly through our published papers in the peer-reviewed scientific literature. Based on the results of this research, we believe Jet Oil 11 is of negligible risk to maintenance workers, passengers, and flight staff potentially exposed to an oil vapour or mist. …

- The Committee notes NICNAS’ statement in it’s submission that its findings relate to absorption of TCOP through the skin and that ‘there are (sic) no reliable data on absorption via inhalation’.

#### Labelling of Cans Containing Mobil Jet Oil II

- In his evidence to this inquiry, Dr Winder drew the inquiry’s attention to a change that has occurred in label information on cans of Mobil Oil II, while displaying two oil can labels:

This is a container of Mobil jet oil 2 with a pre-1992 label which states:

Warning!

Contains Tricresyl Phosphate.

Produces paralysis if taken internally.

Do not use as a medicine or food product.

Wash thoroughly after handling.

- Dr Winder went on to comment:

The label was modified after 1992. The small square is the warning on the pre-1992 label and the warnings are now in this white box in 13 languages. It says:

Warning!

Contains Tricresyl Phosphate.

Swallowing this product can cause nervous system disorders including paralysis.

Prolonged or repeated breathing of oil mist, or prolonged or repeated skin contact can cause nervous system effects.

- According to Dr Winder; “The important thing is it is recognised that the tri-orthocresyl, especially the orthocresyl phosphate containing molecules in the tri-orthocresyl mixture, cause nervous system effects.” He went on to state; “While I accept it is unlikely that anybody flying and exposed to this material is going to get paralysis sufficient that they would need to be put in a wheelchair for the rest of their lives, I do not accept that lesser exposures do not cause other nervous system or even neuro behavioural effects.”

- In Mobil Australia’s supplementary submission to the inquiry the company dealt with the issue of how their cans of Mobil Oil II came to be labelled in the way they are. According to the company:

… animal studies showed the jet oils tested, containing a maximum of 3% TCP, might be potentially harmful. Subsequently we updated the product Material Safety Data Sheets to include this information and recommended that exposure via skin, inhalation and ingestion be minimised. The emphasis was on ingestion as there had been reports that individuals in certain developing countries may have suffered from delayed neurotoxic effects after ingestion of foodstuffs or beverages adulterated with aryl phosphate esters. …

and

A formal risk assessment was conducted by Mobil in 1990 which provided confirmation that ingestion was, in fact, the principal route of exposure that could potentially produce neurotoxic effects. Because of the ingestions that had earlier been reported, it was decided that communication (through labelling) of this potential ingestion hazard to individuals working directly with the jet oils was appropriate. The risk assessment clearly showed that a potentially harmful dose is not possible via inhalation at levels at or even higher than Threshold Limit Value of 5.0 mg/m3 for the oil mist. These levels would produce a clearly visible oil mist. Also, an accidental contamination of the entire body surface with an oil containing 3% TCP for 6 hours would not result in the absorption of more than an estimated non-toxic single dose. …

Additional joint toxicology studies by Mobil and a major manufacturer of TCP confirmed that an oil with 3% TCP could produce neurotoxic effects in animals administered very high oral doses. This led Mobil to adopt a very conservative labelling approach for its jet oils by including language recommending minimising exposure by all routes and emphasising the importance of good personal hygiene practices. The decision was made in the early part of 1997 and labelling was phased in during the year. …

Mobil's decision to label these products was based solely on its own policies and product safety stewardship practices. …

In summary, recent changes that have been made to the label and Material Safety Data Sheets do not reflect any underlying change in product composition or any new information about health hazards. Mobil Jet Oil 11 has been, and continues to be, safe for its intended purposes. The changes to the label were based solely on Mobil's own product stewardship practices and a very conservative approach to labelling, It must be emphasised that the revised labelling and MSDS statements do not reflect new information on Mobil Jet Oil II, suggesting hazard, where none exited before.

- Mr Plummer of Mobil Oil also told the inquiry:

… we resubmitted the basis for our labelling and that Mobil jet oil II is non-hazardous by Worksafe criteria to the National Occupational Health and Safety Commission. On 17 June last year we received their reply, agreeing that our labelling is correct and that Mobil jet oil II is correctly classified as non-hazardous.

#### Development of a new Mobil jet oil

- The inquiry was told by British Aerospace that trials are currently taking place in Australia and Europe to produce a Jet oil with different contents to those in Mobil Jet Oil II. Mr Black of British Aerospace noted:

“Though recognising that no firm scientific link has been made between the sick people and the contents of this oil, we have immediately launched an action to try to change to the oil which is claimed to not have these things in it.”

and “We have never actually seen any of these dangerous chemicals getting through into the cabin. Nevertheless, due to that fact that those chemicals exist in the oil and that we have sick people at the other end of the chain, we have initiated this action to try to find a better oil which does not contain those constituents.”

- Mr Plummer of Mobil Oil told the inquiry his company has developed a new jet oil named jet oil 291. Mr Plummer advised that:

… we have got a product now which has lower deposit forming tendencies both in the liquid and vapour phase …. it has a non-toxic additive pack. We have developed a TCP which has effectively eliminated the ortho isomers, which were the 140 parts per million of toxic elements previously.

- Mr Plummer went on to stress that:

… we want to make it clear that the oil was not developed just to eliminate the toxic elements of TCP. That was just one of a number of development parameters for it.

- Captain Trevor Jensen of Ansett told the inquiry:

Ansett is currently trialing Mobil 291, a new generation oil, on the BAel46-300 series aircraft. Depending on the results of the trial (engine wear and tear, etc.) Ansett will investigate using Mobil 291 on the other aircraft types.

- The claimed advantages of the new oil were not supported by Dr Balouet when he stated:

I understand that evidence presented to the committee suggests that new generation modern jet oils have been modified so that the concentrations of some toxic ingredients will be reduced. Please note that these jet oils are still being tested and are not yet in commercial use. Jet engines still contain the older generation of jet oils, known to be toxic, while removing TOCP will not necessarily solve the problem.

### The Alysia Chew case

- On 28 April 1999 a judgement was delivered in a case brought by an Ansett cabin crew member, Ms Alysia Chew, heard in the Compensation Court of New South Wales. The basis of Ms Chew’s claim was that between January 1992 and 30 October 1993, when a flight attendant with Eastwest Airlines, she was exposed to fumes, toxic substances and other irritants whilst carrying out duties as a flight attendant on BAe 146 aircraft. Ms Chew also claimed that fumes within the aircraft to which she was exposed contained Mobil Jet Oil II which contained the substance triorthocresyl phosphate (TOCP).

- Ms Chew alleged two alternative causes for her illness:

  - that TOCP caused damage to her physiology which gave rise to her chronic ongoing symptoms and disabilities diagnosed by her doctors as Multiple Chemical Sensitivity (MCS); or

  - Alternatively she alleged her symptoms and incapacity resulted from aggravation of a condition of glandular fever or a viral infection described as Epstein Barr virus.

- In respect of Ms Chew’s first basis of claim, Justice Moran noted that:

The applicant puts her case in the alternative so I do not think it necessary for me to decide whether or not a diagnosis of multiple chemical sensitivity is appropriate in this case. I must say at the outset though that there certainly is a weight of medical evidence in this case against such a label …

I prefer the evidence given by the respondent's doctors, in particular, Dr Carroll and Professor Loblay, that the diagnosis of multiple chemical sensitivity is wrong and that the applicant is suffering from an aggravation of glandular fever or Epstein Barr virus.

- Justice Moran ultimately found in Ms Chew’s favour in respect of her second submission and decided that:

- Ms Chew suffered injury arising out of and in the course of her employment with the respondents from January 1992 to 30 October 1993; and

- Section 47 of the Workers Compensation Act applies and that the applicant as a result of the injury, “is unable without substantial risk of further injury to engage in employment of a certain kind because of the nature of that employment shall be deemed to be incapacitated for her employment at that kind.”

- Justice Moran also commented that evidence in relation to contaminants was as follows:

The levels of measured chemical contaminants in the cabin air were not a threat to the health of aircrew or passengers.

Contaminant levels were well below internationally accepted occupational health standards and cannot precipitate any chronic disorders.

The levels of contaminants were hundreds to thousands of times below those levels known to cause neurotoxic sequelae.

- In its submission to the inquiry Ansett referred to the Chew case stating that:

The Compensation Court of NSW made a decision in April 1999 that a Flight Attendant's rare pre-existing viral condition was aggravated by exposure to fumes aboard a BAe 146. The judge accepted expert evidence from witness Dr Crank that there was no toxicity in the fumes coming into the cabin and that they posed no threat to anyone without an extraordinary susceptibility, such as the claimant.

### Medical evidence

- During its public hearing in Sydney on 1 February 2000 the Committee heard evidence from medical professionals, Dr Mark Donohoe and Dr Richard Teo, both of whom have examined patients affected by fumes while working on BAe 146 aircraft. Both Dr Donohoe and Dr Teo gave evidence supporting claims that exposure to fumes on BAe 146 have led to long term illness and evidence of neurotoxicity.

- Dr Donohoe told the inquiry:

I am saying that in this case we have doctors and toxicologists saying that there are long-term health problems. In other words, people who have been exposed to these fumes and developed short-term symptoms at the time have had long-term consequences.

- Dr Teo in his evidence to the inquiry stated:

… the people I have tested have been affected about two years, and they are still not good. So for two years I can say they are not good.

- Dr Robert Loblay put a contrary view at the hearing in Sydney on 1 February 2000:

Almost anything can be toxic if given in sufficiently large dose or if a person is exposed to a sufficient quantity. The real question is: are the people in the cabin – under normal cabin conditions, not when there is a leak – exposed to levels of any of these compounds which could conceivably cause toxic effects? The evidence the expert panel was presented with seemed pretty clear, that that was not at all likely.

### Other clinical symptoms

- It is apparent that although some crew members have reacted to the fumes on the BAe 146, other crew members have had limited, or no reaction to the same exposure. One confidential submission to the inquiry advised that it appeared women were more susceptible than men to the fumes. Dr Winder stated:

There are a whole range of individual reasons why exposure may be increased and also a whole range of issues related to susceptibility. … There are a range of different factors which may underlie why some people are more affected than others.

- Dr Balouet told the inquiry in evidence:

… individual susceptibility is not the same with all people around the world and even within a small population. Some people would be really allergic, for example, to a compound and others would not. …

There might be genetic factors interfering with these problems. In fact, there are a number of enzymes, one of which especially play a major role in eliminating and controlling the effects of organophosphates. Particularly what we have seen from the preliminary studies is that the people sharing the two same enzymes will show very high effects, while those people who have either the R type or the 2 plus R type will not have such severe symptoms.

- In his evidence to the inquiry Dr Chris van Netten noted that:

The most sensitive people get sick first and they are your early warning signs of a potential problem. Often these people are looked upon as hypochondriacs or complainers or whatever else, and this is doing the system an injustice because it is actually quite dangerous to not pay attention to these people. They are really your early warning signs that something is wrong. The person next to you might get sick now because of a certain different physiology or background whereas you might get sick next, and this is the important component we have to worry about.

- Captain Frank Kolver of NJS commented on the health effects he suffered following his exposure to fumes, an incident dealt with later in this report:

In my experience, after the first incident I seemed to become sensitive to very strong chemical smells. I think I noted in my submission an example of when I would go into a hardware store and walk past the shelf with insecticides and pesticides. If I continued to stay there, probably within the next 10-15 minutes I would start getting a headache. This was predominantly once again a pain in the left temple. Some other chemical effects were exhaust fumes from motor vehicles in dense traffic and some strong chemically based perfumes. They all seem to have some effect. If I did not do so something about getting away from the source, I would start trying to suffer a headache….The problem we want to solve here is to rectify the problem we have which we believe has been caused by oil fumes.

See – Submissions 2, 3, 5, 6, 7

Submission 14A, AFAP, p 6

Submission 6, Associate Professor Chris Winder, p 12

Submission 6, Associate Professor Chris Winder, pp 13-14; see also Submission 5, Dr Jean Christophe Balouet, p 1, Dr Jean Christophe Balouet, Evidence, 13 March 2000, p 172, Submission 2, Dr Mark Donohoe, pp 1-2.

Dr Robert Loblay, Evidence, 1 February 2000, p 103

Dr Robert Loblay, Evidence, 1 February 2000, p 104

See Balouet, Winder, FAAA submissions.

Dr Robert Loblay, Evidence, 1 February 2000, p 104

Submission 6, Associate Professor Chris Winder, p. 2

Submission 6, Associate Professor Chris Winder, p 53 (subs vol 1)

Submission 6, Associate Professor Chris Winder, p 6

Submission 6, Associate Professor Chris Winder, p 4 See also BASI Occurrence Brief 199702276 (on internet site) incident involving Captain Kolver on 10 July 1997, p 2.

Submission 6, Associate Professor Chris Winder, p 4

Ansett Australia, Evidence, 2 November 1999, p 52

Mobil Oil Australia Ltd, Evidence, 1 February 2000, p 142

Submission 14A, AFAP, p 4; see NJS, Evidence, 1 February 2000, p 139

Submission 14A, AFAP, p 4; see also Submission 8, Associate Professor C. van Netten, p 12.

Submission 14A, AFAP, p 3

Submission 8, Associate Professor C. van Netten, p 1

Associate Professor C van Netten, Evidence, 14 March 2000, p 214

Submission 20, CASA, p 3

Confidential submission C6

Submission 24, FAAA, pp 7-8

Submission 24, FAAA, p 8

Submission 24, FAAA, p 8

Confidential submission 11, p 4

National Industrial Chemicals Notification and Assessment Scheme, Annual Report, 1991-9, p 47

Submission 12, NICNAS, p 6

Submission 12, NICNAS

An isomer is a member of a group of chemicals. TCP has 10 isomers.

Associate Professor C van Netten, Evidence, 14 March 2000, p 207 - 208

Submission 7, GCAT, pp 2-4

Submission 7, GCAT, p 5

Submission 7, GCAT, p 5

Submission 7, GCAT, p 5

Submission 8, Associate Professor C. van Netten, p 14

Submission 8, Associate Professor C. van Netten, p 2.

Associate Professor Chris Winder, Evidence, 1 November 1999, p 16

Associate Professor Chris Winder, Evidence, 1 November 1999, p 6

Associate Professor Chris Winder, Evidence, 1 November 1999, p 6

Ansett Australia, Evidence, 2 November 1999, p 64

Ansett Australia, Evidence, 2 November 1999, p 64

Submission 12, NICNAS, pp 3-11

Submission 13, Mobil Oil Australia Ltd, p 1

Submission 13, Mobil Oil Australia Ltd, pp 1-2

Submission 13, Mobil Oil Australia Ltd, p 2

Submission13A, Mobil Oil Australia Ltd, p 2

Submission 13A, Mobil Oil Australia Ltd, p 3

Submission 13A, Mobil Oil Australia Ltd, p 4

Mobil Oil Australia Ltd, Evidence, 1 February 2000, pp 141-142

Dr Jean Christophe Balouet, Evidence, 13 March 2000, p 173

Correspondence from Ms S Potts, Manager External Relations, Mobil Oil Australia to the Secretariat dated 5 April 2000 containing letter to Associate Professor Chris Winder dated 24 February 2000, pp 3-4.

Submission 12, NICNAS, p 4

Associate Professor Chris Winder, Evidence, 1 November 1999, p 8, see also AFAP, Evidence, 1 February 2000, p 113.

Associate Professor Chris Winder, Evidence, 1 November 1999, p 8

Associate Professor Chris Winder, Evidence, 1 November 1999, p 8

Submission 13A, Mobil Oil Australia Ltd, pp 3-4

Mobil Oil Australia Ltd, Evidence, 1 February 2000, p 141

British Aerospace, Evidence, 2 November 1999, p 85

British Aerospace, Evidence, 2 November 1999, p 86

Mobil Oil Australia Ltd, Evidence, 1 February 2000, p 142

Mobil Oil Australia Ltd, Evidence, 1 February 2000, p 142

Correspondence from Captain Jensen to the Committee dated 11 February 2000, attachment p 2.

Dr Jean Christophe Balouet, Evidence, 13 March 2000, p 173; see also Submission 14B, AFAP,
pp 17-18.

Alysia Chew v Eastwest Airlines Ltd & Ansett Australia Ltd, Compensation Court of New South Wales, (Matter 19652 of NSW), Moran J.

Judgment given in the Alysia Chew case heard in the Compensation Court of New South Wales and delivered on 28 April 1999, pp 1-2.

Judgment given in the Alysia Chew case heard in the Compensation Court of New South Wales and delivered on 28 April 1999, p 10.

Judgment given in the Alysia Chew case heard in the Compensation Court of New South Wales and delivered on 28 April 1999, pp 12-13.

Submission 11, British Aerospace, p 4; see Judgment given in the Alysia Chew case heard in the Compensation Court of New South Wales and delivered on 28 April 1999, pp 8-9.

Submission 22, Ansett Australia, p 5

Dr Mark Donohoe, Evidence, 1 February 2000, p. 95

Dr Richard Teo, Evidence, 1 February 2000, p. 95

Dr Robert Loblay , Evidence, 1 February 2000, p 105

Confidential submission C11, p 3

Associate Professor Chris Winder, Evidence, 1 November 1999, p 13

Dr Jean Christophe Balouet, Evidence, 13 March 2000, p 176

Associate Professor C. van Netten, Evidence, 14 March 2000, p 210, see also pp 208-209

Captain F Kolver, Evidence, 2 February 2000, p 149