COMPUTER PROGRAMS FOR LOCATING THE MOON

EIMAC DIVISION OF VARIAN

301 INDUSTRIAL WAY

SAN CARLOS, CALIFORNIA

In  two  previous  issues  of the Eimac EME notes, methods

were described for determining antenna aiming data to "hit" the

moon.  In bulletin AS-49, there was a reprint of an article  by

Don Lund, WA0IQN, an article by Victor Michael, W3SDZ, and some

letters  to  the  technical  editor  of QST from Alan Goodacre,

VE3BZS, all  discussing  antenna  aiming.   Also,  in  bulletin

AS-49-1   more   information  on  aiming  data  was  presented.

Bulletin AS-49-1 stressed the use of the Nautical  Almanac  and

the  Hydrographic Office publication HO-214.  It is possible to

determine  the  moon's  position  by   means   of   those   two

publications  without  extensive  mathematics.  However, as one

becomes more involved in the EME project,  the  habits  of  the

moon  become  interesting.  Also, more sophisticated methods of

tracking the moon become attractive.

For those persons having  access  to  handheld  scientific

calculators,   new  techniques  become  available.   For  those

Amateurs having access to a computer, either punch-card  or  an

active  terminal,  another  interesting  hobby  of  writing and

modifying programs develops.

In  this  issue  of  the  EME  notes,  four  programs  are

presented  which  may be of interest.  One is in BASIC, two are

in TYMSHARE SUPERBASIC and one in TYMSHARE SUPERFORTRAN.  As is

usually the case, these  programs  will  probably  need  to  be

modified  to  run  on  the  machine available to the individual

amateur.

PROGRAM 1 [MOONLOC1.BAS]

LANGUAGE: BASIC

This program will run on  almost  all  machines  accepting

BASIC.  The operator must put in data from the Nautical Almanac

for  the  year,  month,  day  and  hours that the elevation and

azimuth is desired.  Also, the latitude and  longitude  of  the

geographic  location in question must be entered.  Statement 20

allows the operator to put in the time  interval,  in  minutes,

between each calculation.

The origin of this program is not known.

PROGRAM 2 [MOONLOC2.BAS]

LANGUAGE: TYMSHARE SUPERBASIC

The  program  is  a  modified  version  of program 1.  The

output format is slightly different.  Also, limits can  be  put

on   the   azimuth  and  elevation  angles  at  the  operator's

discretion.  The program requires latitude  and  longitude  and

time  interval  inputs.   Also,  Nautical  Almanac data for the

year,  month,  day  and  hours  must  be  put  into  the   data

statements.   Statement 610 allows the operator to put the data

in  for  identification.   The  computer  does  not  use   this

statement in the calculations.  The computer will pause and let

you  type  in  the  date.   When inserting the date, do not use

commas for seperating the month, day, and year.  If you do, the

computer will become confused.  When  the  carriage  return  is

depressed the computer will carry on.

PROGRAM 3 [MOONLOC3.BAS]

LANGUAGE: TYMSHARE SUPERBASIC

This  program  was  originally  written  in BASIC by Lance

Collister, WA3GPL.  However, the  BASIC  used  on  the  General

Electric  terminal was sufficiently different from the TYMSHARE

SUPERBASIC that the program would not run.  A few modifications

were made to allow  the  TYMSHARE  machine  to  run.   What  is

presented  here will not run on the GE computer.  Perhaps those

who attempt to use this  program  will  have  to  make  further

changes for their individual cases.

The  program  is  very  useful  because no almanac data is

required.  The program  allows  the  calculation  of  the  moon

position  by  comparing back to the position of the moon at the

start of the Julian calendar.  The operator need  put  in  only

the  latitude,  longitude,  time interval, month, day, year and

hours between which aiming data is  required.   If  rising,  or

setting,   moon   calculations   are   required,  the  computer

determines the time and will execute the printout requested.

PROGRAM 4 [MOONLOC4.FOR]

LANGUAGE: TYMSHARE SUPERFORTRAN

This program was supplied by Louis  Anciaux,  WB6NMT.   As

received,  the  program  was on tape and written in FORTRAN IV.

Again,  the  program  was  modified  to   run   with   TYMSHARE

SUPERFORTRAN.   This  program  requires  nautical almanac data.

However the GHA and declination for 0000, 0600, 1200, 1800, and

2400 hours GMT is all  that  is  required.   The  program  will

interpolate  for  all  othe  times.   Of  course, the latitude,

longitude, and time interval for the calculation must be  typed

in.

PROGRAM 5 [MOONLOC5.BAS]

LANGUAGE: MITS ALTAIR BASIC

This  program  is  a version of Lance Collister's (WB7CCI)

original moon tracking  program  written  in  GE  BASIC.   This

particular  version  was modified by Jay Liebmann, K5JL, to run

on a MITS Altair 8800B.  The program requires about 6700  Bytes

of memory.

##################################################################################