Snowcrest Terminal Emulator

The Snowcrest Terminal Emulator program allows the IBM Personal Computer

series to act as an asynchronous terminal at speeds up to 19,200 baud.  The

emulator program will operate through either the COM1 or COM2 asynchronous

ports of the computer.  Communication can use any parity scheme (8 bit, even

parity, odd parity, forced mark or forced space).  Information received from

the remote computer can be saved on the printer or on the disk (or both).

The user may even pass control to DOS, run other programs and return to the

terminal program without losing information arriving from the remote computer.

Operation -

The terminal emulator program is first copied from the distribution disk to

the user's DOS disk using the DOS copy command.  The name of the program is

SNOTERM.EXE.

The program is then executed by typing SNOTERM.

The copyright screen will appear and wait for you to type any key.  The

terminal screen will then appear.  This screen is 24 lines high by 80 columns

wide.  The 25th line (status line) details the use of the function keys.

f1- This key ends the terminal emulator and returns to DOS.

f2- This key sends a BREAK signal to the remote computer.  The signal will

continue to be sent until this key is pressed again.

f3- This key cycles the baud rates.  Each time the key is pressed a new baud

rate will be selected.

f4- This key cycles the parity options. "8 bit" will send and receive all

eight bits of the keys selected (to send the high bits, it would be

necessary to use the ALT-numeric pad sequence.  That is, to send an "a"

with the high bit on, hold down ALT and type 225 on the numeric keypad.)

"Even" or "odd" will send and receive even- or odd-parity data.  "Mark"

will send and receive data with the parity bit always on.  "Space" will

send and receive data with the parity bit always off.

f5- This key turns the Data Terminal Ready line (dtr) on or off on the COM

port which is selected.  Data Terminal Ready is pin 20 of the connector

and if used with a modem will cause the modem to hang up the telephone

when turned off.

f6- This key turns the Request to Send line (rts) on or off on the COM port

which is selected.  Request to Send must be on for most modems to allow

the computer to transmit data.

f7- This key will return to DOS while not ending the terminal emulator.

Other programs may be run as long as they do not use the selected COM

port. The terminal screen will continue to be updated. Data being sent to

the disk or the printer will continue to be collected (up to the size of

the internal buffers - about 25000 bytes), but that data will not be

written to the printer or disk until the user returns to the terminal

emulator program by typing "exit" to any DOS prompt.

f8- This key selects COM1 or COM2 for communications.

f9- This key turns the printer on or off.  The printer will print information

as it is received.

f10-This key turns disk collection on or off.  All information being received

will be APPENDED to a disk file named CAPTURE.TXT.

This program is provided as a "Freeware" concept program.  Although it is

copyrighted, Snowcrest extends permission to make copies FOR PERSONAL USE.

This program may not be used for commercial gain without written permission

from Snowcrest.  Sources in 8086 Macro Assembly Language are available by

sending $39.95 to Snowcrest Computer Specialties, 527 McCloud Ave., Mt.

Shasta, CA  96067.  Also included are sources to SNOBACK and SNOREST and

the documentation binder for SNOBASIC.

######################################################################################################################