PodYakFlat

[image: image1.png]



PodYakFlat  is a simple Kayak with a flat top and bottom. It is 14 feet long, and has a max width of  just over 30 inches. It has a capacity of 325 lbs, with a weight of approximately 45-55 lbs, if ¼” ply is used. If 1/8” ply is used, it will be significantly lighter, on the order of 25-35 lbs. Interior stiffeners will be needed, and a hole cut in the top panel for the paddler with a seat or pad installed inside. I recommend that air chambers be installed in the ends, for flotation. Construction is of Stitch and Glue.

	Bottom Panel Half

	Station
	X
	Y
	X2
	Y2

	1
	0      
	0      
	0      
	4  1/2 

	2
	0      
	0      
	12      
	6  1/8 

	3
	0      
	0      
	24      
	7  3/8 

	4
	0      
	0      
	36      
	8  1/16

	5
	0      
	0      
	48      
	8  1/2 

	6
	0      
	0      
	60      
	8  7/8 

	7
	0      
	0      
	72      
	9      

	8
	0      
	0      
	84      
	9      

	9
	0      
	0      
	96      
	9      

	10
	0      
	0      
	108      
	8  7/8 

	11
	0      
	0      
	120      
	8  1/2 

	12
	0      
	0      
	132      
	8  1/16

	13
	0      
	0      
	144      
	7  3/8 

	14
	0      
	0      
	156      
	6  1/8 

	15
	0      
	0      
	169      
	4  1/2 

	Side Panel First Chine

	Station
	X
	Y
	X2
	Y2

	1
	0      
	2  5/8 
	0      
	2  5/8 

	2
	12      
	1 15/16
	12      
	4  7/8 

	3
	24      
	1  1/4 
	24      
	6  3/4 

	4
	36      
	  3/4 
	36      
	8      

	5
	48      
	  3/8 
	48      
	8 15/16

	6
	60      
	  1/8 
	60      
	9  3/16

	7
	72      
	0      
	72      
	9  3/8 

	8
	84      
	0      
	84      
	9  3/8 

	9
	96      
	0      
	96      
	9  3/8 

	10
	108      
	  1/16
	108      
	9  3/16

	11
	120      
	  5/16
	120      
	8 15/16

	12
	132      
	  9/16
	132      
	8      

	13
	144      
	1  1/4 
	144      
	6  3/4 

	14
	156      
	1  9/16
	156      
	5  1/4 

	15
	169  5/16
	2  5/8 
	169  5/16
	2  5/8 

	Side Panel Second Chine

	Station
	X
	Y
	X2
	Y2

	1
	0      
	7  7/16
	0      
	7  7/16

	2
	12      
	4      
	12      
	7  3/4 

	3
	24      
	2 15/16
	24      
	8      

	4
	36      
	1 11/16
	36      
	8  1/4 

	5
	48      
	  3/4 
	48      
	8  3/8 

	6
	60      
	  3/16
	60      
	8  7/16

	7
	72      
	0      
	72      
	8  1/2 

	8
	84      
	0      
	84      
	8  1/2 

	9
	96      
	0      
	96      
	8  1/2 

	10
	108      
	  3/16
	108      
	8  7/16

	11
	120      
	  5/8 
	120      
	8  3/8 

	12
	132      
	1  1/2 
	132      
	8  1/4 

	13
	144      
	2 15/16
	144      
	8      

	14
	156      
	4      
	156      
	7  3/4 

	15
	169  1/8 
	7  7/16
	169  1/8 
	7  7/16


	Top Panel Half

	Station
	X
	Y
	X2
	Y2

	1
	0
	0
	0
	4  1/2

	2
	0
	0
	12
	6  1/8

	3
	0
	0
	24
	7  3/8

	4
	0
	0
	36
	8  1/16

	5
	0
	0
	48
	8  1/2

	6
	0
	0
	60
	8  7/8

	7
	0
	0
	72
	9

	8
	0
	0
	84
	9

	9
	0
	0
	96
	9

	10
	0
	0
	108
	8  7/8

	11
	0
	0
	120
	8  1/2

	12
	0
	0
	132
	8  1/16

	13
	0
	0
	144
	7  3/8

	14
	0
	0
	156
	6  1/8

	15
	0
	0
	169
	4  1/2


All measurements are in inches. The end points of the plotted lines are joined by a straight line.

[image: image2.png]



This shows how the measurements are used to lay out the panels. X and Y are the bottom line of the panel and X2 and Y2 are the top line. All points are from the edge of the ply or a baseline. Any points which are N/A are not plotted. They may just be not on the panel or are not important to that line. The two end points are closed with a straight line.

For information on Stitch and Glue, please check the construction of fisher10. The hull was done entirely with stitch and glue, except for the transom/side panel joint, which was first attached with screws and then filleted on the inside and taped on the outside. There are also many good sites with Stitch and glue information. www.bateau.com has an excellent primer on the subject, so do most of the other major plans dealers, such as Glenn-L and Clark Craft. 

