���������������������������������������������������������������������������

�                                                                         �

�                                                                         �

�                                                                         �

�              �����          �����        �������       ���              �

�            ��     ��      ��     ��      ��          �� ��              �

�            ��     ��      ��     ��      ��             ��              �

�            ��     ��      ��     ��      �� ��          ��              �

�              �����        ��     ��      �������        ��              �

�            ��     ��      ��     ��            ��       ��              �

�            ��     ��      ��     ��            ��       ��              �

�            ��     ��      ��     ��      �     ��       ��              �

�              �����          �����         �����         ��              �

�                                                                         �

�                                                                         �

�                                                                         �

�                                                                         �

�                                                                         �

�                                                                         �

�   H    H     A     RRRRr    DDDDd    W     W     A     RRRRr    EEEEEE  �

�   H    H    A A    R    R   D    D   W     W    A A    R    R   E       �

�   H    H   A   A   R    R   D    D   W     W   A   A   R    R   E       �

�   HHHHHH   AAAAA   RRRR     D    D   W  W  W   AAAAA   RRRR     EEEEE   �

�   H    H   A   A   R   R    D    D   W W W W   A   A   R   R    E       �

�   H    H   A   A   R   R    D    D   W W W W   A   A   R   R    E       �

�   H    H   A   A   R    R   DDDD      W   W    A   A   R    R   EEEEEE  �

�                                                                         �

�                                                                         �

�                                                                         �

�                                                                         �

���������������������������������������������������������������������������

Hardware description of the 8051 Microcontrollers

Esslingen in August 1991

Note from the English translators:

----------------------------------

The objective of this translation was to remain as close as possible to

the original German, rather than having a paraphrase. Such translations

are bound to come across as stilted and colloquial. While we have tried

to clear up obvious errors, please note the disclaimer from the original

author about the correctness of the document. However we hope that

English readers will find items of benefit in their 8051 programming tasks.

While we will be grateful for indicators of any translation errors, please

direct all other queries to the original author.

Tracey Lee tracey@sccc.org.sg    Singapore, March 1997

Anthony Khoo

Forward:

--------

This documentation is for beginners, and pertains to the setup and

function of microcontrollers from the MCS-51 family. It is comprehensive

since incomplete knowledge about the hardware would conceal rather than

help in the understanding of the subject.

The documentation was written, so that it may be read like a textbook.

It will try not to introduce ideas which are covered in later chapters.

This naturally is not always possible, when the individual components depend

on each other.

However it attempts to give each keyword as much background information

as possible within individual chapters, so that problems specific to it may

be referred to.

When possible, in the various chapter short examples are given. In addition

in an extra chapter, as an example, the set up and programming of a computer

card with two different PCs connected by serial ports are described.

The basic type of the MCS-51 family is the 8051 controller from Intel.

Other types from Intel, and controllers designed by other companies based on

this family possess the same functions as the 8051. Some have added on

several extra functions. Hence the 8052 has an extra timer and a larger

internal RAM.

The 8031, 8751 types are identical to the 8051, but they possess no internal

ROM and an internal EPROM, respectively.

First we will look at only the 8051, explaining its architecture. Following,

several functions of the other CPU's of the MCS-51 family will be considered.

I cannot guarantee the completeness and absolute correctness of the

documentation. I am always grateful for additions, tips and error corrections.

(c)1991,1993    Werner Hennig-Roleff

Sulzgrieser Str. 101

73733 Esslingen

0711/376718

Table of contents:

-------------------

1.   8051 Characteristics   ................................... 1

2.   Data and Address     ...................................   2

3    Arithmetic core of the 8051 ............................   4

3.1    8051-block diagram                                       4

3.2    Timer/ Cycles                                            7

3.3    Command execution                                        8

3.4    Special Function Register                               10

3.4.1    ACC                                                   11

3.4.2    B                                                     11

3.4.3    PSW                                                   12

3.4.4    SP                                                    13

3.4.5    DPTR = DPH:DPL                                        14

3.4.6    PCON                                                  14

3.5    Ports                                                   16

3.6    external program memory access                          20

3.7    external data memory access                             23

3.8    Power on for an external Bus                            25

3.9    internal RAM                                            28

4.   Instruction set    .....................................  33

4.1    Data transfer                                           35

4.2    Arithmetic operations                                   38

4.3    Logic Link                                              41

4.4    Bit manipulations                                       43

4.5    Unconditional jumps and calls                           44

4.6    Conditional jumps                                       47

5.   Assembler   ............................................  49

5.1    The ASM51                                               49

5.2    Segment assignment under ASM51                          51

5.2.1    fixed address assignment                              52

5.2.2    relocatable address assignment                        54

5.3    ASM51 Call                                              54

5.4    Program examples                                        55

5.5    ASM51 bugs                                              57

6.   further components of the 8051 .........................  58

6.1    Interrupt logic                                         58

6.2    Timer0 and Timer1                                       61

6.3    serial communications                                   62

6.3.1    asynchronous, synchronous mode                        63

6.3.2    serial mode set up                                    63

6.3.3    Interrupt Mode, Polled Mode                           67

6.3.4    Baud rate table                                       70

7.   further controller of the 8051 Family                     71

7.1    the 8052                                                71

7.1.1    additional SFR of the 8052                            71

7.1.2    Timer2                                                71

7.1.3    serial communications with timer2                     73

Appendix

A.1  ASCII table     ........................................  75

A.2  Instruction set in hexadecimal order       .............  77

1. 8051 Characteristics                                      Page 1

���������������������������������������������������������������������������

1.  8051 characteristics

-----------------------

The 8051 is often embedded in control circuits, for that reason the name

microcontroller has been commonly used for the 8051.

The 8051 operates on an 8 bit data bus. Externally it can address 64 kByte of

data and code separately. Additionally it has internal RAM (128 Byte) and

internal ROM (4 kByte) of program memory.

Instructions from addresses 0000 to 0FFFh operate from internal program

memory. Instructions from higher addresses will always be read from external

program memory. Through an input pin the internal program memory can be

switched off.

Most of the instructions need one cycle (1 �s) to execute.

The data manipulation instructions are as follows:

MOV                 (data transfer),

XCH                 (data exchange),

INC, DEC            (increment +1, decrement -1),

ANL, ORL, XRL       (logic operations),

ADD, ADDC, SUBB     (addition, subtraction),

MUL, DIV            (multiplication, division).

A special feature of the 8051 is that almost all its electrical connections

(pins) can be accessed with software. It also has access to two integrated

timers, one serial port (UART) as well as two interrupt inputs.

Additionally it is possible to manipulate single BITs with MOV, SETB, CLR and

CPL.

The 8051 has available conditional and unconditional jump instructions.

Subprograms may be called up. The return address will be placed by the CPU

on the stack. The stack is located in internal RAM and increases upwards

when used.

Division can only be done byte wise (8 BIT). For larger data, more complicated

procedures are necessary. The size of the internal RAM is often limiting.

As a cost effective controller, the 8051 enjoys great popularity.

2. Data and Addresses                                        Page 2

���������������������������������������������������������������������������

2. Data and Addresses:

----------------------

Arithmetic only handles numeric values : for example sizes of measurements,

calculation values, etc. As to the handling of text, the ALU will require

a numeric value to be assigned to every character.

Digital techniques work with two logic values (1=high or 0=low).

With one binary digit 2 numeric values can be selected.

With two binary digits 4 different numeric values can be represented:

0 = 00b,  1 = 01b,  2 = 10b and 3 = 11b (the "b" is a label for a binary value).

The alphabet consists of 26 characters. Distinguishing between the capital

and small letters and catering for several special values in addition,

(1 2 3..0 , + - ? ! : ; / ( � � � and so on) so we have approx. 80 values.

To be able to clearly differentiate between them at least 7 binary

digits are necessary.

The assignment of characters to numeric values has naturally been the norm.

In the ASCII standard (American Standard Code for Information

Interchange) 8 binary digits are used to represent numeric values. Such a

numeric value between 0 and 255 are designated as a BYTE. Each of its binary

digits is a BIT.

The choice of 8 bits per BYTE comes also into consideration in the

representation of numeric values for computations, for in hexadecimal

notation, this can be represented by just two digit hex numbers. Larger values

(with more digits) use several bytes.

To support computing, controllers and processors are equipped with 8 bit wide

data lines.  Newer, faster CPU's in future will hold 16 or 32 BIT wide data

lines. With all this, the ASCII table is in addition used for the coding of

text (see Appendix). The following designations are normally used:

BIT       1 data line

NYBBLE    4 data lines     = � BYTE  one digit hex value   0..0F

BYTE      8 data lines               two digit hex value   0..0FF

WORD     16 data lines               four digit hex value  0..0FFFF

DWORD    32 data lines               eight digit hex value 0..0FFFFFFFF

MSB                                                 LSB

�������������������������������������������������������Ŀ

� Bit7 � Bit6 � Bit5 � Bit4 � Bit3 � Bit2 � Bit1 � Bit0 � BYTE

���������������������������������������������������������

|        High NYBBLE        |        Low NYBBLE         |

MSB  =  most significant Bit

LSB  =  least significant Bit

2. Data and Addresses                                        Page 3

���������������������������������������������������������������������������

The 8051 is an 8 bit computer, that is: numeric values, characters and also

the code are stored as bytes. The data bus has 8 parallel data lines.

On chip, the data is transferred over an interconnecting INTERNAL DATA BUS.

If a processor board is constructed and an external RAM and/or EPROM is used,

then 8 connecting lines must be brought in between the 8051 and the external

memory for the data. This is the EXTERNAL DATA BUS.

The  transition  switching between the internal and the external data bus

takes place over special drivers (see ports).

In order to store a number the size of a BYTE, 8 flip flops will be needed.

To store a complete text, many such 8 flip flop memory locations will be

needed. In addition, each memory location should be individually accessible.

For each memory location a select line is needed.

Memory chips produced today are highly integrated (a 62256 RAM chip has

for example 32768 memory locations: it can store 32 kBytes). In order to

access a given memory location, it is not necessary to have 32768 select

lines. Instead, coded addresses are used, which are decoded only in the

RAM chip. So with 16 address lines, which the 8051 prepares, 65536 memory

locations can be addressed.

To select which memory location will be accessed on the 8051 depends

not only on the address, but also on the data type.

Differences will occur between:  SFR, internal RAM, external RAM, internal

ROM, external EPROM

3.1 Block diagram                                             Page 4

���������������������������������������������������������������������������

3.1  8051 Block diagram:

------------------------

The 8051 uses an 8 bit wide data bus. It has several internal registers,

which are likewise 8 bit wide. It can access external components (as memory)

over drivers. It has internal RAM, 2 Timers, serial input/output and internal

program memory.

ڿ

�Ĵ���Ŀ

�  ��  �

����������Ŀ            ��������������������������Ŀ

ALE  �          �            �                          �

_PSEN �Oscillator������������Ĵ  Arithmetic Logic Unit   �Ŀ

�          �       ����>�                          � �

������������       �    ���������������������������� �

�             �            | <Code-Fetch> |     �

�             �            |              |     �           _EA

Timer:     �             �i           |              |     �  �����������Ŀ

�����������Ŀ      �n           |              |     ��>� Program-  �

�   TMOD    �<����͹t           |         �������������ͳ  Counter, �

�����������Ĵ      �e           |         �i   |        � Address-  �

�   TCON    �<����͹r           |         �n   |     ��ͳ  Register �

�����������Ĵ      �n      ����������Ŀ   �t   |     �e �������������

�  TH0:TL0  �<����͹       � internal �   �.   |     �x

�����������Ĵ      �D      �          �   �A   |     �t

�  TH1:TL1  �<����͹A      �   PROM   �<�ͼd   |     �.

�������������      �T      �          �    r   |     �A

�A      ������������        |     �d

�                           |     �r

�                           |     �

�����������Ŀ      �       Port-Latches:       |     �      Driver:

�   ACC     �<����͹           ��������Ŀ      |     �������>�

�����������Ĵ      �����������>�   P0   �<������������������>� Port 0

�   B       �<����͹           ����������            �

�����������Ĵ      �           ��������Ŀ            �������>�

�   PSW     �<����������������>�   P2   �<������������������>� Port 2

�����������Ĵ      �           ��������Ĵ

�   PCON    �<����������������>�   P1   �<������������������>� Port 1

�����������Ĵ      �           ��������Ĵ

�  DPH:DPL  �<����������������>�   P3   �<������������������>� Port 3

�����������Ĵ      �           ����������

�   SP      �<����͹

�������������      �

�

�       ����������Ŀ

serial port:      �       � internal �

�����������Ŀ      �������>�          �

�   SCON    �<����͹       �   RAM    �

�����������Ĵ      �       �          �

�   SBUF    �<����ͼ       ������������

�������������

3.1 Block diagram                                            Page 5

���������������������������������������������������������������������������

The block diagram is a schematic of the internal hardware setup. It is

naturally not precise in all particulars. It views the structure as a

programmer sees it, rather than the actual electrical set up.

As indicated in the block diagram, the CPU can access the SFR (see chapter

3.4) and internal RAM over the internal data bus. For external memory access

(XDATA) ports P0 and P2 are used. Concurrent with the high address byte being

placed in P2, at first the low address byte will be placed in P0. After the

negative edge of the ALE (address latch enable) data will be expected or output.

The instructions (CODE) that the 8051 is supposed to execute, will be read

from the internal ROM or optionally from an external program memory. A read-in

of CODE occurs twice every instruction cycle. The instructions are likewise

placed as numeric values in the size of a BYTES. Therefore a maximum of 256

different instructions are possible. Every number is assigned a definite

instruction: so means e.g. the CODE 0A4h --> multiplies the  Register

ACC with the Register B (for the complete instruction set see chapter 4).

The next diagram shows an example of an external wiring of the 8051 with an

external RAM and an EPROM. Since Port 0, during the first half of an access,

first guides the low address and then for the second half switch

(multiplexer) to data, the low address must be stored in a latch. (There are

however RAMs, which have multiplexed inputs such as the DPR SAE 81C80 from

Siemens, which does not need a latch).

8051                                          RAM

���������Ŀ                                   ��������Ŀ

�      _WR�����������������������������������>�_WR     �

�      _RD�����������������������������������>�_RD     �

�         �                 high Adr.         �        �

�       P2�����������������������������������>�        �<�����ͻ

�         �                           �A      �        �       �

�         �           LATCH           �D      �        �      D�

�         �          �����Ŀ          �D      ����������      A�

�      ALE����������>�74 LS�          �R        EPROM         T�

�         �          � 373 �low Adr.  �E      ��������Ŀ      A�

�       P0�<��������>�     ����������͹S      �        �       �

�         �    �     �     �          �S      �        �       �

�         �    �     �������          �       �        �<�����͹

�         �    �                      �������>�        �       �

�         �    �                       _PSEN  �        �       �

�    _PSEN����ĺ�����������������������������>�_RD     �       �

�����������    �                              ����������       �

�     external DATA                             �

�����������������������������������������������ͼ

3.1 Block diagram                                            Page 6

���������������������������������������������������������������������������

The diagram of the 8051 here             ����������������Ŀ

belongs together with the           P1.0 � 1            40� + 5V

understanding of the hardware       P1.1 � 2            39� P0.0 (AD0)

and the following explanations      P1.2 � 3            38� P0.1 (AD1)

of the individual components of     P1.3 � 4            37� P0.2 (AD2)

the 8051.                           P1.4 � 5            36� P0.3 (AD3)

P1.5 � 6            35� P0.4 (AD4)

P1.6 � 7            34� P0.5 (AD5)

P1.7 � 8            33� P0.6 (AD6)

reset � 9            32� P0.7 (AD7)

(RxD) P3.0 �10            31� _EA

(TxD) P3.1 �11            30� ALE

(_INT0) P3.2 �12            29� _PSEN

(_INT1) P3.3 �13            28� P2.7 (A15)

(T0) P3.4 �14            28� P2.6 (A14)

(T1) P3.5 �15            28� P2.5 (A13)

(_WR) P3.6 �16            28� P2.4 (A12)

(_RD) P3.7 �17            28� P2.3 (A11)

XTAL2 �18            28� P2.2 (A10)

XTAL1 �19            28� P2.1 (A9)

GND �20            28� P2.0 (A8)

������������������

The 8051 runs on +5V. Most of the connections are through the ports. These

port pins can be directly accessed from software. In addition they have extra

hardware functions (indicated above in brackets). Only the RESET pin, the

external/internal code fetch toggle (_EA), the quartz input, the oscillator

output for the address latch as well as the external program memory read

signal cannot be accessed by software.

Summary:

Port0:                                   XTAL1, XTAL2:

8 bit input/output. Alternatively        Connection point for quartz

serves as the output of the low          or an external oscillator.

address and for write/read when

accessing external memory.             ALE:

Address Latch Enable signal to the

memory of the low address of

Port1:                                     Port0 during access to external

8-Bit input/output                       memory.

Port2:                                   _PSEN:

8-Bit input/output.  Alternatively       Program Store Enable (read signal)

serves as the output of the high         during access of external code.

address when accessing external

memory.

reset:

Port3:                                     must be set high for at least 2

8-Bit input/output. Alternately          cycles, then Reset.

P3.0 and P3.1 is also input

/output for serial port, P3.2         _EA:

and P3.3 are also interrupt in-          External Access: when low, all code

puts, P3.4 and P3.5 as timer             will be read externally.

gates, P3.6 and P3.7 as write/read       When high, code from 0000...0FFFh

signals when accessing external data.    will be internally read,  upper

addresses externally.

3.2 Oscillator                                               Page 7

���������������������������������������������������������������������������

3.2  Oscillator / Cycles:

-------------------------

The logic in the 8051 operates on a clock. The oscillator is derived from a

quartz crystal. The maximum usable frequency is 12 MHz (there are already

newer CPUs with 16 MHz). The quartz crystal must oscillate as suggested.

Additionally, an inverting amplifier must be connected in the oscillator

circuit. This driver is integrated into the 8051. In order that the crystal

does not oscillate at higher frequencies, two small capacitors are connected

to it.

Quartz crystal, max. 12 MHz

30pF          ڿ          30pF

���Ĵ�������Ĵ���������Ĵ���Ŀ

GND�        �    ��    �        �GND

���       �          �       ���

�          �

�          �

------o XTAL1----o XTAL2------

�          �

�          �

�   � \    �

���Ĵ   \��������> to internal

� /            oscillator recovery

Wiring with quartz crystal with 8051 HMOS + CMOS

Warning!: 8051 computers use HMOS and CMOS technologies. The CMOS

implementation contains a somewhat differently wired quartz driver.

In an external wiring with a crystal (as shown above) nothing is

changed. However when using an external oscillator clock and this clock

(TTL Level) feeds it, a separate wiring is needed as shown below:

With HMOS execution the external        With CMOS execution the external oscillator

crystal is fed to pin XTAL2. To         is fed to XTAL1. The output XTAL2 of the

deactivate the internal crystal         internal crystal driver remains open.

drivers connect XTAL1 to GND.

������Ŀ                                ������Ŀ

�  ڿ  � external oscillator            �  ڿ  � external oscillator

� ���� ��������������Ŀ                 � ���� ����Ŀ

�  ��  �              �                 �  ��  �    �

��������              �                 ��������    �

��� GND     �                             �

�          �                         ----o XTAL1----o XTAL2---

--------o XTAL1----o XTAL2---                    �          �

�          �                             �   � \    �

HMOS   �          �                     CMOS    ���Ĵ   \���

�   � \    �                             �   � /

���Ĵ   \������> internal                �

� /           oscillator             �������> internal oscillator

3.3 Command execution                                        Page 8

���������������������������������������������������������������������������

The oscillations that come from XTAL2 or XTAL1 or the crystal driver is

divided into 2. After that a symmetrical oscillation of maximum 6 MHz is

available. This clock will be used as the 8051 system clock: that is, all

logic inside the 8051 will change state based on this clock.

The execution of an instruction will need at least 6 system clocks

(State 1 to 6 = 1 cycle).

Crystal:        12 MHz maximum  (some special CPU at 16 MHz also)

System clock:    6 MHz

Cycles:          1 MHz  -->  1 �s

�Ŀ �Ŀ �Ŀ �Ŀ �Ŀ �Ŀ �Ŀ �Ŀ �Ŀ �Ŀ  ڿ �Ŀ��Ŀ �Ŀ ��

XTAL   �� ��� ��� ��� ��� ��� ��� ��� ��� ��� ������� ��  ��� ���

�Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ����

SysClk    �����   �����   �����   �����   �����   �����   �����

State     |   1   |   2   |   3   |   4   |   5   |   6   |   1

^                       ^                       ^

1.Code-Fetch            2.Code-Fetch

Asymmetrical crystal oscillation will still give a symmetrical system clock

(indicated for States 5 and 6 in above diagram).

3.3 Command execution:

---------------------

For every cycle the CPU always allows for 2 bytes from the program memory

(code fetch). This happens automatically at state 1 and state 4 (see also

timing for external code access):

There are instructions without and with 1 to 2 operands. These cycles

generally read 2 code bytes and over fetching will be rejected. With

instructions which need more than 1 cycle to execute the next instruction will

be repeatedly read until the actual statement is reached.

---  Example:

Example:     Opcode:  Cycles:  Fetch:

--------------------------------------------------------------------------

NOP            00         1    00 05         1 byte too much, ignored

INC SP         05 81      1    05 81

INC DPTR       A3         2    A3 E2 E2 83   in the 2nd cycle next fetch

MOVX A,@R0     E2         2    E2 83 __ __   in 2nd cycle XDATA no fetch

MOVC A,@A+PC   83         2    83 02 __ 02   in the 2nd cycle MOVC access

LJMP 3344      02 33 44   2    02 33 44 ??

3.3 Command execution                                        Page 9

���������������������������������������������������������������������������

With a NOP a code byte too many will be read. It will not be evaluated,

rather re-read in the next cycle. With INC DPTR in first cycle 1 code byte

too many will be read. In the 2nd cycle the code byte of the next instruction

will again be read but also not be evaluated yet.

With an external data memory access (MOVX) there is a special feature. Here

in the 2nd cycle, no code fetch takes place, rather the external RAM will be

accessed.

Accessing a table in program memory through MOVC in the 1st cycle, a byte of

the next instruction will be read along already (too much). In the 2nd cycle,

the access to the table takes place first and then yet another code fetch,

which will however be rejected.

An LJMP instruction needs 3 code bytes and 2 cycles. In the 1st cycle the

opcode and the high byte of the target address (1st operand) will be read. In

the 2nd cycle the low byte of the target address (2nd operand) and (too much)

the opcode of the next code byte will be read. After carrying out the jump,

the program counter will be at the new address. The next code fetch takes

place there. From here is also apparent, why the over-read instructions in

the previous cycles are repeatedly rejected.

The execution of instructions takes place mainly in state 5. There the code

address for the code fetch of the next cycle will again also be already

set.

3.4 SFR                                                      Page 10

���������������������������������������������������������������������������

3.4  SFR (Special Function Register):

-------------------------------------

Contrary to some processors, no special instructions are used to control timer

and serial ports. This control is done with special registers, which are assigned

in the data region of the 8051. Every SFR (special function register) is assigned

a DATA address, the access takes place over the internal data bus.

With enhanced controllers such as the 8052, 80515, 80552 and others, in

comparison to the 8051, additional functions come with extra SFRs. However, they come

with the same instruction set!

Instead of the DATA address, a predefined abbreviation (Mnemonic) can be used

in the case of the ASM51 assembler. The following table holds all SFR of the

8051 with mnemonic and DATA Address:

Ports:                                   Timer:

P0   DATA   80h                          TCON DATA  88h

P1   DATA   90h                          TMOD DATA  89h

P2   DATA  0A0h                          TL0  DATA  8Ah

P3   DATA  0B0h                          TL1  DATA  8Bh

TH0  DATA  8Ch

Arithmetic registers and CPU control:    TH1  DATA  8Dh

PSW  DATA  0D0h

ACC  DATA  0E0h                         Interrupt control:

B    DATA  0F0h                          IE   DATA  0A8h

PCON DATA   87h                          IP   DATA  0B8h

Pointer:                                 serial port:

SP   DATA  81h                           SCON DATA  98h

DPL  DATA  82h                           SBUF DATA  99h

DPH  DATA  83h

Some SFR are bit addressable (always those whose DATA Addresses

end with 0 or 8). For an 8051 these are:

PSW - Bit's:                IE - Bit's                  IP - Bit's

CY   BIT  0D7h              EA   BIT  0AFh              PS   BIT  0BCh

AC   BIT  0D6h              ES   BIT  0ACh              PT1  BIT  0BBh

F0   BIT  0D5h              ET1  BIT  0ABh              PX1  BIT  0BAh

RS1  BIT  0D4h              EX1  BIT  0AAh              PT0  BIT  0B9h

RS0  BIT  0D3h              ET0  BIT  0A9h              PX0  BIT  0B8h

OV   BIT  0D2h              EX0  BIT  0A8h

P    BIT  0D0h                                        SCON - Bit's

P3 - Bit's                    SM0  BIT  9Fh

TCON - Bit's                  RD   BIT  0B7h              SM1  BIT  9Eh

TF1  BIT  8Fh               WR   BIT  0B6h              SM2  BIT  9Dh

TR1  BIT  8Eh               T1   BIT  0B5h              REN  BIT  9Ch

TF0  BIT  8Dh               T0   BIT  0B4h              TB8  BIT  9Bh

TR0  BIT  8Ch               INT1 BIT  0B3h              RB8  BIT  9Ah

IE1  BIT  8Bh               INT0 BIT  0B2h              TI   BIT  99h

IT1  BIT  8Ah               TXD  BIT  0B1h              RI   BIT  98h

IE0  BIT  89h               RXD  BIT  0B0h

IT0  BIT  88h

3.4 SFR                                                      Page 11

���������������������������������������������������������������������������

The SFRs are accessed directly. The have in code stand the HEX address of the

SFR as operand. Example for an SFR access:

MOV    TMOD, #20h       ; a value to be written to TMOD

MOV    89h, #20h        ; the same with DATA addr

ORL    PSW,  #18h       ; change the value in PSW

MOV    A, SBUF          ; load the value from SBUF into accumulator

In this chapter the general CPU registers are explained. The registers which

are assigned to unique components (such as Ports, Timer, Interrupt, serial

port) will be explained in the context of these functions.

3.4.1  ACC (Accumulator):

-------------------------

The accumulator (DATA E0h) serves as the main calculation register.

Arithmetic operations can only be effected in connection with the ACC.

Moreover the ACC is used to access external memory and indirect jumps (see

instructions ADD, ADDC, SUBB, MUL, DIV, MOVX, MOVC, JMP, JZ, SWAP, CLR,...).

The ACC can be addressed bitwise:

ACC.0  BIT  0E0h                 ACC.4  BIT  0E4h

ACC.1  BIT  0E1h                 ACC.5  BIT  0E5h

ACC.2  BIT  0E2h                 ACC.6  BIT  0E6h

ACC.3  BIT  0E3h                 ACC.7  BIT  0E7h

When accessing the ACC, special (shorter and faster) instructions are

available. To make the distinction, the ASM51 mnemonic abbreviation "A" is

written instead of "ACC". For the 8051 different opcodes are generated.

Example:

MOV ACC, #44h           Hex code: 75 E0 44       2 cycles

or   MOV A, #44h             Hex code: 74 44          1 cycle

3.4.2  B-Register:

------------------

The B-Register (DATA F0h) serves as a general purpose register. It can be used

as intermediate storage for values. In multiplication and division it is used

as an assisting calculation register. It can be treated bitwise:

B.0  BIT  0F0h                    B.4  BIT  0F4h

B.1  BIT  0F1h                    B.5  BIT  0F5h

B.2  BIT  0F2h                    B.6  BIT  0F6h

B.3  BIT  0F3h                    B.7  BIT  0F7h

3.4 SFR                                                      Page 12

���������������������������������������������������������������������������

3.4.3  PSW (Program Status Word):

----------------------------------

The PSW (DATA D0h) holds the most important flags of the CPU as well as two

bits for register bank switching (see 3.9 internal RAM). The flags are

important for computation operations and comparisons:

P      BIT  0D0h                  RS1   BIT  0D4h

PSW.1  BIT  0D1h                  F0    BIT  0D5h

OV     BIT  0D2h                  AC    BIT  0D6h

RS0    BIT  0D3h                  CY    BIT  0D7h

�����������������������������������������������Ŀ

� CY  � AC  � F0  � RS1 � RS0 � OV  �PSW.1�  P  �   PSW    (D0h)

�������������������������������������������������

�������Ŀ     ����Ŀ

� PSW.1 � and � F0 � have no hardware function. They can be used by the

���������     ������ user for any purpose. PSW.5 is predefined by

ASM51 as F0 (Flag 0). PSW.1 and PSW.5 can be used as

intermediate storage for any bit value.

���Ŀ

� P �  is the parity flag. It always reflects the status of the ACC.

�����  If the sum of all bits in the ACC is even (even Parity), then P = 0.

If the sum is odd (odd Parity), then P = 1. The parity flag cannot

be written by software (only read).

Example:  ACC = 11h = 00010001b --> � = 2, even   --> P = 0

ACC = E3h = 11100011b --> � = 5, odd    --> P = 1

����Ŀ

� OV �  is the overflow flag (overflow flag). It is set, after a

������  multiplication, when the result is larger than a byte (then it

no longer fits in the ACC alone). It will be set when division by 0

occurs. In addition, it will be set during additions and subtractions when

there is a sign error.

In computations with signed numbers, the highest bit represents the sign

(example: 31 = 1Fh, -31 = 9Fh). So only numbers from -128 to +127 are

represented. Now when a positive number is added to a positive number, the

result must (again) be positive. A negative number minus a negative number

must (again) be negative. With an overflow, this is not the case:

Example:  42h + 47h = 89h = -9      --> Error! true result +137

����Ŀ

� AC �  is the auxiliary carry flag (auxiliary overflow flag): it will be set

������  when there is a carry over from bit 3 during addition and subtraction.

3.4 SFR                                                      Page 13

���������������������������������������������������������������������������

����Ŀ

� CY �  is the carry flag (carry flag): it will be set on a carry from

������  bit 7 during addition, subtraction, by decimal adjust and on

rotate left through carry. Besides it will be set on a carry

from bit 0 by rotate right through carry and when the left operand is less

than the right during "Compare and Jump if Not Equal".

As in the ACC, there are special instructions also for accessing the carry

flag. These are characterised by the use of the mnemonic "C" which stands for

"CY".

Example:     SETB CY            hex code:  D2 D7          1 cycle

SETB C             hex code:  D3             1 cycle

�����Ŀ     �����Ŀ

� RS1 � and � RS0 �  serve to select the register bank. These bits are

�������     �������  set by software (only read through the CPU).

RS1:RS0 = 00 -->  Reg Bank 0          RS1:RS0 = 10 -->  Reg Bank 2

RS1:RS0 = 01 -->  Reg Bank 1          RS1:RS0 = 11 -->  Reg Bank 3

3.4.4  SP (Stack Pointer):

--------------------------

The CPU sets up a stack memory in internal RAM. The CPU will place the

return address in the stack with every subprogram call (LCALL or ACALL).

The same takes place automatically through the CPU when entering an interrupt

routine. A RET or RETI will fetch the return addresses from the stack. An

application program can access stack memory with the PUSH and POP

instructions.

The SP (DATA 81h) always shows the final pushed value. To store a byte

the SP will first increment and then the value will be written into internal

RAM address, where the SP points to. Return addresses are of type WORD, with

which the CPU will first put the low byte then the high byte to the stack.

The stack grows from the bottom to the top. After a reset the SP shows the

internal RAM address 07h. The first pushed byte will be written in 08h

(with that reg bank 0 will not be overwritten by the stack). An application

program should soon after a reset load the SP with the program's last used

internal RAM address, so that the stack will be placed above the program's

data.

Warning! The 8051 internal RAM ends at address 7Fh. Should the SP show upper

address values, an additional PUSH will increment it- with a POP garbage

comes back!

3.4 SFR                                                      Page 14

���������������������������������������������������������������������������

3.4.5  DPL, DPH (Data pointer):

-------------------------------

The data pointer (DPTR) is the sole 16 bit register of the 8051, which can

also be loaded for 16 bit access. It is put together from the Special Function

Registers DPH = data pointer high (DATA 83h) and DPL = data pointer low (DATA

82h). The data pointer serves to access external RAM (see MOVX), tables in

code memory (see MOVC) and for indirect jumps (see JMP).

3.4.6  PCON (Power-Control):

----------------------------

The PCON register (DATA 87h) hold further CPU flags. The power control

flags exist only in the CMOS versions (80C51). The most significant

bit (SMOD) belongs to the setting of the baud rate of the serial port

(see serial port). PCON is not addressable bitwise.

�����������������������������������������������Ŀ

�SMOD �  -  �  -  �  -  � GF1 � GF0 � PD  � IDL �   PCON    (87h)

�������������������������������������������������

������Ŀ

� SMOD �  determines the gain factor for the driving oscillator of the serial

��������  port. For serial modes 1, 2 or 3 the baud rate (Shift Time) will be

divided by 2, when SMOD = 0.

�����Ŀ     �����Ŀ

� GF1 � and � GF0 �  have no hardware functions. They cannot be manipulated by

�������     �������  BIT Addresses (set and clear only with ORL respectively

PCON). They can be used from the program for any purpose.

A proposed use is for indicating whether idle mode has been activated.

�����Ŀ

� IDL �  switches the CPU into idle mode (only CMOS!). When IDL = 1 is set

�������  (with OR PCON, #1), the next instruction will not be executed.

The 8051 goes into IDLE mode. This means:

*  Instruction execution stops.

*  the port output pins keep their last electrical value

*  the timer continues to run

*  the serial port continues to operate

*  all other registers keep their values

*  the supply current is reduced by about 23% (at 5V and 12 MHz)

The idle mode can be discontinued by activating an interrupt or by a reset.

The interrupt must also be enabled. If an interrupt is activated, the entry

into the interrupt routine will clear IDL. The interrupt routine will

execute. After the RETI instruction execution will continue at the

3.4 SFR                                                      Page 15

���������������������������������������������������������������������������

instruction following the idle mode activation. GF1 or GF0 may serve to

indicate idle mode was set, by the code for the interrupt routine (IDL will

be cleared by hardware).

If a reset occurs during the idle mode, then the IDL in PCON will be cleared

immediately. The execution of instructions continues with subsequent

instructions --> until two cycles later the reset logic starts up and all SFRs

as well as the program counter are set back. Execution starts again at the

reset address 0000.

First of all the two or three instructions following the idle mode will be

executed. The correct execution of the instruction in this condition is not

guaranteed. Accessing the ports or external data memory should therefore

be avoided after idle mode activation (possibly insert 3 NOP's).

Access to the internal RAM is not a problem, for the hardware prevents access

to the internal RAM at this time.

After leaving the idle mode upon reset, all SFRs are set back. Also GF1 and

GF0 are cleared. But the internal and external RAM are unchanged.

����Ŀ

� PD �  switches the CPU into power down mode (only CMOS!). If PD = 1

������  is set (via OR PCON, #2), then the next instruction will not be

executed. The 8051 goes into POWER DOWN Mode. This means:

*  the instruction execution stops.

*  the port output pins keep their last electrical value

*  ALE and _PSEN goes low

*  the timers stop

*  the serial port stops

*  the supply current reduces itself to about 0.3% (at 5V and 12 MHz)

The power down mode can only be exited through a reset. This reset  will then

reset all SFRs (also ports to 0FFh), but not the internal and external RAM.

During the power down state the supply voltage can be reduced to 2V without

the contents of the internal RAMs being lost.

To do this successfully, first PD is activated, and the voltage must go back

to 5V, before the reset is released.

3.5 Ports                                                    Page 16

���������������������������������������������������������������������������

3.5  Ports:

-----------

Every 8 electrical connections of the 8051 are connected to a port. There

is direct access to the port pins from the software. The connections are also

used for other functions, which can only be controlled from the hardware,

such as for the code fetch from an external program memory.

In order that the ports may be used for input and output, they have a special

driver:

�             /�read pin

�       �����< �����������������������������������Ŀ

�       �     \�                                   �

i�       �                                          �

n�       �     /�read latch              +5V o      �

t�\���<�������< ����Ŀ                       �      �

e�             \�    �                     ���      �

r�                   �               �����Ĵڿ      �

n�                   �               �       �      �  Pin0.x

a�          �����Ŀ  �               �       �������������o

l�\����>���ĴD   Q����    switch     �       �

�          �     �      �����Ŀ     �     ���

D�          �   /Q������Ĵ  _\ �����������Ĵڿ

a�          �������      �������     �       �

t�           Latch          �        �       �

a�                          �        �

�                    Address/Data output

The above picture shows a simplified setup of a pin of Port 0. Port 2

has in principle the same setup, except that there is an extra pull up

resistor at the output.

alternative function of P0, P2:

--------------------------------

Port 0 and Port 2 alternatively serve to read from the external program

memory (code fetch) and to read from and write to the external data memory.

The low byte of the addresses are given out over Port 0 and the data read or

written. Port 2 functions as output for the high byte of the address.

If code is executed from the internal ROM only and no external data memory

is used (no MOVX instruction), then the alternative functions of port 0 and

port 2 will never be activated. Port 0 and Port 2 can then be dedicated to

any function (in/output pins). At Port 0, pull up resistances will then be

required.

If port 0 and port 2 are used for code fetches, then the other functions

will mostly be shut out. Then the hardware automatically clears the contents

of the port 0 latches with every external data access.

With MOVX @Ri instructions, port 2 will not be controlled by the hardware

with high addresses.

3.5 Ports                                                    Page 17

���������������������������������������������������������������������������

Writing to port P0 and P2:

--------------------------

In the "normal" function a byte written into the port address (P0 DATA 80h,

P2 DATA A0h) over the internal data bus will be stored in the latches.

A switch changes between the "normal" function and the alternative function,

regardless of when the alternative function value has given out values. The

content of the latch stays unchanged (Exception: Read from P0 for Code-Fetch,

MOVX and MOVC).

The upper transistor will only take control, when a logic 1 for the alternate

function happens to be placed on the pin. If there is a 1 in the latch, and

no value is given for the alternate function, then the output at Port 0 is at

high impedance (floating). At port 2, the integrated pull up takes effect. In

both cases, the pin can be used as input. If there is a 0 in the latch, then

the lower transistor will be activated, and the pin will always be low (not

available as input).

Read from Port P0 and P2:

-------------------------

At Port 2, no reading of data for the alternate function of the memory access

takes place (issue only high address). At Port 0 data will be read over a

tristate driver to the internal data bus. Before every read access of external

memory, the hardware automatically writes #0FFh in the Port P0 - latches (only

with alternative functions). With "normal" read accesses, the user himself

must see to it that the latch outputs are high, since only then can a read be

properly performed over a port pin (this condition is satisfied after a

Reset).

If through a software instruction a "normal" read from a port is done through

an SFR address, then this occurs after the instruction, from the pins directly

over a driver or over another tristate driver from the latch output. With all

read-modify-write accesses the reading is done from the latch, otherwise from

the pin.

An example:

CPL P2.1 is a read-modify-write instruction: The value in the latch from pin

P2.1 is to be changed. If there is a 0 inside, then a 1 is to be written in

and vice versa.

If read from a pin, a mistake can      o    |         <--- Load

occur when the circuit is switched on      �    |         ������o

as in the example given here.             ��   |         �

The base of the NPN transistor is         � �   |         �

driven directly from the pin - made       ��� P2.1       /

possible by the internal pull up.          �����o�����Ĵ<    NPN

With the latch set, the voltage at         �    |        \E

the pin will only be 0.6 V. From the     ���    |         �

pin, a low level will be read back,    �Ĵڿ    |         �

even though there is a 1 in the latch.     �    |        ���

The CPL instruction will be wrongly       ���   |

executed.

Read-modify-write instructions are:

JBC           jump when bit (in latch) is set and clear latch

DJNZ          decrement port (SFR of the ports) and jump when not 0

CPL           change (Port-) Bit

ANL, ORL, XRL, INC, DEC change (Port-) Byte

3.5 Ports                                                    Page 18

���������������������������������������������������������������������������

to the Distinction:

JB   (jump, when bit is set) is not a read modify write instruction, since

the contents of the latch here is not used. Here the pin will be read.

Problems may arise with the following instruction sequence:

PUSH P2            PUSH P2 is not a read modify write instruction, and for

MOV P2, #80h       that reason the value of the port pin's will be placed on

MOV R0, #0         the stack. This can be the contents of the latch, but

MOVX A, @R0        that need not be so (see circuit on previous page).

POP P2             The contents of the latch may only be reliably placed on

the stack, when no port pin is turned on as input, and

the load on the output pins is such that the electric level is not wrong.

Differences between Port P1 and P3 to P0:

-----------------------------------------

P1 and P3 are different from port P0 in that they have no multiplexer. As far

as we are concerned, the output for the alternative function connects with the

latch output over a NAND gate. Input will not go to the internal data bus,

but directly to the components of the alternative functions.

�             /�Pin read

�       �����< �����������������������������Ŀ

�       �     \�                             �

i�       �                                    �

n�       �     /�Latch read        +5V o      �

t�\���<�������< ����Ŀ                 �      �

e�             \�    �                ��     �

r�                   �                � �     �

n�          �����Ŀ  �                ���     �  Pin 3.x

a�\����>���ĴD   Q��Ĵ     NAND        �������������o

l�          �     �  �    ����Ŀ     ���      �  Pin 1.x

�          �   /Q�  ����Ĵ &  �o���Ĵڿ      �

D�          �������   ���Ĵ    �       �      �

A�           Latch    �   ������       �      �

T�                    �                       �

A�                    �                 /�    �

�                    �           �����< ������

�           �     \�

alternative:         �

output       input

In regard to the "normal" access to a port address (SFR) through the software

there is no difference from port P0 (see Comments about P0).

P1 and P3 possess integral pull up resistors. The upper transistor drawn into

the diagram for P0 does not exist. However there is a special feature: On the

chip of the 8051 the pull up resistors are not achieved through permanently

fixed resistors rather through special field effect transistors. If a pin is

switched from 0 to 1, port P1 and P3 will, for the duration of 2 crystal

clocks (= 1/6 cycle) switch the resistance of the

3.5 Ports                                                    Page 19

���������������������������������������������������������������������������

FET, so that a low impedance is imposed (at port 2 this will be for the entire

period of the alternate function of the high address output). Thereby, faster

settling time of the output signal of a pin is achieved, and yet the pull up

is of sufficiently high impedance so that the pin can be used for input.

With port P3, no alternative output takes place, so the input of the NAND

gate is high. For this case the logic condition of the pins depends on the

latch.

Should the pin be used for input, in the same way as with P0, #0FFh is written

to the port latches from P1 or P3. However this must take place if alternate

output functions are to be used, since contrary to Port P0 and P2, no switch

exists with P1 and P3.

3.6 external program memory                                 Page 20

���������������������������������������������������������������������������

Alternate functions of the 8051:

---------------------------------

Port 0:       Address (low Byte) and data for external memory access.

Port 2:       Address (high Byte) for external memory access.

P3.7 = _RD:   Read signal for external data memory access

P3.6 = _WR:   Write signal for external data memory access

P3.5 =  T1:   Timer 1 Gate

P3.4 =  T0:   Timer 0 Gate

P3.3 = _INT1  external interrupt input 1

P3.2 = _INT0  external interrupt input 0

P3.1 =  TxD   Send output from the serial port

P3.0 =  RxD   Receive input from the serial port

electrical characteristics of the ports:

----------------------------------------

Isink  =  2,4 mA             Port 0        (UoL < 0,45 V guaranteed)

Isink  =  1,6 mA             Port 1,2,3    (UoL < 0,45 V guaranteed)

Pull-Up   10...30 k�         Port 1,2,3

Idrive =  0,4 mA             Port 0        (UoH > 2,4 V  guaranteed)

Important: with all port pins there must be a 1 in the latch, when the pin

is to be used for input!

3.6 Access of the external program memory:

-----------------------------------------

The accessing of external program memory takes place automatically 2 times

every cycle through the hardware (see 3.3 instruction execution). The 8051

has an internal ROM and an input pin _EA (External Access).

_EA = 1 :  The instructions of code addresses 0000-0FFFh are read internally.

The instructions of code addresses 1000-FFFFh are read externally.

(8052 has 8 kByte internal ROM: addresses 0000-1FFFh)

_EA = 0 :  All the instructions of code addresses are read externally.

The _EA-Pin serves to turn off the internal ROM. _EA cannot be read like the

port pins from the software.

The 8031 has no internal ROM. Here all instructions are read externally.

It is nonetheless recommended that the _EA pin (since it is available with

the 8031 as it has a similar package construction) be put to GND. In the case

of the 8344 (the ROMless version of the 8044) I was already able to note that

it does not function correctly when _EA = 1. It seemed as though it had an

internal ROM. Perhaps it is more convenient for the manufacturer to make one

mask and then print the chips separately.

3.6 external program memory                                 Page 21

���������������������������������������������������������������������������

_PSEN (Program Store Enable serves as a read signal for the external program

memory. Similarly, the _PSEN cannot be accessed via software.

external program memory read:

-----------------------------

(compare also with 3.3 instruction execution and 3.5 ports):

-- previous ------->|<----------------  this cycle  --------------->|

|       |       |               |       |       |

���Ŀ   ����Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ

SysClk   ��   �����    �����   �����   �����   �����   �����   �����   �

|       |       |       |       |       |       |

State     5   |   6    |   1   |   2   |   3   |   4   |   5   |   6   |

|   ^1.Fetch    |       |   ^2.Fetch    |       |

Ŀ            |   �������Ŀ   |       |   �������Ŀ   |       |

ALE       ������������������   |   �����������������   |   �������������

����Ŀ        |   �����������Ŀ       |   �����������Ŀ       |

_PSEN          ��������������   |       �������������   |       ���������

|       |       |       |       |       |       |

P2       CCCCCCCCCCCCCCCCCCCCC-CCCCCCCCCCCCCCCCCCCCCCC-CCCCCCCCCCCCCCCCC

|       |       |       |       |       |       |

P0cpuOut   AAAAA---------?????---AAAAAAAA--------?????---AAAAAAAA---------

P0epromOut  -----DDDDDDDDDDDDD------------DDDDDDDDDDDD-------------------D

CCC = code address

DDD = data out

?   = data read

--- = high impedance

The code fetch access begins already in state 5 of the previous cycle with the

application of address. In the middle of state 5 the ALE goes low. For

external access ALE serves as a signal for the address latch to take over the

low address from P0 to store them (see 3.1 ff).

With the end of state 5 the _PSEN goes low and simultaneously P0 goes high

impedance (as input: indicated with a ---). The associated EPROM now places

data to the external data bus (as long as _PSEN = 0).

From the start of state 1 the P0 in 8051 expects data (indicated by the ???,

in addition high impedance). These lie already at the output from the EPROM.

In the middle of state 1 the data will be transferred over P0 to the internal

data bus and _PSEN and ALE will both again be high.

Towards the end of state 1 the (incremented) address for the next code fetch

will be placed.

The above timing diagram shows the ideal execution of both code fetch accesses

in a cycle. Actually all transfers are affected by the switching times.

For correct functioning of the 8051 the observance of the following

timings are required :

- at the latest tPLIV after _PSEN is low, the program memory must put valid

data to the bus.

3.6 external program memory                                 Page 22

���������������������������������������������������������������������������

- at the latest tAVIV after valid addresses from the 8051 have been given,

the program memory must put valid data to the bus.

The time value depends on the 8051 type (manufacturer). Exact values are to

be taken from the data sheet. Typical valid values (with T = 1/fOscillation):

tPLIV = (3 * T) - 105 ns .... (3 * T) - 150 ns    > 100 ns (at 12 MHz)

tAVIV = (5 * T) - 105 ns .... (5 * T) - 150 ns    > 267 ns (at 12 MHz)

For program memory often EPROMs and PROMs are used. A typical EPROM indicated

as 27256-250 means that the EPROM has 256 kBIT (= 32 kByte) in terms of

memory cells with an address access time of 250 ns.

Indicated is the address access time: the time from the issue of the address

until valid data is put on the output. The chip enable access time is about

the same (time of application of an active chip select signal until valid

data). The output enable access time (time of application of an active _PSEN

until valid data) is significantly smaller (range in the order of 20 ns to 60

ns). The critical time for the use as program memory of 8051 is the chip

access time (= address access time). If these requirements are fulfilled,

then the other times are also ok.

The input memory must have an address access time of 267 ns

(at 12 MHz CPU crystal). If the chip select is formed with the help of a gate

from the address, the program memory at the gate must have a faster

propagation time.

A GAL 16V8-25 has a propagation time of 25 ns. It may be used with a program

memory with an access time of less than 242 ns. The indicated time tAVIV

(address valid instruction valid) is always important for the input CPU type

and the usable CPU crystal in the data sheets plus the gate propagation time

of the chip select logic.

With a CPU crystal of 12 MHz, one is always safe with EPROMs with 200 ns. For

many uses, 250 ns is enough.

With a MOVC instruction, the read access takes place at the program memory

through the software in the second cycle instead of the 1st code fetch.

3.7 external data memory                                     Page 23

���������������������������������������������������������������������������

3.7  Accessing external data memory:

------------------------------------

The access of the external data memory takes place in the second cycle of the

execution of the MOVX instruction (see 3.3 instruction execution). That's why

in this cycle there is no code fetch (the only the case where ALE and _PSEN

do not appear). An XDATA access lasts 6 system clocks (twice as long as a

CODE access).

_RD (Port P3.7 pin) serves as a read signal for the external data memory.

_WR (Port P3.6 pin) server as a write signal.

When MOVX instructions are used, these bits in Port 3 - Latch are to be

set (this occurs after a reset).

XDATA read:

------------

-- previous ------->|<----------------- this cycle ---------------->|

|       |       |               |       |       |

���Ŀ   ����Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ

SysClk   ��   �����    �����   �����   �����   �����   �����   �����   �

|       |       |       |       |       |       |

State     5   |   6    |   1   |   2   |   3   |   4   |   5   |   6   |

Ŀ            |       |       |       |   �������Ŀ   |       |

ALE       ������������������������������������������   |   �������������

|       |       |       |       |       |       |

�������������Ŀ       |       |       �������������������������

_RD                    �������������������������       |       |       |

|       |       |       |       |       |       |

P2       XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX_CCCCCCCCCCCCCCCCC

|       |       |       |       |       |       |

P0cpuOut   XXXXXXXXXX--------------------?????-----------CCCCCCCCC--------

P0ramOut   ---------------DDDDDDDDDDDDDDDDDDDDDDDD------------------------

XXX = XDATA address

(also compare with 3.6 program memory access)

The XDATA address will already be placed at state 5 of the first cycle of the

MOVX instruction. With MOVX A,@DPTR: at P2 the high address (DPH) and at P0

the low address (DPL). With MOVX A,@Ri: the value from the latch (SFR of P2)

remains at P2 and the address (from Ri) will be placed at P0.

In the middle of state 5 ALE will go low. Consequently, the addresses from P0

can be transferred into an (external) address latch (see 3.1 ff). About the

end of State 6, P0 will go to high impedance (address taken), following which

_RD becomes low. A connected RAM now lays data on the external data bus

(_RD = 0).

From the beginning of state 3, the 8051 expects data at P0 (indicated by the

???). In the middle of state 3, the data will be moved over onto the internal

data bus, following which _RD and ALE will again be high.

Towards the end of state 4 the address for the code fetch of the next cycle

will be placed if a code fetch takes place externally (if internally, then P0

and P2 remain at high impedance - see 3.6 and 3.3).

3.7 external data memory                                     Page 24

���������������������������������������������������������������������������

An access of the external data memory lasts longer than a code fetch. The ALE

is once not given out. For the access longer times are available:

- at the latest tRLDV after _RD becomes low, the external data memory must

place valid data on the bus.

- at the latest tAVDV after valid addresses from the 8051 have been given,

valid data from the external data memory must be put on the bus.

Generally it is something like (with T = 1/fOsc):

tRLDV = (5 * T) - 105 ns .... (5 * T) - 165 ns    > 251 ns (at 12 MHz)

tAVDV = (9 * T) - 105 ns .... (9 * T) - 165 ns    > 585 ns (at 12 MHz)

XDATA write:

------------

-- previous ------->|<-------------- this cycle ------------------->|

|       |       |               |       |       |

���Ŀ   ����Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ   ���Ŀ

SysClk   ��   �����    �����   �����   �����   �����   �����   �����   �

|       |       |       |       |       |       |

State     5   |   6    |   1   |   2   |   3   |   4   |   5   |   6   |

Ŀ            |       |       |       |   �������Ŀ   |       |

ALE       ������������������������������������������   |   �������������

|       |       |       |       |       |       |

�������������Ŀ       |       |       �������������������������

_WR                    �������������������������       |       |       |

|       |       |       |       |       |       |

P2       XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX_CCCCCCCCCCCCCCCCC

|       |       |       |       |       |       |

P0cpuOut   XXXXXXXXX-DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD----CCCCCCCCC--------

P0ramOut   --------------------------------------?------------------------

XXX = XDATA address

(compare also with 3.6 program memory access)

The XDATA address will already be placed at state 5 of the first cycle of the

MOVX instruction. With MOVX A,@DPTR: at P2 the high address (DPH) and at P0

the low address (DPL). With MOVX A,@Ri: at P2 the value from the latch

(SFR of P2) and at P0 the address (from Ri).

In the middle of state 5 ALE will go low. Consequently, the addresses from P0

can be transferred into an (external) address latch.

Towards the end of state 6 at P0, the address will be taken away and data be

placed, thereafter the _WR goes low.

At the end of state 3 _WR will again be high. With it the external data

memory takes over the data from P0. Thereafter, P0 will again be at high

impedance and ALE again high.

At the end of state 4 the address for the code fetch of the next cycle will

be placed if the code fetch takes place externally (if internally, then P0

and P2 remains at high impedance - see 3.6 and 3.3).

3.8 external connection, example                           Page 25

���������������������������������������������������������������������������

- The 8051 guarantees, that at least tQVWH before _WR is high, valid

data lies on the bus.

- the 8051 guarantees, that at least tWLWH + tAVWL before _WR is high,

a valid address has been given out.

Generally, the following are true (with T = 1/fOsc):

tQVWH = (7 * T) - 105 ns .... (7 * T) - 150 ns    > 433 ns (at 12 MHz)

tWLWH = (6 * T) - 100 ns                          = 400 ns (at 12 MHz)

tAVWL = (4 * T) - 70 ns ..... (4 * T) - 130 ns    >  96 ns (at 12 MHz)

For the data memory there exists therefore the requirements of address access

time of 585 ns for reading and 496 ns for writing (at 12 MHz CPU clock). Most

of the types of the series 6264 ...  62256 (static RAMs) have address access

time under 150 ns, so that here no problem should occur.

3.8  Connection for the external data bus (example):

----------------------------------------------------

The address space for CODE and XDATA is separate. The electrical distinction

is achieved through read signals _PSEN (for CODE) and _RD (for XDATA).

A typical connection with EPROM and RAM has already been shown in 3.1. Now we

have here a connection of the overall address space with 64 kByte RAM and 64

kByte EPROM:

�������Ŀ

��������������_WR��>�       �      ���Ŀ

� ������������_RD��>� RAM   �<����o�INV�Ŀ

� �                 �       �      ����� �

� �       ���������>� 32kx8 �<��ͻ       �

8051 CPU          � �       �         ���������    �       �

��������Ŀ         � �       �         �������Ŀ    �       �

�     _WR�������������������ĺ���_WR��>�       �    �       �

�     _RD�������������������ĺ���_RD��>� RAM   �<��ĺ���������� A15

�        �                   �         �       �    �

�      P2��������ADDRESSBUS�����������>� 32kx8 �<��͹

�        �          �����Ŀ  �         ���������    �

�     ALE����������>�LATCH�  �         �������Ŀ    �

�      P0�<��������>�     ��͹         �       �    �

�        �    �     �������  ���������>� EPROM �<��͹

�        �    �                        �       �    �

�   _PSEN����ĺ����������������PSEN���>� 64kx8 �<Ŀ �

����������    �                        ���������  � �

����DATABUS��������������������������ͼ

Since static RAMs are available only up to 32kx8, two RAM chips are needed.

The 1st RAM receives as chip select the address A15 and the second RAM the

inverted value of A15. As CODE memory, an EPROM 64kx8 is used. Its chip

enable pin can be put at GND (always selected).

3.8 external connection: example                              Page 26

���������������������������������������������������������������������������

The 1st RAM is active at all addresses, at which A15 = 0, so with

0000...7FFFh. The 2nd RAM becomes active with 8000...FFFFh (A15 = 1).

In hardly any circuitry will such a large external data memory be needed.

Therefore most use only one RAM chip. Often a yet smaller RAM is used

(cheaper). Recommended are the types 6264 (8kx8), 62128 (16kx8) and 62256

(32kx8). Smaller types than 8kx8 can admittedly be used, however they are

more expensive.

If only a RAM is inserted in XDATA space, then its chip enable pin can be

placed at GND (always selected). The same memory cell in the RAMs can be

accessed over different XDATA addresses (multiply selected). This is not a

problem when the software is ok.

--- IO Region ------------------------------------------------------

The XDATA region, besides being used for external data memory, can be used

for other purposes. Some examples:

- for the reading of jumper settings or other data through a tristate driver.

- as universal port (8 bit input/output) over bidirectional tristate

drivers (74LS245).

- for connecting special components such as to external analog/digital

converters, to a SCC (serial communication controller) for a second serial

port, to a display module and others.

- as ports to other processors through 8 bit register (realised by the Intel

8044 BITBUS processor card) or over DPR (Dual Port RAM).

(-- see example in Appendix)

With IO functions installed, one or several XDATA addresses must be

reserved. In general an address decoder is then needed.

Often the RAM will be placed in the region from 0000...7FFFh (A15 as Chip

Select) and the IO Region above 8000h.

--- load driver code --------------------------------------------------------

If a constructed processor card with a port is connected for example to a PC,

then it makes sense for one part of the CODE region to use a RAM in order to

load the CODE over the port. An EPROM is always needed, at least for the

loader routine of the port. The code at address 0 (reset address) is read

from the EPROM.

When using an EPROM 16kx8 and a RAM 32kx8 the following setups can, for

example, be made:

XDATA 0000...7FFFh  -->  access on   RAM 0000...7FFFh

CODE  0000...3FFFh  -->  access on EPROM 0000...3FFFh

CODE  4000...7FFFh  -->  access on   RAM 4000...7FFFh

3.8 external connection, example                           Page 27

�������������������������������������������������������������������������

�                                                 �           �

_PSEN �� _PSEN ����������������������������������������Ĵ_OE        �

�                    �                            �     EPROM �

P2.6 �� A14 ������������������������������������������Ĵ_CE   16kx8�

�        �           �  �����Ŀ                   �������������

�        �           ��Ĵ     �     �����Ŀ

�        �  �����Ŀ     � NOR �����Ĵ     �       �����������Ŀ

8051  �        ��Ĵ INV �����Ĵ     �     � NOR �������Ĵ_OE        �

CPU   �           �������     �������  ��Ĵ     �       �           �

�                       �����Ŀ  �  �������       �           �

P3.7 �� _RD ����������������Ĵ INV ����             ��Ĵ_CE        �

�                       �������                �  �     RAM   �

�                                                 �      32kx8�

P3.6 �� _WR ������������������������������������������Ĵ_WR        �

�                                                 �           �

The diagram shows a possible connection of _PSEN and _RD to the

generation of the RAM read signals for the above example. In this setup,

the CODE up till address 3FFFh from the EPROM and CODE above 4000h works

from RAM.

CODE is loaded into RAM through MOVX. The interrupt vectors are in EPROM.

There the vectors can be connected to the load region via LJMP 4003h, LJMP

400Bh ... , as long as they are not used by the loader routine.

An especially simple realisation of the code execution from the external RAM

(load driver code) can be achieved, when the load routines are burnt into

internal EPROM and _EA = 1 is set. Now _PSEN and _RD need only be connected

over an AND gate. An external EPROM is no longer required.

�   o +5V

_EA �����                              �           �

�                 �����Ŀ       ��Ĵ_CE        �

_PSEN �� _PSEN ��������Ĵ     �       �  �           �

�                 � AND ����������Ĵ_OE        �

P3.7 �� _RD ����������Ĵ     �          �     RAM   �

�                 �������          �      32kx8�

P3.6 �� _WR ���������������������������Ĵ_WR        �

�                                  �           �

3.9 internal RAM                                             Page 28

���������������������������������������������������������������������������

3.9  internal RAM:

------------------

A special feature of the 8051 is its internal RAM of 128 bytes. The access is

faster compared to access over the external RAM and there are different

access methods. With simple programs the 8051 can (in combination with the

internal EPROM) act as a stand alone processor.

Access methods for the internal RAM:

------------------------------------

direct:    the internal RAM address is given in the code

register:  special instructions for the access of register R0..R7,

which lie in internal RAM at addresses 00..1Fh.

indirect:  through pointer.  R0, R1, SP can act as pointers

bit:       the internal RAM addresses 20h..2Fh can additionally be selected

bitwise.

� � � � � � � � FF         Shown here is the partitioning of the

|             |            internal RAM as far as addressing

|  80h .. FFh |            possibilities are concerned.

|             |

80 �������������Ŀ            The 8051 has an internal RAM of 128

�             � 7F         bytes. The overall regions can be addressed

|             |            directly or indirectly.

|             |            The 8052 and others have an internal RAM of

�             �            256 bytes. Only 00...7Fh may be directly

�             �            addressed. The direct DATA addresses above

�             �            80h are reserved for SFR (see 3.4)

30 �������������Ĵ

28 �    Bit      � 2F         The addresses 20...2Fh can be bitwise

20 �������������Ĵ            addressed. Assigned are the BIT-

18 � Reg.-Bank 3 � 1F         addresses 00..7Fh. The BIT addresses above

10 � Reg.-Bank 2 �            80h are reserved for SFR.

08 � Reg.-Bank 1 �

00 � Reg.-Bank 0 �            The registers are situated from 00 to 1Fh

���������������            (see also 3.4.3 PSW).

Register:

---------

The 8051 has 8 general purpose registers defined. These are not counted as

SFRs (special function registers), since they do not have special hardware

functions such as possessed by TCON, SCON and other SFRs.

The registers are differentiated by the fact that there are special (simple

and fast) instructions for accessing them. They are not like the SFRs, placed

in extra memory cells, rather their values are stored in internal RAM.

This takes place in 8 sequential internal RAM addresses = one register bank.

The active register bank can be selected through two BITs in the program

status word (see 3.4.3 PSW).

If register bank 0 is selected (this is the case after a reset, or

after ANL PSW, #11100111b or CLR RS0 and CLR RS1), R0 is associated with

internal RAM address 00, R1 with 01 and so on. If register bank 3 is

selected, then R0 is associated with address 18h, R1 with 19h and so on.

3.9 internal RAM                                             Page 29

���������������������������������������������������������������������������

RS1:RS0    Reg Bank     DATA address

------------------------------------

00          0          00..07

01          1          08..0Fh

10          2          10..17h

11          3          18..1Fh

The different register banks have benefits when using interrupts:

If a register is used in an interrupt routine, then its contents must be

be saved (PUSH on Stack) upon entry to the interrupt routine. Before the

RETI the register must be restored (POP from Stack). A register bank can

be assigned to an interrupt routine. Now only the register bank has to

be switched around (faster!).

Although the 8051 has five interrupt sources, one can make do with 4

register banks, since there are only 2 interrupt priority levels. An

interrupt can never be intruded upon by another interrupt of the same or

lower priority. Interrupts of the same priority can therefore be

assigned to the same register bank, provided that from the RETI no

values in the register have to be retained until the next interrupt.

Since by a reset only some SFRs, but not the internal RAM are cleared, the

contents of the register during a reset remain provided reg bank 0 was

selected. With a reset, the register bank is switched back to reg bank 0.

R0 and R1 still have, besides the possibility of general purpose use through

the software, the function of index registers in indirect addressing in

internal RAM and pagewise access of the external RAM (see instruction

set).

indirect addressing:

--------------------

In addition, one can have access to the internal RAM indirectly through

pointers. The registers R0 and R1 or the stack pointer (SP) can serve as

pointers. For RAM addresses above 80h (only 8052...) the only possibility

of accessing internal RAM is indirect addressing.

An indirect access through the SP to the internal RAM takes place

automatically with every call up of a subprogram, and also with entry into an

interrupt routine. At the same time, the CPU lays the return address to the

stack (low byte first). For the user, there is also the possibility of placing

values, etc on the stack with the instructions PUSH and to be read back with

a POP (see also 3.4.4 Stack Pointer).

When using R0 or R1 to access internal RAM the address of the memory location

must first be loaded into R0 or R1. The access takes place with a further

instruction:

3.9 internal RAM                                             Page 30

���������������������������������������������������������������������������

Example:

MOV R0, #30h        ; internal RAM address 30h is loaded

MOV @R0, #0Dh       ; #0Dh is written in internal RAM 30h

An indirect access must seem complicated, but there are applications, where

indirect addressing is indispensable (e.g. with the handling of text strings,

with receive and send buffers...).

direct addressing:

------------------

Direct addressing permits quick access of data. From the programmer's

viewpoint, this allows for the storage of reserved values at prearranged

fixed addresses in internal RAM memory location. The assembler permits the

definition of variable names.

In the internal RAM of the 8051 (and also the enhanced CPU's 8052, 80535,...)

only 128 bytes can be directly addressed (addresses 00...7Fh). With the

linking of a program the address is put into the code.

Example:

Mnemonic:                                          Opcode:

MOV  A, 20h        ; content of int RAM 20h in ACC       -->  E5 20

MOV  A, #20h       ; the difference: here value in ACC   -->  74 20

direct BIT addressing:

---------------------

The internal RAM addresses 20h...2Fh can be selected bitwise. These

memory locations can also be used for flag bank (8 bits to a byte).

The access takes place with special bit instructions.

MSB                                 LSB

���������������������������������������Ŀ

� 7F � 7E � 7D � 7C � 7B � 7A � 79 � 78 �  DATA 2Fh

� 77 � 76 � 75 � 74 � 73 � 72 � 71 � 70 �  DATA 2Eh

� 6F � 6E � 6D � 6C � 6B � 6A � 69 � 68 �  DATA 2Dh

� 67 � 66 � 65 � 64 � 63 � 62 � 61 � 60 �  DATA 2Ch

� 5F � 5E � 5D � 5C � 5B � 5A � 59 � 58 �  DATA 2Bh

� 57 � 56 � 55 � 54 � 53 � 52 � 51 � 50 �  DATA 2Ah

� 4F � 4E � 4D � 4C � 4B � 4A � 49 � 48 �  DATA 29h

� 47 � 46 � 45 � 44 � 43 � 42 � 41 � 40 �  DATA 28h

� 3F � 3E � 3D � 3C � 3B � 3A � 39 � 38 �  DATA 27h

� 37 � 36 � 35 � 34 � 33 � 32 � 31 � 30 �  DATA 26h

� 2F � 2E � 2D � 2C � 2B � 2A � 29 � 28 �  DATA 25h

� 27 � 26 � 25 � 24 � 23 � 22 � 21 � 20 �  DATA 24h

� 1F � 1E � 1D � 1C � 1B � 1A � 19 � 18 �  DATA 23h

� 17 � 16 � 15 � 14 � 13 � 12 � 11 � 10 �  DATA 22h

� 0F � 0E � 0D � 0C � 0B � 0A � 09 � 08 �  DATA 21h

� 07 � 06 � 05 � 04 � 03 � 02 � 01 � 00 �  DATA 20h

�����������������������������������������

The LSB of DATA 20h has the BIT address 00. The next bit of DATA 20h

has BIT address 01 and so on:

3.9 internal RAM                                             Page 31

���������������������������������������������������������������������������

The BIT addresses 80h...FFh are reserved for Special Function Registers, even

when not all SFR addresses are used in the 8051. The SFR are always bitwise

addressable, by which the low nybble is address 0 or 8:

���������������������������������������Ŀ                8051-SFB

B         � F7 � F6 � F5 � F4 � F3 � F2 � F1 � F0 �  DATA F0h      --------

���������������������������������������Ĵ

ACC       � E7 � E6 � E5 � E4 � E3 � E2 � E1 � E0 �  DATA E0h

���������������������������������������Ĵ

PSW       � D7 � D6 � D5 � D4 � D3 � D2 � D1 � D0 �  DATA D0h

���������������������������������������Ĵ

IP        � BF � BE � BD � BC � BB � BA � B9 � B8 �  DATA B8h

P3        � B7 � B6 � B5 � B4 � B3 � B2 � B1 � B0 �  DATA B0h

���������������������������������������Ĵ

IE        � AF � AE � AD � AC � AB � AA � A9 � A8 �  DATA A8h

P2        � A7 � A6 � A5 � A4 � A3 � A2 � A1 � A0 �  DATA A0h

���������������������������������������Ĵ

SCON      � 9F � 9E � 9D � 9C � 9B � 9A � 99 � 98 �  DATA 98h

P1        � 97 � 96 � 95 � 94 � 93 � 92 � 91 � 90 �  DATA 90h

���������������������������������������Ĵ

TCON      � 8F � 8E � 8D � 8C � 8B � 8A � 89 � 88 �  DATA 88h

P0        � 87 � 86 � 85 � 84 � 83 � 82 � 81 � 80 �  DATA 80h

�����������������������������������������

Code for internal RAM access:

-----------------------------

For every addressing mode, there are special instructions.  --> see also

instruction set.

With the creation of a program it must be observed, that differently addressed

memory locations do not overlap. Large programs quickly

become unclear. With correct application more capable assemblers and linkers

have a chance of error recognition. Memory locations are classified according

to data type.

The following rules should be noted:

- If possible, data should be defined with names at the beginning of source

files. In the program, do not use instructions with absolute addresses,

otherwise clarity is quickly lost.

- "fill" internal RAM beginning with the lowest address. That is:

*** first consider the registers: (in the case of ASM51 and A51 with USING

0, USING 1, etc.) reserve locations, according to the number of the

used register banks.

*** then define flags (since these can only be accommodated in DATA

(20h...2Fh)

3.9 internal RAM                                             Page 32

���������������������������������������������������������������������������

*** then define direct DATA variables (if there is space, individually

assigned before the flags).

*** then allot locations for use of buffer (IDATA), which are to be

addressed indirectly

*** finally, space is allocated for stack.

Example for ASM51:

USING 1                ; reserve location for register bank 1

USING 0                ; reserve location for register bank 0

FLag1     BIT    0     ; this lies in DATA 20h

Variable1 DATA  10h    ; location is still free (Reg Bank 2 unused)

BufPtr    DATA  11h    ; until 1Fh DATA can still be directly defined

BufCnt    DATA  12h    ;  then watch out because of BIT (addresses)!

buffer    IDATA 21h    ; the buffer overlaps DATA and BIT

bufferLength  EQU 20h

stack     IDATA 41h    ; Stack begins on top of the buffer - location

; to 7Fh! After reset the SP is to be initialised

; with: MOV SP, #(stack - 1)

the assignment of addresses is performed here "by hand". The ASM51 also offers

the possibility to let this be done through the linker (see assembler).

4. Instruction set                                           Page 33

���������������������������������������������������������������������������

4.  8051 Instruction set:

-------------------------

All controllers of the 8051 family use the same instruction set. With the

newer (further developed) types, only some SFRs are used to control the

additional hardware functions.

A program consists of a series of instructions. The instructions are stored

in the form of coded numeric values in program memory (fixed value memory

e.g. EPROM). The 8051 has an internal translate table, in which every numeric

value is assigned a definite operation (see appendix).

In order that the 8051 executes a series of instructions (program), the

programmer must write a series of numeric values in the EPROM (machine code

or hexcode). For every instruction, he must look up the numeric value in a

table. This is very unclear and complicated.

A tool which relieves this work is the assembler: it translates a program

written in an abbreviated symbolic language into a series of numeric values.

The abbreviations of the individual instructions are shown as mnemonics. A

program so written down becomes itself an assembler sourcecode or machine

language. With it is processed that which the program language assembler

instructions use at the machine level.

Every instruction which the 8051 should execute later must therefore be

written as mnemonics.

A further popular tool for the generation of 8051 machine code is a C

compiler. C is a high level language, it provides predefined standardised

functions, which according to the call functions in the C sourcefile becomes

linked to the finished program. The functions are written as subroutines in

assembler, then compiled and placed in a library.

The advantage of a high level language such as C is that programs can be

created, which can be linked with another library and then run on different

computers. The programs for the 8051 is not so important, since the C

functions for the 8051 are very special. However, C offers several functions

such as floating point computations, which make the use of C compilers for the

8051 interesting. Many are also already familiar with the language C, which

besides a better clarity of the source files, often also becomes a deciding

point in favour of C.

In the following the complete instruction set of the 8051 is explained.

Indicated are the mnemonics, how they are used by the assembler and the

assigned hexcode. With some instructions, an extra one or two operands

are attached after the op code such as an address or a constant (coded

numeric values, then the instruction label).

In the mnemonics, abbreviations such as MOV (transfer), ADD (addition) etc,

are used. Instead of absolute addresses variable names are allowed. The

SFR names are already known as defaults to some assemblers and can as such be

loaded (for SFRs of the 8051 see 3.4).

4. Instruction set                                           Page 34

���������������������������������������������������������������������������

Additionally, there are some special abbreviations:

---------------------------------------------------

#       constant value follows

@       indirect addressing: Value from the memory digit, on which the

following register shows

A       Accumulator (= ACC, but a special opcode is generated, when

A instead of ACC is used)

C       Carry-Flag (= CY, but a special opcode is generated, when

C instead of CY is used)

DPTR    data pointer (for accessing external data) - is together set from

DPH and DPL.

PC      program counter (holds code address)

Ri      (with i = 0...1)  --> used to indicate that R0 or R1 is allowed for

this instruction.

Rn      (with n = 0...7)  --> used to indicate that R0 ... R7 are allowed

for this instruction.

In the following every 8051 instruction is briefly explained. To the left

is the code in ASM51 mnemonics and in the square in the middle the opcode

(hexvalue). Every square corresponds to a byte in program code. All the way

to the right is the number ofthe required cycles and the flags, in so far as

they themselves vary.

Mnemonic:             Hex Code:                        Cycles:  Flags:

---------------------------------------------------------------------------

To execute many instructions only a CPU cycle is needed. Some, especially

those with access to external memory require 2 cycles. A cycle lasts 6 CPU

system clocks (at 12 MHz quartz oscillator, 1 cycle lasts 1�s). In every

cycle the timer, if they are running, increase by 1 increment each.

4.1 Data transfer                                            Page 35

4.1 Data transfer:

--------------------

With the MOV instruction data from an internal register or from internal RAM

can be copied to another internal memory location.

�����������Ŀ

MOV A, Rn             � 1110 1nnn �                          1    P

�����������Ĵ

MOV Rn, A             � 1111 1nnn �                          1    -

�����������������������Ŀ

MOV A, direct         �    E5h    �  dataAdr  �              1    P

�����������������������Ĵ

MOV direct, A         �    F5h    �  dataAdr  �              1    -

�������������������������

MOV A, @Ri            � 1110 011i �                          1    P

�����������Ĵ

MOV @Ri, A            � 1111 011i �                          1    -

�����������������������Ŀ

MOV A, #data          �    74h    �   data    �              1    P

�����������������������Ĵ

MOV Rn, #data         � 0111 1nnn �   data    �              1    -

�����������������������Ĵ

MOV @Ri, #data        � 0111 011i �   data    �              1    -

�����������������������Ĵ

MOV Rn, direct        � 1010 1nnn �  dataAdr  �              2    -

�����������������������Ĵ

MOV direct, Rn        � 1000 1nnn �  dataAdr  �              2    -

�����������������������Ĵ

MOV @Ri, direct       � 1010 011i �  dataAdr  �              2    -

�������������������������

�����������������������Ŀ

MOV direct, @Ri       � 1000 011i �  dataAdr  �              2    -

�����������������������������������Ŀ

MOV direct, #data     �    75h    �  dataAdr  �   data    �  2    -

�����������������������������������Ĵ

MOV direct1, direct2  �    85h    � dataAdr2  � dataAdr1  �  2    -

�������������������������������������

Each operand which is placed on the right in the mnemonic is written on

the left.

What is worth noting is that in the hex code for "MOV direct1, direct2",

the sequence in the hexcode across that in the mnemonic is interchanged.

Example:  MOV  A, R5     ; ED        ->write contents of R5 to ACC

MOV  PSW, A    ; F5 D0     ->write contents of A to PSW

MOV  A, @R1    ; E7        ->write contents of internal RAM,

;             address which is in R1, to ACC

MOV  R2, #22h  ; 7A 22     ->write 22h to R2

MOV  R7, 22h   ; AF 22     ->write contents of internal RAM

;             address 22h to R7

MOV  30h, SP   ; 85 81 30  ->write SP to internal RAM addr 30h

4.1 Data Transfer                                           Page 36

���������������������������������������������������������������������������

With the XCH instruction, data will be exchanged between internal register

or internal RAM and the accumulator.

�����������Ŀ

XCH A, Rn           � 1100 1nnn �                          1    P

�����������������������Ŀ

XCH A, direct       �    C5h    �  dataAdr  �              1    P

�������������������������

XCH A, @Ri          � 1100 011i �                          1    P

�������������

Example:  if reg bank 1 is active, put #16h in DATA 9, in ACC, #9Fh

XCH  A, R1     ; C9    -> in ACC #16h and in DATA 9 #9Fh

XCHD interchanges the low nybble of the addressed byte in internal RAM with

the low nybble from the ACC.

�����������Ŀ

XCHD A, @Ri         � 1101 011i �                          1    P

�������������

Example:   Put R0 in #33h, in DATA 33h put #16h, in ACC #9Fh

XCHD  A, @R0   ; D6    -> in ACC #96h and in DATA 33h #1Fh

Load data pointer with constant:

������������������������������������Ŀ

MOV DPTR, #data16   �  90h  � data16(high) � data16(low) � 2    -

��������������������������������������

This is the only 16 bit operation of the 8051. It has the same

function as:

MOV DPH, #HIGH(data16)

and   MOV DPL, #LOW(data16)

4.1 Data Transfer                                            Page 37

-��������������������������������������������������������������������������

external data memory access:

�����������Ŀ

MOVX A, @DPTR       �    E0h    �                          2    P

�����������Ĵ

MOVX @DPTR, A       �    F0h    �                          2    -

�����������Ĵ

MOVX A, @Ri         � 1110 001i �                          2    P

�����������Ĵ

MOVX @Ri, A         � 1111 001i �                          2    -

�������������

When accessing external data memory the _RD becomes the read signal

and _WR becomes the write signal. The access takes place through the

alternative functions of Port 0 and Port 2 (see 3.5). The value in the

latch of Port 2 remains the same. The value in the latch of Port 0

will be overwritten during reading with 0FFH by the hardware.

IMPORTANT!

When indirectly accessing with Ri, only the low address multiplexed

with Port 0 is given out. The content of the latch of P2 remains during

the access at the port output. The user has to write the high byte of

the address into P2 before access when using external RAM.

Example:

MOV   DPTR, #0FF00h     ; load data pointer

MOVX  A, @DPTR          ; access (read) external RAM

MOV   R0, #0            ; this routine clears the external

MOV   P2, #1            ; RAM from address 100h to 1FFh. P2

CLR   A                 ; is loaded with the high byte of the

clearPage1:                  ; address (=Page), R0 gets the LOW byte

MOVX  @R0, A            ; R0 is continuously decremented.

DJNZ  R0, clearPage1

Read byte from the program memory:

�����������Ŀ

MOVC A, @A+DPTR     �    93h    �                          2    P

�����������Ĵ

MOVC A, @A+PC       �    83h    �                          2    P

�������������

A byte from the program memory (external or internal: depending on

the address and /EA Pin) is read into the ACC. The address is the

sum of the contents of ACC and the DPTR or the program counter (earlier

increased by 1 increment) (see also 3.3, 3.5 and 3.6).

The content of the latch of P2 doesn't change in the process, while

before access the hardware writes 0FFh into P0.

4.2 Arithmetic Operations                                    Page 38

���������������������������������������������������������������������������

Byte to/from stack:

�����������������������Ŀ

PUSH direct         �    C0h    �  dataAdr  �              2    -

�����������������������Ĵ

POP direct          �    D0h    �  dataAdr  �              2    -

�������������������������

Upon a PUSH the Stack Pointer is incremented by 1 and then the value

is written to this address. With a POP the SP is first read and then

decremented. The stack pointer always points to the final pushed byte.

Subprogram calls and interrupts also push values on to the stack

(see also ACALL, LCALL, RET, RETI). If the stack is full (SP points

to addresses above 7Fh, nevertheless with a PUSH, the SP is further

incremented, the data however gets lost.

The 8044 has an internal RAM of 192 bytes (to address 0BFh). The

8052 has an internal RAM of 256 bytes. When SP overruns over 0FFH

the register region of the 8052's internal RAM is overwritten.

After a Reset the SP stands at address 07.

4.2  Arithmetic operations:

---------------------------

Adding value, register, internal RAM Addr to ACC:

�����������������������Ŀ

ADD A, #data        �    24h    �    data   �              1    C AC P OV

�������������������������

ADD A, @Ri          � 0010 011i �                          1    C AC P OV

�����������Ĵ

ADD A, Rn           � 0010 1nnn �                          1    C AC P OV

�����������������������Ŀ

ADD A, direct       �    25h    �  dataAdr  �              1    C AC P OV

�������������������������

After the execution the ACC holds the result, the C Flag holds the

overflow in Bit 7 and AC holds the overflow from Bit 3. The OV flag is

set, when a sign error occurs (see 3.4.3 PSW).

Example:             before ADD:          after ADD:

ADD  A, #44h    ; A = 33h    -->    A = 77h, C = 0, AC = 0, OV = 0

ADD  A, #44h    ; A = 43h    -->    A = 87h, C = 0, AC = 0, OV = 1

ADD  A, #4Ah    ; A = 2Ah    -->    A = 74h, C = 0, AC = 1, OV = 0

ADD  A, #93h    ; A = 0B1h   -->    A = 44h, C = 1, AC = 0, OV = 1

4.2 Arithmetic Operations                                    Page 39

���������������������������������������������������������������������������

Add value, register, internal RAM addr. to ACC with Carry:

�����������������������Ŀ

ADDC A, #data       �    34h    �    data   �              1    C AC P OV

�������������������������

ADDC A, @Ri         � 0011 011i �                          1    C AC P OV

�����������Ĵ

ADDC A, Rn          � 0011 1nnn �                          1    C AC P OV

�����������������������Ŀ

ADDC A, direct      �    35h    �  dataAdr  �              1    C AC P OV

�������������������������

As addition without carry. However, the value of the Carry flag is added

as well.

Example:               before ADD:          after ADD:

ADDC  A, #44h    ; A = 33h, C = 1  -->  A = 78h, C = 0, AC = 0, OV = 0

ADDC  A, #44h    ; A = 33h, C = 0  -->  A = 77h, C = 0, AC = 0, OV = 0

Subtract value, register,internal RAM addr. from ACC with Carry:

�����������������������Ŀ

SUBB A, #data       �    94h    �    data   �              1    C AC P OV

�������������������������

SUBB A, @Ri         � 1001 011i �                          1    C AC P OV

�����������Ĵ

SUBB A, Rn          � 1001 1nnn �                          1    C AC P OV

�����������������������Ŀ

SUBB A, direct      �    95h    �  dataAdr  �              1    C AC P OV

�������������������������

Subtract immediate data, indirect byte, contents of Rn rsp. direct

byte plus the Carry flag from ACC. Flags see ADD.

Increment = the contents of the given register increases by 1.

�����������Ŀ

INC A               �    04h    �                          1    P

�����������Ĵ

INC Rn              � 0000 1nnn �                          1    no Flags

�����������Ĵ

INC @Ri             � 0000 011i �                          1    no Flags

�����������������������Ŀ

INC direct          �    05h    �  dataAdr  �              1    no Flags

�������������������������

INC DPTR            �    A3h    �                          2    no Flags

�������������

4.2 Arithmetic Operations                                    Page 40

���������������������������������������������������������������������������

Decrement = the contents of the indicated register is decreased by 1.

�����������Ŀ

DEC A               �    14h    �                          1    P

�����������Ĵ

DEC Rn              � 0001 1nnn �                          1    no Flags

�����������Ĵ

DEC @Ri             � 0001 011i �                          1    no Flags

�����������������������Ŀ

DEC direct          �    15h    �  dataAdr  �              1    no Flags

�������������������������

Multiply ACC with B-Register:

�����������Ŀ

MUL AB              �    A4h    �                          4    C P OV

�������������

The result is in ACC. When the result is a word, the low Byte is in ACC,

the high byte in B and the OV flag is set. The C Flag is always cleared

(C = 0).

Divide ACC by B register:

�����������Ŀ

DIV AB              �    84h    �                          4    C P OV

�������������

The result is in ACC and the remainder in B. When an error (Division

by 0) occurs the OV - Flag is set. The carry flag is always cleared.

Decimal adjust of ACC:

�����������Ŀ

DA A                �    D4h    �                          1    C P

�������������

When two BCD numbers are added together, the result must thereafter be

corrected, it should also come out in BCD format.

If the AC flag is set, or the low nybble > 9, then 6 is added. If the

C flag is set, or the high nybble > 9, then 60h is added. When there is

an overflow of bit 7 of the ACC in the result, then the carry flag (C=1)

is set.

4.3 logical associations                                Page 41

���������������������������������������������������������������������������

Example:                       bcd_59 = 0101 1001

bcd_68 = 0110 1000

+  ������������

after ADD (without correction): bcd_C1 = 1100 0001    (Flags: AC=1, C=0)

because of (AC=1):                       0000 0110

because of (high nybble > 9):            0110 0000

+  ������������

result (with correction):       bcd_27 = 0010 0111    (Flags: C=1)

4.3  logical associations:

--------------------------

�����������������������Ŀ

ANL A, #data        �    54h    �   data    �              1    P

�������������������������

ANL A, @Ri          � 0101 011i �                          1    P

�����������Ĵ

ANL A, Rn           � 0101 1nnn �                          1    P

�����������������������Ŀ

ANL A, direct       �    55h    �  dataAdr  �              1    P

�����������������������Ĵ

ANL direct, A       �    52h    �  dataAdr  �              1    no Flags

�����������������������������������Ŀ

ANL direct, #data   �    53h    �  dataAdr  �   data    �  2    no Flags

�������������������������������������

bitwise logical AND association:

when both bits 1, then 1

�����������������������Ŀ

ORL A, #data        �    44h    �   data    �              1    P

�������������������������

ORL A, @Ri          � 0100 011i �                          1    P

�����������Ĵ

ORL A, Rn           � 0100 1nnn �                          1    P

�����������������������Ŀ

ORL A, direct       �    45h    �  dataAdr  �              1    P

�����������������������Ĵ

ORL direct, A       �    42h    �  dataAdr  �              1    no Flags

�����������������������������������Ŀ

ORL direct, #data   �    43h    �  dataAdr  �   data    �  2    no Flags

�������������������������������������

bitwise logical OR association:

when one of the two bits is 1, then 1

4.3 logical associations                                    Page 42

���������������������������������������������������������������������������

�����������������������Ŀ

XRL A, #data        �    64h    �   data    �              1    P

�������������������������

XRL A, @Ri          � 0110 011i �                          1    P

�����������Ĵ

XRL A, Rn           � 0110 1nnn �                          1    P

�����������������������Ŀ

XRL A, direct       �    65h    �  dataAdr  �              1    P

�����������������������Ĵ

XRL direct, A       �    62h    �  dataAdr  �              1    no Flags

�����������������������������������Ŀ

XRL direct, #data   �    63h    �  dataAdr  �   data    �  2    no Flags

�������������������������������������

bitwise logical XOR association:

when both bit are different, then 1

�����������Ŀ

CLR A               �    E4h    �                          1    P

�������������

clear ACC  -->  ACC = 0

�����������Ŀ

CPL A               �    F4h    �                          1    P

�������������

Complement to ACC  -->  ACC is bitwise inverted

�����������Ŀ

RL A                �    23h    �                          1    no Flags

�������������

rotate ACC bitwise to the left:

slides contents of ACC bitwise to the left. The highest bit is written

to the lowest. The parity flag does not change, since the sum above all

the bits remains the same.

�����������Ŀ

RLC A               �    33h    �                          1    P C

�������������

rotate ACC bitwise from the left through Carry :

slides contents of ACC bitwise to the left. The highest bit is

consequently in C and C is written to the lowest bit.

4.4 Bit manipulations                                        Page 43

���������������������������������������������������������������������������

�����������Ŀ

RR A                �    03h    �                          1    no Flags

�������������

rotate ACC bitwise to the right:

slides contents of ACC bitwise to the right. The lowest bit is

written to the highest. The parity flag itself is not changed, since

the sum over all the bits stays the same.

�����������Ŀ

RRC C               �    13h    �                          1    P C

�������������

rotate ACC bitwise to the right through Carry:

slides Contents of ACC bitwise to the right. The lowest bit is in C

and C is written into the highest bit.

�����������Ŀ

SWAP A              �    C4h    �                          1    no Flags

�������������

the low nybble of the ACC is interchanged with the high nybble

4.4  Bit-Manipulation:

----------------------

�����������������������Ŀ

MOV C, bit          �    A2h    �  bitAdr   �              1     C

�����������������������Ĵ

MOV bit, C          �    92h    �  bitAdr   �              2     no Flags

�������������������������

copy contents from bit address to C or C to bit address.

�����������������������Ŀ

ANL C, bit          �    82h    �  bitAdr   �              2     C

�����������������������Ĵ

ANL C, /bit         �    B0h    �  bitAdr   �              2     C

�������������������������

logical AND bit or inverted value of bit to C

4.5 absolute jumps and CALL's                            Page 44

���������������������������������������������������������������������������

�����������������������Ŀ

ORL C, bit          �    72h    �  bitAdr   �              2     C

�����������������������Ĵ

ORL C, /bit         �    A0h    �  bitAdr   �              2     C

�������������������������

logical OR bit or inverted value of the bit to C

�����������Ŀ

CLR C               �    C3h    �                          1     C

�����������������������Ŀ

CLR bit             �    C2h    �  bitAdr   �              1     no Flags

�������������������������

clear bit or Carry-Flag  -->  Bit = 0

�����������Ŀ

CPL C               �    B3h    �                          1     C

�����������������������Ŀ

CPL bit             �    B2h    �  bitAdr   �              1     no Flags

�������������������������

invert bit or carry flag

�����������Ŀ

SETB C              �    D3h    �                          1     C

�����������������������Ŀ

SETB bit            �    D2h    �  bitAdr   �              1     no Flags

�������������������������

set bit or Carry-Flag  -->  Bit = 1

4.5  absolute jumps and CALLs:

----------------------------------

CALL codeAdress

This is not a machine instruction of the 8051. The assembler itself

determines here whether a ACALL or LCALL is to be used. By "Forward

References", i.e. addresses, which the assembler at this point does not

yet know, a LCALL is always inserted, even when an ACALL is enough.

4.5 absolute jumps and CALL's                            Page 45

���������������������������������������������������������������������������

JMP codeAdress

This is not a machine instruction of the 8051. The assembler itself

determines here if an SJMP, AJMP or LJMP is to be used. With "forward

references" an LJMP is always inserted, even if it is not efficient.

��������������������������������������������Ŀ

ACALL codeAdr  � A10 A9 A8 1 0001 � A7 A6 A5 A4 A3 A2 A1 A0 �   2  no Flags

����������������������������������������������

Absolute CALL (within a 2 KByte Page):

The PC (Program Counter) is incremented by 2 and the return address

is placed on the stack (low byte first). Then the following

address is loaded into the PC:

A11 to A15 into PC (Program Counter) remain unchanged,

the lower 11 bits are loaded from the opcode.

����������������������������������Ŀ

LCALL codeAdr  �   12h   � CodAdrHigh � CodAdrLow �         2    no Flags

������������������������������������

Long CALL:

The PC (Program Counter) is incremented by 3 and the return address is

loaded on the stack (low byte first). Then it is loaded with the value

from the opcode.

�����������Ŀ

RET                 �    22h    �                           2    no Flags

�������������

Return from subprogram (CALL):

The return address is fetched from the stack (high byte first) and

jumped to.

�����������Ŀ

RETI                �    32h    �                           2    no Flags

�������������

Return from Interrupt:

The CPU has taken note in an internal register, which interrupts

are being handled. There can also be two interrupts, since there are

two priority levels, and a routine at a lower level of an interrupt

can be disrupted by one of a higher level. When an interrupt is being

serviced, all other interrupts at equal or lower priority level are

blocked. With RETI, the interrupt with the higher value is deleted and

the interrupts are free again.

4.5 absolute jumps and CALL's                            Page 46

���������������������������������������������������������������������������

The return address is fetched from the stack (high byte first) and

jumped to.

Warning:  Should a software reset          MOV A, #0

take place within an interrupt          PUSH ACC

routine through a jump to 0000,         PUSH ACC

the jump must then take place           MOV A, #LOW(retiReset)

over a double RETI, since               PUSH ACC

otherwise the internal Enable           MOV A, #HIGH(retiReset)

Register contrary to a hardware         PUSH ACC

reset, would not be cleared.      retiReset:

RETI

�����������������������Ŀ

SJMP codeAdr      �    80h    � relOffset �                 2     no Flags

�������������������������

Short JUMP (-128 +127):

The PC is incremented by 2 and with it is the relOffset added.

The highest bit of the value relOffset acts as the sign.

For computation: the relOffset (BYTE) is first converted to a WORD

(on an 8088 compatible CPU by means of CBW). If the relOffset is negative

(high bit is set), 0FF00h is added to it, otherwise 0. This WORD is

added to the incremented PC.

Example:     relOffset = 47h  --> 0047h  +  PC

relOffset = 87h  --> FF87h  +  PC

��������������������������������������������Ŀ

AJMP codeAdr   � A10 A9 A8 0 0001 � A7 A6 A5 A4 A3 A2 A1 A0 �  2   no Flags

����������������������������������������������

Absolute JUMP (within 2 KByte Page):

The PC is incremented by 2 and the address is then constructed in the

following manner: A11 to A15 in PC (program counter) remains unchanged,

the lower 11 bits are loaded from the opcode.

����������������������������������Ŀ

LJMP codeAdr   �   02h   � CodAdrHigh � CodAdrLow �         2     no Flags

������������������������������������

Long JUMP:

Jump to the address indicated in the opcode.

4.6 conditional jumps                                         Page 47

���������������������������������������������������������������������������

�����������Ŀ

JMP @A+DPTR         �    73h    �                           2    no Flags

�������������

Jump indirectly to Accumulator + Data Pointer:

Add contents from ACC and DPTR, then jump to this address.

4.6  conditional jumps:

-----------------------

Jump by the relative offset, when the condition is fulfilled. For the

computation of the target address see SJMP (4.5 unconditional jump).

�����������������������Ŀ

JZ codeAddress      �    60h    � relOffset �               2     no Flags

�����������������������Ĵ

JNZ codeAddress     �    70h    � relOffset �               2     no Flags

�������������������������

jump, when ACC = 0 or when ACC not = 0

�����������������������Ŀ

JC codeAddress      �    40h    � relOffset �               2     no Flags

�����������������������Ĵ

JNC codeAddress     �    50h    � relOffset �               2     no Flags

�������������������������

jump, when C = 1 or C = 0

���������������������������������Ŀ

JB bit, codeAdress  �   20h   �  bitAdr   � relOffset �     2    no Flags

���������������������������������Ĵ

JNB bit, codeAdress �   30h   �  bitAdr   � relOffset �     2    no Flags

���������������������������������Ĵ

JBC bit, codeAdress �   10h   �  bitAdr   � relOffset �     2    no Flags

�����������������������������������

jump, when bit = 1, or Bit = 0:

In the case of JBC: Jump, when bit = 1 and clear bit.

4.6 conditional jumps                                        Page 48

���������������������������������������������������������������������������

�����������������������������������Ŀ

CJNE A, direct,codeAdr   �    B5h    �  dataAdr  � relOffset �  2     C

�����������������������������������Ĵ

CJNE A, #data, codeAdr   �    B4h    �    data   � relOffset �  2     C

�����������������������������������Ĵ

CJNE Rn, #data,codeAdr   � 1011 1nnn �    data   � relOffset �  2     C

�����������������������������������Ĵ

CJNE @Ri,#data,codeAdr   � 1011 011i �    data   � relOffset �  2     C

�������������������������������������

compare and jump, when not equal:

If the left operand less than the right, the carry flag is subsequently

set (C = 1).

�����������������������Ŀ

DJNZ Rn, codeAdr    � 1101 1nnn � relOffset �              2    no Flags

�����������������������������������Ŀ

DJNZ direct, codeAdr�    D5h    �  dataAdr  � relOffset �  2    no Flags

�������������������������������������

Decrement and jump after that, when contents not equal to 0

�����������Ŀ

NOP                 �    00h    �                          1    no Flags

�������������

no action. NOP is often used when a waiting time has to be passed through.

With a crystal of 12 MHz one NOP takes 1 �s.

�����������Ŀ

reserved            �    A5h    �                          ?    ?

�������������

invalid opcode. No instruction is assigned to this HEX code.

5. Assembler                                                 Page 49

���������������������������������������������������������������������������

5.  Assembler:

~~~~~~~~~~~~~~

An assembler is used to translate a program written with mnemonic symbols

into executable HEX code. Every instruction is individually written in

source code. Some assemblers may offer the possibility of defining MACROs,

but even there every single instruction is defined in the source file. MACROs

act only as an abbreviation for instructions which often repeat themselves.

The assembler translates every individual instruction to the corresponding

HEX code (see 4. instruction set). The assembler also calculates the target

addresses for a JMP or CALL.

For the 8051 there are a number of commercial assemblers such as the ASM51

from Intel or the A51 from Keil. But there are also several shareware products

such as TASM, which are equally suitable. From the magazine Elektor, there is

an assembler course which is currently being carried out (start: 09.1991) and

at the same time, there is also a cost effective assembler EASM51. However

EASM51 generates HEX which is self defined. Most other assemblers generate HEX

code in Intel HEX format.

--- There is now also a public domain macro assembler (AS), from

Alfred Arnold, Pr�lat-Lewen-Str.3, 5206 Neukirchen 1

Request from:  ftp.rog.rwth-aachen.de   (directory pub/assembler)

5.1  ASM51:

~~~~~~~~~~~~~~~

The ASM51 and the A51 are largely compatible. The assembler itself generates

an object file. This file cannot be loaded into an EPROM or into an 8051

processor. The object file contains as yet no absolute addresses. In the

source code, defined segments (in the code region), being relocatable, can

still be moved around within the 64 kByte address location.

The address allocation is handled by the Linker (RL51.EXE). The linker

generates an absolute file. The program is now assigned code addresses.

This process has its advantages in large programs. The linker can link

together several object files into an absolute file. A program can be created

from several smaller modules.

It remains clearer, besides, the ASM51 is this way suitable for use in

companies where often several people work on a program.

In addition, there is the possibility of putting together into a library file

general routines, which are used frequently (LIB51.EXE). Through the Linker,

library routines can be bound into an absolute file.

The absolute file still cannot be loaded into various Eproms. It still

requires a converter program from Absolute to HEX format (OH.EXE).

Assembler:         ASM51   -->   Object file

Linker:            RL51    -->   Absolute file

Converter:         OH      -->   HEX-File

5. Assembler                                   Page 50

���������������������������������������������������������������������������

Note: The absolute and the object file have the same structure:  The

files consist of contiguous RECORDs. Every record starts with a byte,

which defines the record type, after that follows a word with the number

of bytes which follow in the record body. At the end another byte follows

with a checksum. The structure of the record body varies according to

the record type. Absolute and object files hold code in binary

form. Additionally it can contain variable names defined in the source

file.

Even when an object file contains code in binary form, it should not be

confused with a binary file. Binary files are used by simple assemblers

and eprom programmers. Binary files only contain code. The offset inside

the files is equal to the address.

Binary files such as those of short programs which begin at offset 0000

are very compact. With object files, the record type and the record

length are still placed at the beginning. With programs which contain

code with high addresses, the binary file becomes more substantial

quickly. Unused addresses which lie before that must be filled up with

0FFh.

HEX-Files contain only code. Every byte is represented as an ASCII HEX

digit. They can be viewed with a normal editor. Hex Files are standard

for loading onto many Eprom programmers.

In this description we will not go into the various enhancements of an

assembler such as the ASM51. Rather, we'll explain only briefly the essential

syntax involved in the creation of a small program.

The ASM recognizes all mnemonics, such as defined in chapter 4. The ASM51

also recognizes the default SFR and SFB definitions of the 8051. Therefore

the abbreviation PSW DATA 0D0h need not, for example be defined; PSW can be

directly used.

For bit addresses the predefined SFB name can likewise be brought in. The SFR

name of the (DATA-)byte, in which this bit is located, can also be used. The

the number of the bit can be indicated after a period.

Example:

The Auxiliary Carry flag (BIT 0D6h) is located   SETB 0D6h

in the Program Status Word (DATA 0D0h).          SETB AC

The following instructions lead to the same      SETB PSW.6

result.

If code for another CPU is be generated, then the additional SFR and SFB must

be defined. Here an Include File is used in practice. With ASM51, include

files with the SFR definitions for other CPUs are included on the disk. The

Include files contain all the SFR definitions for the CPU, including those

which the ASM51 already recognizes by default (of the 8051). The ASM51 brings

in such a case an error message; for that reason the SFR definitions which are

known to be default will be turned off with the instruction $NOMOD51.

5. Assembler                                           Page 51

���������������������������������������������������������������������������

Instruction (directives) can be indicated as well in the source file to

control the assembler. Only the most important are explained here. Most of

the others are concerned anyway with the format of the listing file.

$noMOD51    the SFRs for the 8051, which are otherwise known to be the

default, are turned off. $NOMOD51 must be placed at the beginning of

the source file.

$INCLUDE(fileName)        the mnemonics which are contained in the indicated

file will take over, at the location in the assembler source file where

$INCLUDE stands. The Include file must contain valid source code.

$DEBUG     the variable definitions are taken over as debug information into

the Object File. $DEBUG is placed at the beginning of the source files.

$DEBUG and $NOMOD51 can also be given at the command line (then without $).

With USING n (n = 0..3) the Register bank can be reserved. The Linker reserves

the appropriate addresses in the internal RAM. Default is set at USING 0.

ASM51 knows the following code addresses by default. These definitions

are not switched off even with the instruction noMOD51.

RESET   CODE  00h                  Note: for the 8052 the definition

ExtI0   CODE  03h                        Timer2 CODE 2Bh should

Timer0  CODE  0Bh                        still be inserted

ExtI1   CODE  13h                        into the Include file

Timer1  CODE  1Bh                        with the SFR definitions.

SINT    CODE  23h

5.2  Segments under ASM51:

-------------------------------

A program can already be prepared with fixed addresses in the source code, or

written to be relocatable. These options may also be used in combination.

The addressing mode of the type DATA, IDATA, XDATA, BIT or CODE is

correspondingly assigned to the variables of the various memory regions or of

the code. A segment indication of the correct type is to be preassigned to

each variable definition or code procedure.

5. Assembler                                             Page 52

���������������������������������������������������������������������������

5.2.1  fixed address assignment:

-----------------------------

A fixed address assignment is the simplest way to create a program in ASM51.

Here the addresses which are already fixed are placed in the object file. The

linker cannot move the translated code regions about anymore.

At professional programmer sites, fixed segments are only used where the

hardware necessitates fixed addresses. For example, with jump addresses for

reset and the interrupts. But also for certain fixed addresses in XDATA

region (if a hardware extension is built under a certain definite address

there).

A fixed address assignment is achieved by first using one of the following

key words:

CSEG,  XSEG, DSEG, ISEG, BSEG

The additional "AT addr" may follow the above keywords. With that, the

beginning of the segment is set on a definite fixed address.  Otherwise, the

ASM51 by default begins every segment with 0000.

A segment remains open until a new key word (from the above) is indicated. At

the beginning of the source files the assembler assumes a default CSEG AT 0000.

Naturally, code can only be entered in the CSEG. For that, the mnemonics,

which are explained in chapter 4 are used. In addition, there are the

following pseudo instructions to facilitate the inclusion of text, tables or

fixed values in the code:

DB byte         ; reserve 1 Byte (? means value = any)

DW word         ; reserve 1 Word

Example:

DB 0A5h                        ; Hex value A5h

DB 'This is a Text.',0         ; Text in quotes followed by hex value 0

DW label1                      ; Hex value of the address of Label1. The

; high byte is first stored.

In other segments (DSEG, ISEG, XSEG or BSEG) code mnemonics cannot be stored.

Here only variable names are defined. The location for every defined variable

is defined with a DS. In every segment type, variables are fixed by a label.

Every label has a : placed after it.

DS   n            reserve n Bytes

DBIT n            reserve n Bit

5. Assembler                                                 Page 53

���������������������������������������������������������������������������

Example:

ISEG AT 30h

buffer:   DS  20       ; 20 bytes reserved for buffer

stack:    DS  12

CSEG                      ; now continues with Code = ISEG closed

.......

For a very simple program the use of segments may be omitted. Then, the

variable names are defined with following directives (example):

dAdr1     DATA      23h

iAdr1     IDATA     8Fh

xAdr1     XDATA   1234h

bAdr1     BIT        0

cAdr1     CODE   0FFF0h

value1    EQU        1             ; Definition of a fixed value

This form is used also for the definition of SFRs for other CPUs other than

the 8051, in order to incorporate it with an INCLUDE.

Simple assemblers often recognize only the EQU assignment. There, variables

of type DATA, XDATA, IDATA and BIT are also defined with EQU. The ASM51 can do

this as well. However, error messages which are generated by ASM51 are

lost, if a variable with a wrong type is used in an instruction.

Simple assemblers also do not use the directive CSEG AT Addr, in order to

begin the assembling of a code address, rather they use ORG addr.

When creating a program, often only the following directives are needed:

END           ; This key word must be placed at the end of the assembler

; source file.

$             ; instead of this symbol, the ASM51 always brings in the

; actual address of the PC.

5. Assembler                                                 Page 54

���������������������������������������������������������������������������

---------------------------------

With relocatable segment statements, first the segment names are defined in

the following form. Several identical segment types can also be defined.

segNameC   SEGMENT  CODE

segNameX   SEGMENT  XDATA

segNameD   SEGMENT  DATA

segNameI   SEGMENT  IDATA

segNameB   SEGMENT  BIT

The following keywords are then to be placed before the individual areas:

RSEG  segNameC

RSEG  segNameX

RSEG  segNameD

RSEG  segNameI

RSEG  segNameB.

The entry of code mnemonics and variable definition takes place just as with

fixed segments.

In connection with relocatable written program modules, the following

directives are important:

PUBLIC varName1, varName2,...   ; Variable names are made known to the

; linker

EXTRN CODE (cAdr1,cAdr2,...)    ; Variable names are externally declared.

EXTRN DATA (dAdr1,dAdr1,...)    ; With that, the assembler knows that they

EXTRN IDATA (iAdr1,....)        ; are defined in another segment.

EXTRN XDATA (xAdr1,....)

EXTRN BIT (bAdr1,bAdr2....)

EXTRN NUMBER (value1,value2...)

5.3  ASM51 call:

------------------

The calling up of the assembler, linker and converter program occurs in the

form below. The statements in the <> are optional. Otherwise, as default,

the object file receives the extension .obj, the listfile .lst and the

absolute file receives no extension. The default setting of the size of the

internal RAM in the linker is 128 bytes.

ASM51 sourcename <OJ(objectname)> <PR(listname)>

RL51 objectname <TO absolutename> <RAMSIZE(256)>

OH absolutename TO hexname

5. Assembler                                                 Page 55

���������������������������������������������������������������������������

5.4  Program example:

---------------------

Following program example shows how the variable names are defined

absolutely. A serial interrupt routine is shown in which a received character

is sent back as an echo.

We are working only with CSEG. The code begins with the default setting of

CSEG AT Reset with address 0. The interrupt routine is fixed and situated at

the address 23h.

����������������������������������������������������������������������Ŀ

�TxFlag    BIT  00            ; Bit0 in DATA 20h as  flag              �

�stack     IDATA 21h          ; address in internal RAM for stack      �

�                                                                      �

�;CSEG AT reset               ; Code starts at default label Reset     �

�     AJMP init               ; Jump to the initialisation             �

�                                                                      �

�CSEG AT SINT                 ; address 23h is the entry point for SINT�

�     JNB RI, sintChkTx       ; has something been received?           �

�     PUSH ACC                                                         �

�     MOV A, SBUF             ; read in                                �

�     CLR RI                  ; receive buffer again ready             �

�     JB TxFlag, echoChar                                              �

�     JNB TI, $               ; wait until send buffer is free         �

�echoChar:                                                             �

�     MOV SBUF, A             ; send back as echo                      �

�     CLR TxFlag                                                       �

�     CLR TI                  ; Mark that send buffer is full          �

�     POP ACC                                                          �

�sintChkTx:                                                            �

�     JNB TI, sintRet         ; when send buffer empty interrupts, then�

�     SETB TxFlag             ; flag is set, thereafter clear TI so    �

�     CLR TI                  ; serial interrupt is accessible again   �

�sintRet:                                                              �

�     RETI                                                             �

�                                                                      �

�init:                                                                 �

�     MOV SP, #(stack - 1)    ; stack placed                           �

�     MOV IE, #90h            ; enable SINT                            �

�     MOV SCON, #50h          ; serial port and timer 1                �

�     MOV TH1, #0FDh          ; initialise                             �

�     MOV TMOD, #20h                                                   �

�     SETB TR1                                                         �

�     JMP $                   ; Waiting loop. Interrupt on receive     �

�                                                                      �

�END                                                                   �

������������������������������������������������������������������������

In the above program, the subsequent placement of the code in program

memory is already fixed during the assembly.

In the following page, another program example with relocatable code

is presented:

5. Assembler                                                 Page 56

���������������������������������������������������������������������������

����������������������������������������������������������������������Ŀ

�idataSeg SEGMENT IDATA       ; Name for relocatable segments          �

�dataSeg SEGMENT DATA         ; are defined.                           �

�codeSeg SEGMENT CODE                                                  �

�                                                                      �

� PUBLIC initRingBuf                                                   �

� PUBLIC putCharToBuf, getCharFromBuf                                  �

�                                                                      �

� USING 0                     ; reserve Register bank 0                �

�                                                                      �

�RSEG dataSeg                 ;                                        �

� rdBufPtr: DS 1              ; Read pointer for ringbuffer            �

� wrBufPtr: DS 1              ; Write pointer for ringbuffer           �

�                                                                      �

�RSEG idataSeg                ; read in                                �

� ringBuf:    DS 30           ; 30 Bytes for Ringbuffer                �

�                                                                      �

�RSEG codeSeg                                                          �

� initRingBuf:                ; Initialisation routine                 �

�     MOV rdBufPtr, #ringBuf  ; read pointer on buffer start           �

�     MOV wrBufPtr, #ringBuf  ; write pointer on buffer start          �

�     RET                                                              �

�                                                                      �

� putCharToBuf:                             ; wrBufPtr shows the       �

�     XCH A, wrBufPtr                       ; location in buffer,      �

�     INC A                                 ; where it will be written �

�     CJNE A, #(ringBuf+30), noRingBufTop   ; to next.                 �

�     MOV A, #ringBuf                                                  �

� noRingBufTop:                             ; write without regard,    �

�     CJNE A, rdBufPtr, leaveRdBufPtr       ; as to whether reading    �

�     INC rdBufPtr                                                     �

�     CJNE rdBufPtr, #(ringBuf+30), leaveRdBufPtr                      �

�     MOV rdBufPtr, #ringBuf                ; has been completed.      �

� leaveRdBufPtr:                            ; If a character is not    �

�     XCH A, wrBufPtr                       ; read in time it will be  �

�     MOV @R0, A                            ; lost.                    �

�     RET                                                              �

�                                                                      �

� getCharFromBuf:                                                      �

�     MOV A, rdBufPtr                   ; when the read pointer is     �

�     CJNE A, wrBufPtr, getCharDo       ; equal to the write pointer,  �

�     SETB C                            ; the buffer is empty. Then RET�

�     RET                               ; with carry.                  �

� getCharDo:                                                           �

�     MOV R0, A                                                        �

�     MOV A, @R0                        ; After reading, increment the �

�     INC R0                            ; read pointer.                �

�     CJNE R0, #(ringBuf+30), leaveRdBufPtrR0                          �

�     MOV R0, #ringBuf                                                 �

� leaveRdBufPtrR0:                      ; The read pointer points to   �

�     MOV rdBufPtr, R0                  ; the next byte to be read.    �

�     CLR C                                                            �

�     RET                                                              �

�END                                                                   �

������������������������������������������������������������������������

5. Assembler                                                 Page 57

���������������������������������������������������������������������������

The above program is fully relocatable. It can be linked with another program

by the linker. The program can be used by routines which have been made ready

here to handle ringbuffers by invoking it through CALLs. Note however that

R0 which passes over the character in the accumulator, is destroyed and that

in a read the carry is set in the event the buffer is empty, and that as

such no character can be read.

In practice such routines are often used in connection with an interrupt

routine. For example with serial reception. In any case, in an interrupt

routine, firstly all used registers and flags must be saved and then the

received character is registered in a ringbuffer.

The procedure putCharToBuf is correspondingly to be completed. The reading of

the ringbuffer takes place through the main program.

5.5  ASM51 - Bugs:

------------------

The ASM51 has some peculiarities, be forewarned:

The ASM51 does not recognize PCON DATA 87h, although this is an SFR of

the 8051.

Several editors set asc26 as end of file code at the end of the

source code. ASM51 brings this up as an error, but continues correctly.

With such files the error message 2 ERRORS FOUND can be ignored.

When reading in characters from a file, ASM51 clears the most significant

bit of every byte. As such, asc8Dh is treated like asc0Dh. For that reason

values greater than asc7Fh cannot be used in strings (also not �, �, �, �).

asc8Dh = '�' (will also be confused with end of line inside a comment.)

The character asc25h = '%' is reserved in ASM51 for the identification

of MACROs. It may not be used in strings. ASM51 might even make an error,

when a '%' is placed in a comment.

The character ascA5h = '�' may also not be used.

Only the first 31 characters of variable names will be evaluated, extra

characters are ignored. This is not a bug, the programmer should only take

note.

6.1 Interrupt logic                                          Page 58

���������������������������������������������������������������������������

6.  Further hardware components of the 8051:

-------------------------------------------

6.1  Interrupt logic:

---------------------

The 8051 has 5 sources of interrupts. The interrupt logic incorporates the

registers IE (Interrupt Enable), IP (Interrupt Priority) and several flags

of TCON (Timer Control).

When the requirement for an interrupt is fulfilled and the interrupt is

enabled, then the CPU interrupts the current program, pushes the return

address onto the stack and then it will activate an interrupt

routine. However the flags will not be automatically saved (such as in an

8088). A fixed jump address is assigned to every interrupt source.

EXTI0   CODE  0003h     ; external interrupt input _INT0 (P3.2)

Timer0  CODE  000Bh     ; Timer 0 overflow

EXTI1   CODE  0013h     ; external interrupt input _INT1 (P3.3)

Timer1  CODE  001Bh     ; Timer 1 overflow

SInt    CODE  0023h     ; serial interrupt: Receive (RI) or Transmit (TI)

The interrupts can be disabled with software. They can only be generated

when the corresponding bit in the Interrupt Enable register is set.

Additionally, EA must be 1. The flags in the IE register can be bit addressed.

Consequently, all interrupts can be disabled with CLR EA. With SETB EA all

those that were set are enabled again. After a reset all interrupts are

disabled (IE = 0xx00000b).

�����������������������������������������Ŀ

� EA � _ � _ � ES � ET1 � EX1 � ET0 � EX0 �   IE DATA 0A8h

�������������������������������������������

EX0  BIT  0A8h       ; enable external INT0

ET0  BIT  0A9h       ; enable Timer0 interrupt

EX1  BIT  0AAh       ; enable extern INT1

ET1  BIT  0ABh       ; enable Timer1 interrupt

ES   BIT  0ACh       ; enable serial interrupt

EA   BIT  0AFh       ; enable/disable all set interrupts

The 8051 works with 2 interrupt priority levels. Through the Interrupt

Priority register (IP) a priority level may be assigned to every interrupt

source (1 = high Priority).

����������������������������������������Ŀ

� _ � _ � _ � PS � PT1 � PX1 � PT0 � PX0 �   IP DATA 0B8h

������������������������������������������

PX0  BIT  0B8h       ; external INT0 priority

PT0  BIT  0B9h       ; Timer0 interrupt priority

PX1  BIT  0BAh       ; external INT1 priority

PT1  BIT  0BBh       ; Timer1 interrupt priority

PS   BIT  0BCh       ; serial interrupt priority

6.1 Interrupt logic                                          Page 59

���������������������������������������������������������������������������

An interrupt in process cannot be disrupted by another interrupt of the same

or lower priority level. But an interrupt with higher priority

can do so with a lower interrupt. The 8051 takes note in an internal register

which interrupts are in process. There can be a maximum of 2 simultaneously,

since there are only 2 priority levels. With a RETI the highest interrupt is

the one to be executed.

When two interrupts of equal priority occur simultaneously, EXTI0 has

preference over Timer0, EXTI1, Timer1, SINT (this row-sequence within a

priority level).

In state 5 of every cycle (see 3.2 Oscillator and 3.3 instruction execution)

the interrupt requirements will be latched in the flags (see below). In the

following cycle these requirements will be examined (poll). If this is the

last cycle of an instruction (there are instructions which last 2 cycles) an

LCALL to the interrupt routine takes place. When this is not so, polling

is done one cycle later.

However an LCALL is not generated, when the interrupt is disabled, when

another interrupt is in process (priority?) or when the last instruction is a

RETI or there was an access of IE or IP. Sometimes this is the result, that

when an interrupt is in service, and another is activated, that after the

RETI, firstly at least one instruction of the main routine will be executed,

before a jump into the activated interrupt takes place.

Before every poll, the new interrupt requirements will be taken over anew in

state 5 of the previous cycle. The CPU will not jump on an active interrupt,

when it was blocked, and now removed.

The following flags serve as interrupt requests :

IE0    (in TCON)    external INT0 flag

TF0    (in TCON)    Timer 0 overflow flag

IE1    (in TCON)    external INT1 flag

TF1    (in TCON)    Timer 1 overflow flag

RI     (in SCON)    serial Receive full

TI     (in SCON)    serial Transmit empty

An interrupt request occurs when the corresponding flag is set. In a serial

interrupt one of these two suffices: RI or TI.

For that the software must check in the interrupt routine, which (RI or TI

or both) interrupt has to be cleared.

RI is set by the hardware, as soon as a character is received over the serial

port and is waiting in SBUF. TI is set by the hardware as soon as when sent,

SBUF is empty again. RI and TI must be reset again by software.

TF0 and TF1 will be set by the hardware, when a timer overflow past 0

takes place. By jumping into the interrupt routine TF0 and TF1 will be

automatically cleared by the hardware.

6.1 Interrupt logic                                          Page 60

���������������������������������������������������������������������������

The external interrupts have a special feature: they can be changed to be

"edge triggered" with flags IT0 or IT1 (in TCON).

With level triggering (IT0 = 0, rsp. IT1 = 0) the inverted value of the port

pins _INT0 = P3.2 rsp. _INT1 = P3.3 are placed into IE0 or IE1 in state 5

of every cycle. Level triggering therefore has no value to set or clear the

flags IE0 and IE1 by software. The flags always reflect the status of the

inputs.

With edge triggering the flags IE0 or IE1 will first be set by the hardware,

when a negative edge occurs at _INT0 or _INT1. By jumping into the interrupt

routine IE0 or IE1 will automatically be cleared by the hardware. But they

can also be changed by the software.

TCON contains different flags for controlling the Timer and for the Interrupt

logic. The flags can be bit addressed:

�����������������������������������������������Ŀ

� TF1 � TR1 � TF0 � TR0 � IE1 � IT1 � IE0 � IT0 �  TCON DATA 88h

�������������������������������������������������

IT0  BIT  88h       ; external INT0 are edge/_level triggered

IE0  BIT  89h       ; external INT0 Flag

IT1  BIT  8Ah       ; external INT1 are edge/_level triggered

IE1  BIT  8Bh       ; external INT1 Flag

TF0  BIT  8Dh       ; Timer 0 overflow Flag

TF1  BIT  8Fh       ; Timer 1 overflow Flag

TR0  BIT  8Ch       ; Timer 0 run

TR1  BIT  8Eh       ; Timer 1 run

6.2 8051 Timer                                               Page 61

���������������������������������������������������������������������������

6.2  8051 Timer:

----------------

The 8051 has two timer/counters: timer 0 and timer 1. Each timer consists

of two 8 bit registers. They can be set to 4 different running modes.

The timer will (when running) be incremented by 1 at every cycle. They can

also be used as a counter, and will be incremented depending on the count

input, by T0 or T1.

The mode is selected with the TMOD register. TMOD is not bit addressable.

The high nybble of TMOD selects the mode for Timer 1 and the low nybble for

Timer 0:

���������������������������������������������Ŀ

� GATE � C/T � M1 � M0 � GATE � C/T � M1 � M0 �  TMOD DATA 89h

�����������������������������������������������

C/T = 0     -->  Timer mode (increment by 1 every cycle)

C/T = 1     -->  Counter Mode (increment by 1 for every negative edge at

port pin T0 = P3.4 or T1 = P3.5)

GATE = 0    -->  Timer/Counter always running, when TRx = 1 (in TCON)

GATE = 1    -->  Timer/Counter runs only when TRx = 1 and _INTx = 1

the interrupt input can act as a gate for the Timer/

Counter.

M1:M0 = 00  -->  Mode 0:  13 bit Timer/Counter

The 8 bits from THx and the lowest 5 Bits from TLx serve

as counter.

M1:M0 = 01  -->  Mode 1:  16 Bit Timer/Counter

M1:M0 = 10  -->  Mode 2:  8 bit Timer/Counter with auto reload

TLx serves as counter. With every overflow the value

from THx will be loaded into TLx.

M1:M0 = 11  -->  Mode 3:  TL0 acts as an 8 bit timer/counter. TL0 uses

the usual control bits of Timer 0.

TH0 acts as another 8 bit timer/counter.

TH0 uses the control bits from Timer 1(_INT1 as Gate, T1

as counter input, TR1, TF1). Timer 1 is stationary when

it finds itself in mode 3.

When Timer 0 finds itself in Mode 3, TR1, T1 and _INT1 cannot control Timer

1 anymore. When Timer 1 overflows it does not set TF1 anymore and cannot

generate any more interrupts. Timer 1 can be stopped by switching it to Mode

3, otherwise it runs. Although no more control bits are assigned to Timer 1,

it may run when Timer 0 is in Mode 3, and is good for generating baud rate

for the serial port.

6.3 Serial Communications                                    Page 62

���������������������������������������������������������������������������

6.3 Serial Communications

----------------------------

The 8051 can send and receive either synchronously or asynchronously over

the serial port. In asynchronous mode it can send and receive simultaneously.

The 8051 has separate registers for sending and for receiving. The contents

are separate from each other, but both use the same DATA address (SBUF DATA

99h).

The following diagram shows a simplified version of the function of the

serial port in asynchronous mode.

|                       ���������������Ŀ

S |����������������������>� SBUF Tx-Shift �������������> TxD

o |                       �����������������

f |                        �����������Ŀ

t |           SysTime ���>��    � 16   �

w |           Timer 1 ���>������������Ĵ

a |           Timer 2 ���>�� DPLL � 16 �������Ŀ

r |                        �������������       �

e |      ���������Ŀ      ���������������Ŀ    �

|<����ĳ SBUF Rx ������Ĵ Rx-Shift Reg. �������������< RxD

|      �����������      �����������������

When a character is written to SBUF (transmit), the port starts shifting the

data out into TxD. When all the bits are sent, the logic sets the

TI flag (BIT 99h in SCON). The shift clock comes from an internal 16 bit-

counter.

Received bits are first shifted into the receive Shift Register. When

all bits are received, the character is taken into SBUF (receive) and RI is

set. The shift clock comes from a DPLL, which is synchronised with

the received data.

The following diagram shows a simplified version of the function of the

serial port in synchronous mode.

|                       ���������������Ŀ

S |����������������������>� SBUF Tx-Shift ����Ŀ

o |            �����Ŀ    �����������������    �

f | SysTimer��>� � 6 �����������Ĵ   send      �

t |            �������              o���������ĳ������<> TxD

w |                                / receive   �

a |                              �             �

r |      ���������Ŀ      ���������������Ŀ    �

e |<����ĳ SBUF Rx ������Ĵ Rx-Shift Reg. ������������<> RxD

|      �����������      �����������������

Send and receive shift functions the same way as in asynchronous mode. But

the data is sent and received at RxD. At TxD, the shift clock is sent or

synchronously received.

6.3 Serial Communications                                    Page 63

���������������������������������������������������������������������������

6.3.1  Asynchronous Mode, Synchronous Mode:

-------------------------------------------

In synchronous mode the shift clock is synchronized with the data

transferred. The receiver uses as such the shift clock from the sender. The

transmission distance is limited, as over longer distances the phase

angle between data and clock can diverge.

In asynchronous mode a clock is not transmitted. The clock must be recovered

from the edges of the received data bits. This is done through a 16 bit

Digital Phase Locked Loop (DPLL).  This is a 16 bit counter, which is

synchronised with the edges of the received data:

A negative edge at RxD causes the output of the DPLL to be set to 1.

It remains there for 8 DPLL cycles, then it goes to 0 and so on for 8

DPLL cycles. Every negative edge forces this synchronisation.

Over a Universal Asynchronous Receive Transmit port (UART), data is

transferred only on a character basis as defined in the case of 8051, of a PC

or of a terminal. Every character is led by a start bit (=0) and concluded by

1, 1� or 2 stopbits (=1). The start bit in combination with the stop bit

guarantees the identification of the start of a character upon reception and

the synchronisation of the DPLL.

The LSB (low bit) is always transmitted first. Sender and receiver must

agree in the number of stopbits. The baud rate should also agree, so that

during the transfer of a character the DPLL does not "run away" too far.

Example for transmitting the character 'A' = asc41h in asynchronous mode:

������Ŀ     �����Ŀ                             �����Ŀ     ����������

�������     �������������������������������     �������

|Start| LSB |     |     |     |     |     |     | MSB |Stop |

6.3.2  8051 serial mode setting:

--------------------------------

The 8051 can run at a maximum of 1 MBit/s in synchronous mode. The maximum is

375 kBit/s in asynchronous mode.

The serial port can operate in 4 different modes. The mode is set in  SCON

(Serial Control Register). SCON is bit addressable.

���������������������������������������������Ŀ

� SM0 � SM1 � SM2 � REN � TB8 � RB8 � TI � RI �  SCON DATA 98h

�����������������������������������������������

6.3 serial port                                            Page 64

���������������������������������������������������������������������������

SM0 (BIT 9Fh) and SM1 (BIT 9Eh) acts as mode setting.

SM2 (BIT 9Dh) enables special reception requirement.

REN (BIT 9Ch) is the Receiver Enable Flag.

TB8 (BIT 9Bh) is also the 9th bit sent.

In RB8 (BIT 9Ah) the 9th Bit is received.

TI (BIT 99h) is the Transmit Interrupt Flag.

RI (BIT 98h) is the Receive Interrupt Flag.

For the meaning and applications of these flags, see below.

Mode 0:  Synchronous Mode with 1/12 Oscillator frequency (max. 1 MBit/s):

-------------------------------------------------------------------------

SM0:SM1 = 00

8 data bits (LSB first) are sent and received over RxD. The shift clock is

is sent or received over TxD. It is not possible to receive and send

simultaneously.

The shift clock is derived by sending in association with the system clock.

The shift clock corresponds to the cycle frequency (= 1 MHz at 12 MHz

oscillator). When SBUF (transmit) is once again empty, TI will be set.

To accept characters the receiver must be enabled. For this the software has

to set REN = 1 and RI = 0. The character will be received into an internal

shift register. Only when this is full, will the character be moved to SBUF

and RI set.

In Mode 0, SM2 = 0 must be chosen, TB8 and RB8 have no meaning.

Particulars of Mode 0:

----------------------

The shift clock TxD is low during State 3, 4 and 5 of the system clock

(see chapter 3.2 and 3.3). With the positive edge in the shift clock, the

value changes at RxD and the next bit is shifted out.

After a new character to be sent is written to SBUF, a full cycle passes first

before the shifting starts. After the last bit (MSB) of the character has

been shifted out, another 1 will be shifted behind it (so that RxD remains

undisturbed at 1), only then will TI be set. 10 cycles will pass after the

write SBUF event by the software.

During reception, the positive edge at TxD shifts the data into the receive

shift register. For this time period SBUF can still hold the previous data.

When all the bits are received, then will the hardware overwrite it with the

new character. After which RI = 0 is set. For REN = 0 (receiver not enabled)

or RI = 1 (SBUF still full), incoming data will not be received and will be

lost.

6.3 Serial Communications                                    Page 65

���������������������������������������������������������������������������

Mode 1:  Asynchronous mode with variable baud rate as 8 Bit UART:

------------------------------------------------------------------

SM0:SM1 = 01

A clock will not be sent, only the data. What is sent over TxD is 1 start

bit, then 8 data bits (LSB first) and 1 stop bit. When SBUF (transmit) is

empty again, TI will be set.

Receiving takes place over RxD. The clock will be recovered in the receiver

from the edges of the data. To receive characters we must have REN = 1

and RI = 0 (Receive enable). In addition, if SM2 = 1, the received stop bit

must in addition = 1 making it valid. That character is received in an

internal shift register. Only when this is full, will the value in SBUF be

transferred and RI set. The stop bit is registered in RB8.

Particulars of Mode 1:

----------------------

Sending starts with the first overflow of the clock generator after a

character has been written to SBUF. First, a start bit of 0 is sent, then the

next 8 bits of the character (LSB first). Finally a 1 is sent as a stop bit.

Then will TI = 1.

The reception starts with the first negative edge at RxD. The start bit

is watched out for in states 7, 8 and 9 of the shift clock count. It must

always be 0 (for data integrity, to mask out disturbances). If this is not

the case, or REN not = 1, the reception will be cancelled and the logic goes

back to the initial state (wait for negative edge).

After recognizing the start bit and REN = 1, the incoming bits will be moved

into the receive shift register. At this point in time old data can still

remain in SBUF, RI need not be 1 yet. The reception is completed upon

receiving the stop bit. Only when the following conditions are met, will

the character be placed into SBUF, RI = 1 be set and the stop bit written

into RB8.

RI = 0   and   SM2 = 0

or  RI = 0   and   SM2 = 1   and   Stopbit = 1.

Mode 1 shift clock:

-------------------

The shift clock used in sending is derived from a 16 digit counter, triggered

from the Timer1 overflow. The shift clock for receiving is also derived from

a DPLL, which is also triggered from the Timer1 overflow. If the SMOD bit in

PCON is cleared (SMOD = 0), the Timer1 output will in the first place have

been further divided by 2. The shift clock is given by the following formula:

Osci       1         SMOD + 1    1        Osci * (SMOD + 1)

BaudRate = ���� * ���������� *  �������� * ����   =  ������������������

12    100h - TH1        2       16       384 * (100h - TH1)

�         �            �        ���� 16 bit-counter / DPLL

�         �            ������������ divide by 2

�         ������������������������ count value of Timer 1

��������������������������������� Trigger for timer per cycle

6.3 Serial port                                              Page 66

���������������������������������������������������������������������������

Alternatively, the 8052 can select the Timer2 overflow for triggering the

shift clock counter and/or the DPLL. If Timer2 is to be used for sending,

the bit TCLK in T2CON is set (see 8052 Timer). If Timer2 is to be used for

receiving, bit RCLK in T2CON is set. In this way sender and receiver can run

with different baud rates (one with Timer1 and the other with Timer2), but

they can also be the same: both from Timer1 or both from Timer2.

Mode 2:  Asynchronous mode with fixed baud rate for 9 Bit UART:

---------------------------------------------------------------

SM0:SM1 = 10

No clock will be sent, only data. What is sent over TxD is 1 start bit, then

8 data bits (LSB first) the bit from TB8 and 1 stop bit. When SBUF (transmit)

is empty again, TI will be set.

Receiving takes place over RxD. The clock will be recovered in the receiver

from the edges of the data. To receive characters we must have REN = 1

and RI = 0 (Receive enable). If SM2 = 1, so additionally must the received 9th

bit = 1. That character is received in an internal shift register only. Only

when this is full will the value in SBUF be transferred and RI is set. The

stop bit will be put into RB8.

Details of Mode 2:

------------------

Sending begins with the first overflow of the clock generator after a

character has been written to SBUF. First a startbit of 0 is sent, then the

next 8 bits of the character (LSB first). After that the bit from TB8 is sent

and finally a 1 as stop bit. Then will TI = 1.

The reception starts with the first negative edge at RxD. The start bit

will be watched out for in states 7, 8 and 9 of the shift clock count. It

must always be 0 (for data security, to mask out disturbances). If this is

not the case, or REN not = 1, the reception will be cancelled and the logic

goes back to the initial state (wait for negative edge).

After recognizing the start bit and REN = 1, the incoming bits will be

shifted into the receive shift register. At this time, old data can remain in

SBUF-RI need not be 1 yet. The reception is completed upon receiving the stop

bit. Only when the following existing conditions are met, will the character

be placed into SBUF, RI = 1 be set and the stop bit written into RB8.

RI = 0   and   SM2 = 0

or  RI = 0   and   SM2 = 1   and   9th Bit = 1.

6.3 Serial port                                              Page 67

���������������������������������������������������������������������������

Mode 2 baud rate:

-----------------

The baud rate for sending/receiving is derived from a 16 digit counter/DPLL,

triggered by the system clock (= � oscillator clock, max 6 MHz at 12 MHz

crystal). When the bit SMOD in PCON is cleared (SMOD=0), Timer1 will

additionally be divided by 2.

The baud rate is given by the following formula:

Osci    SMOD + 1    1           Osci * (SMOD + 1)

BaudRate = ���� *  �������� * ����    =    ������������������

2         2        16                 64

�         �         ���� 16 bit counter / DPLL

�         ������������� divide by 2

���������������������� System clock

Mode 3:  Asynchronous mode with variable baud rate as 9 Bit UART:

-----------------------------------------------------------------

SM0:SM1 = 11

Transmission takes place as in mode 2, but the baud rate is derived as in

Mode 1.

In Modes 2 and 3 a field bus system can be built with the help of RB8, TB8 and

SM2. Several 8051 CPUs can be driven through a bus line. A suitable protocol

(e.g. master-slave) is necessary. A node, which has set SM2 = 0, receives all

characters sent over the bus . A node, with SM2 set = 1, receives only such

characters which are sent with TB8 = 1. So two nodes can communicate

together, without disturbing the others.

6.3.3  Interrupt Mode or Polled Mode:

-------------------------------------

When sending over the serial port, the next character can only be written into

SBUF after the old character is gone. As soon as SBUF (transmit) is empty, the

hardware sets the TI flag. The user can write the next character into SBUF

only when TI = 1.

As soon as a character is received over the serial port, the hardware sets

the RI flag. Only when RI = 1, is a valid character present in SBUF (receive).

The hardware sets only RI and TI, but does not clear them. However after a

reset, RI, TI as well as REN (SCON = 0) are cleared.

6.3 Serial Communications                                    Page 68

���������������������������������������������������������������������������

Polled Mode:

------------

For correct functioning of the serial port, the software must observe the

following :

- only write to SBUF when TI is 1.

- after writing to SBUF then clear TI.

- before reading from SBUF, check RI.

- after reading from SBUF, clear RI.

Proceeding in this way, the following problem occurs: After a reset TI is 0

although SBUF (transmit) is empty. The hardware never sets it, since there is

no sending, and the end of transmission status never appears. The first

character to be sent may as such be written to SBUF, although TI is 0. It is

advisable to set TI to 1 soon after a reset.

Example for serial transmission in Polled Mode:

-----------------------------------------------

iniSCON:

CLR   ES              ; serial interrupt cleared

CLR   ET1             ; Timer 1 interrupt cleared

MOV   TH1, #-3        ; preload value for Timer 1 = -3

MOV   TL1, TH1

SETB  TR1             ; start Timer 1

ANL   PCON, #7Fh      ; SMOD = 0

MOV   SCON, #52h      ; Mode 1, SM2=0,TB8=0,RB8=0, TI=1, REN=1,RI=0

RET

sendeChar:

JNB  TI, sendeChar    ; wait until SBUF (transmit) is empty

MOV  SBUF, A

CLR  TI

RET

receiveChar:

JNB  RI, receiveChar

MOV  A, SBUF

CLR RI

RET

after a reset the serial port is initialised for 8 bit UART at 9600 baud

(with 11,059 MHz crystal) as above. The receiver is enabled and TI=1 is set.

Sending will wait until SBUF is empty. Then the character which is called up

by the subprogram is written into SBUF from the accumulator. TI must be

cleared by the software.

In the course of receiving, a wait also happens, until a character is

received. It is OK for the CPU not to be doing anything. Since the program

remains here in a loop, when no characters are received.

6.3 Serial Communications                                  Page 69

���������������������������������������������������������������������������

Interrupt Mode:

---------------

With larger, more comprehensive programs, if the CPU must be processing other

routines, often the serial receiver can only be polled at certain particular

times. If polling is not done often enough, data can be lost. Sometimes it is

also not permitted to wait until a character can be sent. The solution is to

have the serial port running in interrupt mode.

With a set RI or TI flag, the hardware will generate an interrupt.  For that,

the interrupt must be enabled (bit ES = 1). By jumping into the interrupt

routine (at CODE 0023h) RI and TI will not be cleared by the hardware. They

are to be cleared inside the interrupt routine by the software, otherwise

after the RETI the interrupt routine will again be activated.

Under normal conditions the send buffer is empty. When working with interrupts

TI cannot remain unchanged. As for code for send buffer empty, an extra flag

must be defined.

Example for serial transmission in interrupt mode:

----------------------------------------------------

TxEmpty BIT 0                ; as an example, the serial port

; is driven in asynchronous mode (Mode 2).

iniSCON:                     ; The reception takes place thru the interrupt.

SETB  TxEmpty           ; Send in polled mode. (TxEmpty flag)

ORL   PCON, #80h        ; SMOD = 1, so that 375 kBit/s at 12 MHz crystal

MOV   SCON, #98h        ; SM2=0, REN=1 and RI=0 enables receive

SETB  ES                ; TI is cleared, in its stead the flag

SETB  EA                ; TxEmpty is set.

RET                     ; The interrupt routine reads the character

; from SBUF and stores it in a ring buffer,

CSEG AT 0023h                ; from where a main program retrieves it.

JNB  RI, chkTxInt       ; This digit might cause a jump to a

PUSH PSW                ; command dispatch or similar routine.

PUSH ACC                ; An example for a character

MOV  A, R0              ; insert routine (Ringbuffer) is found

PUSH ACC                ; in section 5.4.

MOV  A, SBUF            ; In any case, the interrupt routine must

CLR  RI                 ; save the used registers and the flags

CALL putCharToBuf       ; (here ACC, R0 and PSW).

POP  ACC                ; If TI is set, TxEmpty is set. TI will as

MOV  R0, A              ; such be cleared. The send routine polls

POP  ACC                ; TxEmpty instead of TI.

POP  PSW                ; Caution: Care is requested, when the send

chkTxInt:                    ; routine is part of an interrupt routine.

JNB  TI, sintRet        ; Then both TI and TxEmpty are to be polled,

SETB TxEmpty            ; since no interrupt comes through and so

CLR  TI                 ; TxEmpty will not be set.

sintRet:

RETI

6.3 Serial Communications                                  Page 70

���������������������������������������������������������������������������

6.3.4  Baud Rate Table:

-----------------------

This table contains the control values used to obtain a specific baud rate.

For the equation see section 6.3.2

CrystalSM0:SM1 SMOD  TH1    BaudRate

-------------------------------------        With a 12 MHz crystal most of

12 MHz   00   X      X       1 MBit/s        the standard baud rates cannot

12 MHz   10   1      X     375 kBit/s        be set exactly.

12 MHz   10   0      X   187.5 kBit/s

12 MHz   01   1     -1   62 500 Bit/s

12 MHz   01   1     -3   20 800 Bit/s        Since the DPLL holds the phase

12 MHz   01   1     -7    8 929 Bit/s        over 10 bit cells at the most,

12 MHz   01   1    -14    4 808 Bit/s        it is enough that the baud rates

12 MHz   01   1    -26    2 404 Bit/s        are accurate up to 4800 baud.

12 MHz   01   1    -52    1 202 Bit/s

12 MHz   01   1   -104      601 Bit/s

12 MHz   01   1   -208      300 Bit/s

12 MHz   01   0   -208      150 Bit/s        BR in Mode 01 and 11 are the same

Crystal    SM0:SM1 SMOD  TH1    BaudRate

-----------------------------------------     With a special crystal, baud

11.0592 MHz   01   1     -1   57 600 Bit/s    rates up to 19200 baud

11.0592 MHz   01   1     -3   19 200 Bit/s    can be set exactly.

11.0592 MHz   01   1     -6    9 600 Bit/s    But now the CPU does not

11.0592 MHz   01   1    -12    4 800 Bit/s    work at the maximal frequency

11.0592 MHz   01   1    -24    2 400 Bit/s    and the time base which otherwise

11.0592 MHz   01   1    -48    1 200 Bit/s    was 1 �s for a cycle is lost.

11.0592 MHz   01   1    -96      600 Bit/s

11.0592 MHz   01   1   -192      300 Bit/s

11.0592 MHz   01   0   -192      150 Bit/s

Baud rates lower than 150 bit/s (with these crystals) are not obtainable. It

is possible to feed T1 with a clock and operate Timer 1 as a counter. With it

the following baud rate is given:

Count * (SMOD + 1)

BaudRate =   �������������������

32 * (100h - TH1)

In order to obtain lower baud rates a further possibility exists with Timer 1

operating as a 16 bit timer. In addition, a Timer 1 interrupt routine is

installed, which writes a 16 bit reload value into TH1:TL1.

The 8052 permits more exact baud rates to be set with Timer 2 (see chapter

7.1).

7.1 The 8052                                                 Page 71

���������������������������������������������������������������������������

7.  Further controllers of the 8051 family:

------------------------------------------

7.1 The 8052:

-------------

Contrary to the 8051, the 8052 possesses a larger internal RAM, a larger

internal ROM and an additional timer: otherwise it is identical with the 8051.

internal RAM 8051:    128 Byte    00....7Fh

internal RAM 8052:    256 Byte    00...0FFh

internal ROM 8051:    4 kByte    0000...0FFFh      ; 8031 and 8032 do not

internal ROM 8052:    8 kByte    0000...1FFFh      ; have internal ROM

; 8751 and 8752 possess

; an internal EPROM

internal RAM:

-------------

In a stack overflow (SP over 0FFh), the SP will be further incremented.

This is shown at the internal RAM addresses from 00. In this way the stack

overwrites the register area. Thus, care should be taken that no stack

overflow occurs.

The internal RAM addresses 80h...FFh cannot be directly addressed. This

region can only be accessed indirectly (MOV @Ri, ..).

(compare also chapter 3.9)

7.1.2  additional SFR of the 8052:

----------------------------------

In the case of the 8052, several additional SFRs exist for control of Timer2:

T2CON    DATA  0C8h

RCAP2L   DATA  0CAh

RCAP2H   DATA  0CBh

TL2      DATA  0CCh

TH2      DATA  0CDh

7.1.3  Timer 2:

---------------

Timer2 is a 16 bit counter. Its counter register is made up of TH2:TL2

(Timer2 High and Timer2 Low byte). T2CON (Timer2 Control) serves to control

the timers/counters.

7.1 The 8052                                                 Page 72

���������������������������������������������������������������������������

An interrupt can be generated by a counter overflow. The jump address is at

002Bh. In addition the following bits in the interrupt enable and interrupt

priority register are defined:

ET2     BIT  0ADh

PT2     BIT  0BDh

Timer2  CODE 002Bh

Timer2 runs as a 16 bit counter/timer selectively with reload or capture

functions. Besides, Timer2 has a capture/reload register (RCAP2H:RCAP2L) in

addition to a 16 bit count register (TH2:TL2).

In the reload mode, an overflow or external trigger to Port1 input pin T2EX

will cause the value from RCAP2 to be loaded into the count register.

In capture mode an external trigger will cause the value in the count register

to be loaded into RCAP2.

When Timer2 operates as a timer, TH2:TL2 will be incremented by 1 every CPU

cycle (except as a baud rate generator: see RCLK, TCLK). When operating as

a counter it will increment on every negative edge signal at Port1 input pin T2.

T2EX  BIT  091h                 ; = P1.1

T2    BIT  090h                 ; = P1.0

��������������������������������������������������������Ŀ

� TF2 � EXF2 � RCLK � TCLK � EXEN2 � TR2 � C_T2 � CP_RL2 � T2CON DATA 0C8h

����������������������������������������������������������

CP_RL2 BIT 0C8h    -->  With CP_RL2 = 1 the capture mode is selected,

otherwise the reload mode. Reload mode will also be selected when Timer2 is

used as a baud rate generator for the serial port. (see also EXEN2, RCLK and

TCLK).

C_T2 BIT 0C9h   -->  With C_T2 = 1 Timer2 will operate as a counter, or else

as a timer.

TR2 BIT 0CAh   -->  Timer2 Run Flag

EXEN2 BIT 0CBh   -->  The external trigger input pin T2EX is only active with

EXEN2 = 1. Only then, and with a negative edge at T2EX, will a transfer take

place in RCAP2 (in Capture Mode) or a Reload (in Reload Mode).

TCLK BIT 0CCh   -->  When TCLK is set, the Timer2 overflow acts as a trigger for

the baud rate generator for the serial port when sending.

7.1 The 8052                                                 Page 73

���������������������������������������������������������������������������

RCLK BIT 0CDh   -->  When RCLK is set, the Timer2 overflow acts as a trigger

for the baud rate generator (DPLL) for the serial port when receiving.

EXF2 BIT 0CEh   -->  External trigger interrupt flag. EXF2 will be set by

the hardware, after an overflow has occurred due to there being a negative

edge at T2EX (only when EXEN2 = 1). EXF2 must be cleared by software.

TF2 BIT 0CFh   -->  Timer2 overflow interrupt flag. TF2 will be set by the

hardware after an overflow from 0FFFFh to 0000 in TH2:TL2 occurs. TF2 must be

cleared by software.

Timer2 Interrupt:

-----------------

Through Timer2 an interrupt is processed, when TF2 or EXF2 are set. It is

enough that either one is set. The CPU saves the return address to the

stack and jumps to CODE address 002Bh. While jumping into the interrupt routine

TF2 and EXF2 are preserved, for only so can the interrupt routine detect what

causes the interrupt:

TF2    -->  Timer2 overflow

or     EXT2   -->  external trigger and transfer

TF2 and EXF2 must be cleared by software (in the interrupt routine). Otherwise

after a RETI a renewed jump into the interrupt routine takes place .

TF2 and/or EXF2 may be cleared or set with software. If one of the two, EXF2

or TF2 is not used, it can be misused to produce a software interrupt.

7.1.4  Serial port with Timer2:

-----------------------------------------

If either TCLK and RCLK is set, Timer2 will act as a trigger for at least

one of the baud rate generators (send, receive or both) of the serial port.

Timer2 runs in the reload mode (independent of CP_RL2). In this mode, Timer2

will not be incremented every CPU cycle, rather once every system clock

(crystal frequency / 2). In this way it runs faster.

The prerequisite for the generation of the serial baud rates with Timer 2

is that the serial port must be running in mode 1 (SM0:SM1 = 01) or Mode 3

(SM0:SM1 = 11). By default the Timer1 overflow is used to trigger the

baud rate generators. When TCLK or/and RCLK is set, the Timer2 overflow will

be used for sending or receiving.

TCLK = 0  RCLK = 0   -->  Timer1 for sending and receiving

TCLK = 0  RCLK = 1   -->  Timer1 for sending and Timer2 for receiving

TCLK = 1  RCLK = 0   -->  Timer2 for sending and Timer1 for receiving

TCLK = 1  RCLK = 1   -->  Timer2 for sending and receiving.

7.1 The 8052                                                 Page 74

���������������������������������������������������������������������������

When using Timer2 the following gives the baud rate :

Osci          1          1

BaudRate = ���� * �������������� * ����             ; RCAP2 = RCAP2H:RCAP2L

2     10000h - RCAP2    16

�           �           �� 16 bit counter / DPLL

�           ���������������� numeric value of Timer 2

������������������������������ trigger for timer every system clock

Baud rate table:

----------------

This table contains the control values for obtaining a given baud rate.

Compare also chapter 6.3.4. Here SM0:SM1 = 01 or 11.

Crustal: RCAP2     BaudRate

------------------------------

12 MHz      -1   375    kBit/s      With a 12 MHz crystal not all standard

12 MHz      -2   187,5  kBit/s      baud rates can here be set exactly

12 MHz      -3   125    kBit/s      either

12 MHz      -4    93750  Bit/s

12 MHz      -5    75000  Bit/s

12 MHz      -6    62500  Bit/s       11,0592 MHz     -3   115200  Bit/s

12 MHz      -7    53571  Bit/s       11,0592 MHz     -6    57600  Bit/s

12 MHz      -9    41667  Bit/s

12 MHz     -10    37500  Bit/s       11,0592 MHz     -9    38400  Bit/s

12 MHz     -20    18750  Bit/s       11,0592 MHz    -18    19200  Bit/s

12 MHz     -39     9615  Bit/s       11,0592 MHz    -36     9600  Bit/s

12 MHz     -78     4808  Bit/s       11,0592 MHz    -72     4800  Bit/s

12 MHz    -156     2404  Bit/s       11,0592 MHz   -144     2400  Bit/s

12 MHz    -312     1202  Bit/s       11,0592 MHz   -288     1200  Bit/s

12 MHz    -625      600  Bit/s

12 MHz   -1250      300  Bit/s

12 MHz   -2500      150  Bit/s

12 MHz   -3409      110  Bit/s

12 MHz   -5000       75  Bit/s

Adjustable by PC:

115200, 57600, 38400, 28800, 23040, 19200, 16457,

A negative number can be used in an assembler. But it can also be added up:

with -1 = 0FFFFh, -2 = 0FFFEh ....

Example for 1200 baud with Timer2:

CLR   ES                     ; clear serial interrupt

CLR   ET2                    ; clear Timer 2 interrupt

MOV   RCAP2H, #HIGH(-312)    ; preload value for Timer 2

MOV   RCAP2L, #LOW(-312)

MOV   TH2, RCAP2H

MOV   TL2, RCAP2L

MOV   T2CON, #00110100b      ; RCLK, TCLK, TR2

MOV   SCON, #52h      ; Mode 1, SM2=0,TB8=0,RB8=0, TI=1, REN=1,RI=0

A.1 ASCII Table:                                             Page 75

���������������������������������������������������������������������������

A.1  ASCII Table:

-----------------

The following page shows the assignment of characters to numeric values

(ASCII table):

The values less than 20h (blank characters) in general are not printable, and

also many editors and text processing systems do not display them. There and

in data transmission they often act as control characters.

The important ones are:

BEL = Ring character (bell)

BS  = backward delete (Back Space)

HT  = horizontal TAB

VT  = vertical Tab

CR  = at the beginning of lines (Carriage Return)

LF  = Line advance (Line Feed)

FF  = new page (Form Feed)

Although a graphic character is also assigned to the characters under 20h, in

this table however, the character is not represented, rather the meaning for

data transmission is indicated (since there is problem anyway of viewing the

representation on many editors).

A.1 ASCII table                                              Page 76

���������������������������������������������������������������������������

NUL  0  00      BEL  7  07      SO  14  0E      NAK 21  15      FS  28  1C

SOH  1  01      BS   8  08      SI  15  0F      SYN 22  16      GS  29  1D

STX  2  02      HT   9  09      DLE 16  10      ETB 23  17      RS  30  1E

ETX  3  03      LF  10  0A      DC1 17  11      CAN 24  18      US  31  1F

EOT  4  04      VT  11  0B      DC2 18  12      EM  25  19

ENQ  5  05      FF  12  0C      DC3 19  13      SUB 26  1A

ACK  6  06      CR  13  0D      DC4 20  14      ESC 27  1B

32  20      0   48  30      @   64  40      P   80  50      `   96  60

!   33  21      1   49  31      A   65  41      Q   81  51      a   97  61

"   34  22      2   50  32      B   66  42      R   82  52      b   98  62

#   35  23      3   51  33      C   67  43      S   83  53      c   99  63

$   36  24      4   52  34      D   68  44      T   84  54      d  100  64

%   37  25      5   53  35      E   69  45      U   85  55      e  101  65

&   38  26      6   54  36      F   70  46      V   86  56      f  102  66

'   39  27      7   55  37      G   71  47      W   87  57      g  103  67

(   40  28      8   56  38      H   72  48      X   88  58      h  104  68

)   41  29      9   57  39      I   73  49      Y   89  59      i  105  69

*   42  2A      :   58  3A      J   74  4A      Z   90  5A      j  106  6A

+   43  2B      ;   59  3B      K   75  4B      [   91  5B      k  107  6B

,   44  2C      <   60  3C      L   76  4C      \   92  5C      l  108  6C

-   45  2D      =   61  3D      M   77  4D      ]   93  5D      m  109  6D

.   46  2E      >   62  3E      N   78  4E      ^   94  5E      n  110  6E

/   47  2F      ?   63  3F      O   79  4F      _   95  5F      o  111  6F

p  112  70      �  128  80      �  144  90      �  160  A0      �  176  B0

q  113  71      �  129  81      �  145  91      �  161  A1      �  177  B1

r  114  72      �  130  82      �  146  92      �  162  A2      �  178  B2

s  115  73      �  131  83      �  147  93      �  163  A3      �  179  B3

t  116  74      �  132  84      �  148  94      �  164  A4      �  180  B4

u  117  75      �  133  85      �  149  95      �  165  A5      �  181  B5

v  118  76      �  134  86      �  150  96      �  166  A6      �  182  B6

w  119  77      �  135  87      �  151  97      �  167  A7      �  183  B7

x  120  78      �  136  88      �  152  98      �  168  A8      �  184  B8

y  121  79      �  137  89      �  153  99      �  169  A9      �  185  B9

z  122  7A      �  138  8A      �  154  9A      �  170  AA      �  186  BA

{  123  7B      �  139  8B      �  155  9B      �  171  AB      �  187  BB

|  124  7C      �  140  8C      �  156  9C      �  172  AC      �  188  BC

}  125  7D      �  141  8D      �  157  9D      �  173  AD      �  189  BD

~  126  7E      �  142  8E      �  158  9E      �  174  AE      �  190  BE

127  7F      �  143  8F      �  159  9F      �  175  AF      �  191  BF

�  192  C0      �  208  D0      �  224  E0      �  240  F0

�  193  C1      �  209  D1      �  225  E1      �  241  F1   ASCII Table

�  194  C2      �  210  D2      �  226  E2      �  242  F2   -----------

�  199  C7      �  215  D7      �  231  E7      �  247  F7

�  200  C8      �  216  D8      �  232  E8      �  248  F8

�  201  C9      �  217  D9      �  233  E9      �  249  F9

�  202  CA      �  218  DA      �  234  EA      �  250  FA

�  203  CB      �  219  DB      �  235  EB      �  251  FB

�  204  CC      �  220  DC      �  236  EC      �  252  FC

�  205  CD      �  221  DD      �  237  ED      �  253  FD

�  206  CE      �  222  DE      �  238  EE      �  254  FE

�  207  CF      �  223  DF      �  239  EF         255  FF

A.2 Instruction set                                            Page 77

���������������������������������������������������������������������������

A.2  Instruction set in hexadecimal row order:

----------------------------------------------

Code  Bytes  Cycles   Mnemonic

00   1  1  NOP                      30   3  2  JNB   bitAdr, codeAdr

01   2  2  AJMP  codeaddr           31   2  2  ACALL codeAdr

02   3  2  LJMP  codeaddr           32   1  2  RETI

03   1  1  RR    A                  33   1  1  RLC   A

04   1  1  INC   A                  34   2  1  ADDC  A, #wert

05   2  1  INC   dataAddr           35   2  1  ADDC  A, dataAdr

06   1  1  INC   @R0                36   1  1  ADDC  A, @R0

07   1  1  INC   @R1                37   1  1  ADDC  A, @R1

08   1  1  INC   R0                 38   1  1  ADDC  A, R0

09   1  1  INC   R1                 39   1  1  ADDC  A, R1

0A   1  1  INC   R2                 3A   1  1  ADDC  A, R2

0B   1  1  INC   R3                 3B   1  1  ADDC  A, R3

0C   1  1  INC   R4                 3C   1  1  ADDC  A, R4

0D   1  1  INC   R5                 3D   1  1  ADDC  A, R5

0E   1  1  INC   R6                 3E   1  1  ADDC  A, R6

0F   1  1  INC   R7                 3F   1  1  ADDC  A, R7

10   3  2  JBC   bitAdr, codeAddr   40   2  2  JC    codeAdr

11   2  2  ACALL codeAddr           41   2  2  AJMP  codeAdr

12   3  2  LCALL codeAddr           42   2  1  ORL   dataAdr, A

13   1  1  RRC   A                  43   3  2  ORL   dataAdr, #value

14   1  1  DEC   A                  44   2  1  ORL   A, #wert

15   2  1  DEC   dataAddr           45   2  1  ORL   A, dataAdr

16   1  1  DEC   @R0                46   1  1  ORL   A, @R0

17   1  1  DEC   @R1                47   1  1  ORL   A, @R1

18   1  1  DEC   R0                 48   1  1  ORL   A, R0

19   1  1  DEC   R1                 49   1  1  ORL   A, R1

1A   1  1  DEC   R2                 4A   1  1  ORL   A, R2

1B   1  1  DEC   R3                 4B   1  1  ORL   A, R3

1C   1  1  DEC   R4                 4C   1  1  ORL   A, R4

1D   1  1  DEC   R5                 4D   1  1  ORL   A, R5

1E   1  1  DEC   R6                 4E   1  1  ORL   A, R6

1F   1  1  DEC   R7                 4F   1  1  ORL   A, R7

20   3  2  JB    bitAdr, codeAddr   50   2  2  JNC   codeAdr

21   2  2  AJMP  codeAddr           51   2  2  ACALL codeAdr

22   1  2  RET                      52   2  1  ANL   dataAdr, A

23   1  1  RL    A                  53   3  2  ANL   dataAdr, #value

24   2  1  ADD   A, #value          54   2  1  ANL   A, #value

25   2  1  ADD   A, dataAddr        55   2  1  ANL   A, dataAdr

26   1  1  ADD   A, @R0             56   1  1  ANL   A, @R0

27   1  1  ADD   A, @R1             57   1  1  ANL   A, @R1

28   1  1  ADD   A, R0              58   1  1  ANL   A, R0

29   1  1  ADD   A, R1              59   1  1  ANL   A, R1

2A   1  1  ADD   A, R2              5A   1  1  ANL   A, R2

2B   1  1  ADD   A, R3              5B   1  1  ANL   A, R3

2C   1  1  ADD   A, R4              5C   1  1  ANL   A, R4

2D   1  1  ADD   A, R5              5D   1  1  ANL   A, R5

2E   1  1  ADD   A, R6              5E   1  1  ANL   A, R6

2F   1  1  ADD   A, R7              5F   1  1  ANL   A, R7

A.2 Instruction set                                          Page 78

���������������������������������������������������������������������������

Code  Bytes  Cycles   Mnemonic

60   2  2  JZ    codeAddr           90   3  2  MOV   DPTR, #value16

61   2  2  AJMP  codeAddr           91   2  2  ACALL codeAdr

62   2  1  XRL   dataAdr, A         92   2  2  MOV   bitAdr, C

63   3  2  XRL   dataAdr, #value    93   1  2  MOVC  A, @A+DPTR

64   2  1  XRL   A, #value          94   2  1  SUBB  A, #value

65   2  1  XRL   A, dataAddr        95   2  1  SUBB  A, dataAddr

66   1  1  XRL   A, @R0             96   1  1  SUBB  A, @R0

67   1  1  XRL   A, @R1             97   1  1  SUBB  A, @R1

68   1  1  XRL   A, R0              98   1  1  SUBB  A, R0

69   1  1  XRL   A, R1              99   1  1  SUBB  A, R1

6A   1  1  XRL   A, R2              9A   1  1  SUBB  A, R2

6B   1  1  XRL   A, R3              9B   1  1  SUBB  A, R3

6C   1  1  XRL   A, R4              9C   1  1  SUBB  A, R4

6D   1  1  XRL   A, R5              9D   1  1  SUBB  A, R5

6E   1  1  XRL   A, R6              9E   1  1  SUBB  A, R6

6F   1  1  XRL   A, R7              9F   1  1  SUBB  A, R7

70   2  2  JNZ   codeAddr           A0   2  2  ORL   C, /bitAddr

71   2  2  ACALL codeAddr           A1   2  2  AJMP  codeAddr

72   2  2  ORL   C, bitAddr         A2   2  1  MOV   C, bitAddr

73   1  2  JMP   @A+DPTR            A3   1  2  INC   DPTR

74   2  1  MOV   A, #value          A4   1  4  MUL   AB

75   3  2  MOV   dataAddr, #value   A5   -  -  reserved

76   2  1  MOV   @R0, #value        A6   2  2  MOV   @R0, dataAddr

77   2  1  MOV   @R1, #value        A7   2  2  MOV   @R1, dataAddr

78   2  1  MOV   R0, #value         A8   2  2  MOV   R0, dataAddr

79   2  1  MOV   R1, #value         A9   2  2  MOV   R1, dataAddr

7A   2  1  MOV   R2, #value         AA   2  2  MOV   R2, dataAddr

7B   2  1  MOV   R3, #value         AB   2  2  MOV   R3, dataAddr

7C   2  1  MOV   R4, #value         AC   2  2  MOV   R4, dataAddr

7D   2  1  MOV   R5, #value         AD   2  2  MOV   R5, dataAddr

7E   2  1  MOV   R6, #value         AE   2  2  MOV   R6, dataAddr

7F   2  1  MOV   R7, #value         AF   2  2  MOV   R7, dataAddr

80   2  2  SJMP  codeAddr           B0   2  2  ANL   C, /bitAddr

81   2  2  AJMP  codeAddr           B1   2  2  ACALL codeAddr

82   2  2  ANL   C, bitAddr         B2   2  1  CPL   bitAddr

83   1  2  MOVC  A, @A+PC           B3   1  1  CPL   C

84   1  4  DIV   AB                 B4   3  2  CJNE  A, #value, codeAddr

85   3  2  MOV   dataAddr, dataAddr B5   3  2  CJNE  A, dataAddr, codeAddr

86   2  2  MOV   dataAddr, @R0      B6   3  2  CJNE  @R0, #value, codeAddr

87   2  2  MOV   dataAddr, @R1      B7   3  2  CJNE  @R1, #value, codeAddr

88   2  2  MOV   dataAddr, R0       B8   3  2  CJNE  R0, #value, codeAddr

89   2  2  MOV   dataAddr, R1       B9   3  2  CJNE  R1, #value, codeAddr

8A   2  2  MOV   dataAddr, R2       BA   3  2  CJNE  R2, #value, codeAddr

8B   2  2  MOV   dataAddr, R3       BB   3  2  CJNE  R3, #value, codeAddr

8C   2  2  MOV   dataAddr, R4       BC   3  2  CJNE  R4, #value, codeAddr

8D   2  2  MOV   dataAddr, R5       BD   3  2  CJNE  R5, #value, codeAddr

8E   2  2  MOV   dataAddr, R6       BE   3  2  CJNE  R6, #value, codeAddr

8F   2  2  MOV   dataAddr, R7       BF   3  2  CJNE  R7, #value, codeAddr

A.2 Instruction set                                          Page 79

���������������������������������������������������������������������������

Code  Bytes  Cycles   Mnemonic

C0   2  2  PUSH  dataAddr           E0   1  2  MOVX  A, @DPTR

C1   2  2  AJMP  codeAddr           E1   2  2  AJMP  codeAddr

C2   2  1  CLR   bit Addr           E2   1  2  MOVX  A, @R0

C3   1  1  CLR   C                  E3   1  2  MOVX  A, @R1

C4   1  1  SWAP  A                  E4   1  1  CLR   A

C5   2  1  XCH   A, data Addr       E5   2  1  MOV   A, data Addr

C6   1  1  XCH   A, @R0             E6   1  1  MOV   A, @R0

C7   1  1  XCH   A, @R1             E7   1  1  MOV   A, @R1

C8   1  1  XCH   A, R0              E8   1  1  MOV   A, R0

C9   1  1  XCH   A, R1              E9   1  1  MOV   A, R1

CA   1  1  XCH   A, R2              EA   1  1  MOV   A, R2

CB   1  1  XCH   A, R3              EB   1  1  MOV   A, R3

CC   1  1  XCH   A, R4              EC   1  1  MOV   A, R4

CD   1  1  XCH   A, R5              ED   1  1  MOV   A, R5

CE   1  1  XCH   A, R6              EE   1  1  MOV   A, R6

CF   1  1  XCH   A, R7              EF   1  1  MOV   A, R7

D0   2  2  POP   dataAddr           F0   1  2  MOVX  @DPTR, A

D1   2  2  ACALL codeAddr           F1   2  2  ACALL codeAddr

D2   2  1  SETB  bitAddr            F2   1  2  MOVX  @R0, A

D3   1  1  SETB  C                  F3   1  2  MOVX  @R1, A

D4   1  1  DA    A                  F4   1  1  CPL   A

D5   2  2  DJNZ  dataAddr, codeAddr F5   2  1  MOV   dataAddr, A

D6   1  1  XCHD  A, @R0             F6   1  1  MOV   @R0, A

D7   1  1  XCHD  A, @R1             F7   1  1  MOV   @R1, A

D8   2  2  DJNZ  R0, code Addr      F8   1  1  MOV   R0, A

D9   2  2  DJNZ  R1, codeAddr       F9   1  1  MOV   R1, A

DA   2  2  DJNZ  R2, codeAddr       FA   1  1  MOV   R2, A

DB   2  2  DJNZ  R3, codeAddr       FB   1  1  MOV   R3, A

DC   2  2  DJNZ  R4, codeAddr       FC   1  1  MOV   R4, A

DD   2  2  DJNZ  R5, codeAddr       FD   1  1  MOV   R5, A

DE   2  2  DJNZ  R6, codeAddr       FE   1  1  MOV   R6, A

DF   2  2  DJNZ  R7, codeAddr       FF   1  1  MOV   R7, A