Fault Tolerance Through Redundant Arrays of Inexpensive Disks (RAID)

By

Lucio Gayosso

CMGT/579

Michelle Woolfolk

09/04/06

Contents

Contents	2

Fault Tolerance Through Redundant Arrays of Inexpensive Disks (RAID)	3

RAID Configurations	3

Advantages and Benefits	4

Inexpensive FT Implementation	4

Data Protection	4

Unaffected Productivity during Failures	4

Simple and Quick Configuration	4

IT Staff’s Convenience	4

High Availability for RAID Implementation on New Equipment	5

Trustworthy and Renowned Manufacturers	5

Disadvantages and Limitations	5

System Performance Degradation	5

High Costs on High FT without Performance Degradation	5

Stages with Data Vulnerability on certain RAID Levels	6

Potential Compound Decrease of Organizational Performance	6

RAID should not be the sole FT Measure	6

Vendors and Products	6

Conclusions	7

References	8

APPENDIX A	10

APPENDIX B	12

APPENDIX C	13

APPENDIX D	14

APPENDIX E	15

APPENDIX F	16

Fault Tolerance Through Redundant Arrays of Inexpensive Disks (RAID)

“Companies most valuable assets consist of customer lists, databases, transactions and other electronic data” (ENS, 2004, p. 1). The following paper describes a popular approach to protect and preserve the integrity of data assets: Redundant Arrays of Inexpensive Disks (RAID). RAID levels are presented with alternative solutions, key vendors, and costs of popular solutions. The document lists RAID advantages, disadvantages, benefits, and limitations to provide a guideline of how IT systems can benefit with RAID implementations.

# RAID Configurations

Fault Tolerance (FT) is the set of practices that allow a system to continue functioning even if errors or problems occur. FT techniques are diverse and implemented in multiple areas such as at the physical configuration, network, system, database (DB), software, among others (UoP, 2006) and each area in turn includes specific technologies and techniques. RAID is a popular, inexpensive (for simple configurations), scalable, and widely supported data FT alternative.

Webopedia (2006) explains that RAIDs are "a category of disk drives that employ two or more drives in combination for fault tolerance and performance. RAID disk drives are used frequently on servers but aren't generally necessary for personal computers" (para 1.). Multiple configurations (levels) exist ranging from RAID 0 through 7 with additional variations where a balance between redundancy, cost, and performance is generally a decisive factor for a specific implementation. RAID origins are the copying or “shadowing” of data  to additional disks (RAID 0) and additional levels evolved to include techniques that permitted the verification of copied data, reduce disk utilization, data security implementation, data redundancy, among others. With so vast pool of possible configurations and implementations, RAID solutions range from inexpensive and simple solutions that provide basic data duplication to specialized and very expensive implementations that provide some of the most secure and transparent data protection solutions available. Appendix A provides a description of available RAID configurations.

Advantages and Benefits

RAID implements DB FT based on the consideration that “even more important than the preservation of hardware or programs is the preservation of data” (UoP, 2006, p. 6). Among the most relevant advantages and benefits of RAID implementations:

Inexpensive FT Implementation. RAID 1 (Mirroring) is the complete replication of a server’s data on a secondary drive so that if the primary fails the secondary takes over. Continuous drives’ cost decrease makes RAID 1 affordable to most organizations.

Data Protection. RAID implementations overcome an important limitation of backup procedures: protection of data at all times, even between backups. RAID can provide data security and minimum failover time to ensure minimum service interruption (ENT, 2004, p.1).

Unaffected Productivity during Failures. Systems with RAID implementations can function without any disruption in case of failure allowing users to continue their activities seamlessly. For instance, ENT (2004) describes FPK Architects’ RAID implementation where the main server’s mirroring ensures workers are not shutdown during server problems.

Simple and Quick Configurationn. Products such as CaminoSoft’s Standby server include auto-discovering functions for the primary servers, self-configuration of secondary drive, and automatic initialization of stand-by mode (ENT, 2004) providing organizations lacking sufficient or skilled IT staff the opportunity to exploit RAID benefits.

IT Staff’s Convenience. Certain RAID implementations include automatic error notifications and recovery which generates an environment with reduced stress for responsible IT staff. ENT (2004) describes an implementation where an organization’s server’s secondary drive automatically takes over upon primary’s failures and IT management receives a notification describing the system is running on secondary mode.

High Availability for RAID Implementation on New Equipment. Most new servers’ motherboards already include the necessary drivers to implement SATA RAID (Husted, B., 2006). Organizations wanting to exploit RAID benefits can exploit such availability to implement inexpensive configurations.

Trustworthy and Renowned Manufacturers. Some RAID products represent the shared effort of technology-specific vendors and strong market leaders such as Promise Technology, Inc. that offers SuperTrak EX16350/EX16300 with RAID 6 capability which implements Intel’s processors and Intel’s XScale technology to develop an intelligent RAID architecture for high FT (PR Newswire, 2006). Providers that offer strong solutions ensure that organizations’ investments are protected as no dependency in volatile startups’ solutions are deployed.

Disadvantages and Limitations

RAID implementations include certain disadvantages, limitations, and challenges that need to be considered when choosing the appropriate level such as:

System Performance Degradation. RAID 6, one of the most popular levels, suffers overall system performance degradation after a failure, during the rebuilding of information on the new drive from other drives in the array. Such degradation can amount to a 20% performance drop compared to RAID 5 (Cleland, S., 2005). Appendix E shows a typical RAID 6 solution.

High Costs for High FT without Performance Degradation. 	RAID 6 and higher levels are expensive. Cleland, S. (2005), for example, reports that implementing RAID 6 on an existing RAID 5 system with current market prices represents an additional $300 investment.

Stages with Data Vulnerability on certain RAID Levels. Cleland, S. (2005) explains that RAID 5 arrays are most vulnerable during operation in degraded mode as all data would be lost if a second drive failed during rebuilding. To minimize the likelihood of a second drive failure RAID 6 or the construction of a RAID 5 system with minimum build times is suggested.  Appendix D lists approaches for the construction of RAID 5 systems with reduced build times.

Potential Compound Decrease of Organizational Performance. Cleland, S. (2005) warns that the cost of potential data loss on certain RAID implementations compounded with the time loss of system’s rebuild process can be costly to any organization. As organizations’ data sources and types continue to multiply the potential for some RAID levels’ failures continue to rise increasing the odds of impact on organizational performance. To prepare and avoid such risks companies need to monitor and include the upgrade of RAID configurations in the enterprise-wide security Disaster Recovery (DR) and Business Continuation (BC) initiatives.

RAID should not be the sole FT Measure. A potential risk exists for organizations to consider RAID as the unique source of data FT. RAID should not be a substitute but a complement to data backup and other FT practices as data accidentally deleted or corrupted would be updated on secondary drives in RAID implementations (Husted, B., 2006)

# Vendors and Products

Promise Technology is a main vendor and is the creator and global leader of ATA RAID Solutions that range from SCSI RAID systems, host adapters, RAID controllers, and RAID subsystems (PR Newswire, 2006). This vendor highlights its SuperTrak EX16350/EX16300 solutions with a RAID controller that supports multiple RAID levels such as 0, 1, 5, and 6, among others, and a suggested list price of $849. Appendix B includes a list of these two products’ features with a description of the different RAID levels supported.

To minimize or reduce RAID 6’s performance issues a new generation of RAID 6 is offered by manufactures HP and Compaq (now HP) in the Advanced Data Guarding (ADG) products family. Appendix C provides ADG’s RAID objectives and limitations.

Finally, Seagate Technology’s Maxtor Shared Storage II product family offers RAID implementations with effortless installation, configuration, and management which virtually eliminates any need for special IT staff’s qualifications (M2 Presswire, 2006). Appendix F highlights features of the 1 TB Maxtor Shared Storage II product.

# Conclusions

RAID enables data FT through different levels that balance cost, reliability, protection, and integrity. The main advantages of RAID levels are inexpensive FT implementations (RAID 1), data protection, unaffected productivity during failures, simple and quick FT implementation, IT staff convenience, availability of RAID drivers on new IT equipment, and renowned manufacturers. Some disadvantages include system performance degradation (RAID 6 level mainly), high costs on high RAID implementations without performance degradation, stages with data vulnerability on some RAID levels (RAID 5 primarily), compound effect of Organizational performance degradation if catastrophic RAID failure, and the risk of organizations implementing RAID as the sole FT technique. To implement an adequate RAID level, organizations need to balance the pros and cons of each level to identify the highest ROI for the implementation. Major vendors of RAID equipment: Promise Technology, HP, and Seagate Technology.

# References

Cleland, S. (2005). SATA Recasts Sportlight on RAID 6, but is it right for you?. Enterprise

Networks & Servers. Cedar Park: 11(10), 12, 2. Retrieved September 3rd, 2006 from

ProQuest database

ENS (2004). Mirroring Provides Simple and Cost-Effective Way to Keep Users Productive

When their Server Becomes Unavailable. Enterprise Networks & Servers. Cedar Park:

10(10), 12, 1. Retrieved September 3rd, 2006 from ProQuest database.

HP (2006). Advanced Data Guarding. HP. Retrieved September 3rd, 2006 from

http://h18004.www1.hp.com/products/servers/proliantstorage/arraycontrollers/adg/

Husted, B. (2006). Hard Disk Failure Brings Outpouring of Advice From Readers. The

Atlanta Journal - Constitution. Atlanta, GA:  3. Retrieved September 3rd, 2006 from

ProQuest database

M2 Presswire (2006). Seagate: Seagate Introduces Fast, Centralized Storage for Small Office and

Home Networks. M2 Presswire. Coventry: 1. Retrieved September 3rd, 2006 from

ProQuest database

Maxtor Solutions (2006). Maxtor Shared Storage II 1 TB. Maxtor solutions. Retrieved

September 3rd, 2006 from http://www.maxtorsolutions.com/en/catalog/MSS_II_Dual/

PR Newswire (2006). Promise Technology Introduces New 16-port SATA RAID Controller for

High Capacity Servers and Storage Systems. PR Newswire. New York: 1.

Promise Technology (2006). SuperTrak EX16350 datasheet. Promise Technology, Inc., 2.

Retrieved September 3rd, 2006 from

http://www.promise.com/product/datasheet_list_eng.asp?productId=166

UoP (2006). Fault Tolerance Reading Material. University Of Phoenix: CMGT579, Week 4

Reading, 12.

Webopedia (2006). RAID. Webopedia. Retrieved September 3rd, 2006 from

http://www.webopedia.com/TERM/R/RAID.html

APPENDIX A

RAID levels

| Level | Description |
| --- | --- |
| 0 | Striped Disk Array without Fault Tolerance: Provides data striping (spreading out blocks of each file across multiple disk drives) but no redundancy. This improves performance but does not deliver fault tolerance. If one drive fails then all data in the array is lost |
| 1 | Mirroring and Duplexing: Provides disk mirroring. Level 1 provides twice the read transaction rate of single disks and the same write transaction rate as single disks |
| 2 | Error-Correcting Coding: Not a typical implementation and rarely used, Level 2 stripes data at the bit level rather than the block level. |
| 3 | Bit-Interleaved Parity: Provides byte-level striping with a dedicated parity disk. Level 3, which cannot service simultaneous multiple requests, also is rarely used |
| 4 | Dedicated Parity Drive: A commonly used implementation of RAID, Level 4 provides block-level striping (like Level 0) with a parity disk. If a data disk fails, the parity data is used to create a replacement disk. A disadvantage to Level 4 is that the parity disk can create write bottlenecks. |
| 5 | Block Interleaved Distributed Parity: Provides data striping at the byte level and also stripe error correction information. This results in excellent performance and good fault tolerance. Level 5 is one of the most popular implementations of RAID |
| 6 | Independent Data Disks with Double Parity: Provides block-level striping with parity data distributed across all disks. |
| 0+1 | A Mirror of Stripes: Not one of the original RAID levels, two RAID 0 stripes are created, and a RAID 1 mirror is created over them. Used for both replicating and sharing data among disks |
| 10 | A Stripe of Mirrors: Not one of the original RAID levels, multiple RAID 1 mirrors are created, and a RAID 0 stripe is created over these |
| 7 | A trademark of Storage Computer Corporation that adds caching to Levels 3 or 4 |
| RAID S | EMC Corporation's proprietary striped parity RAID system used in its Symmetrix storage systems. |

Source: Webopedia, 2006

APPENDIX B

SuperTrak EX16350/ EX16300 with RAID 6 Capability Features

Source: Promise Technology, 2006, p. 2

APPENDIX C

RAID Advanced Data Guarding (ADG) Model Comparison

| Model | Purpose | Limitation |
| --- | --- | --- |
| RAID 1+0 | Mirroring: Identical data stored on multiple drives, high fault tolerance, and improved performance. | Requires 50% of capacity to be dedicated to fault protection. Expensive to implement across large volumes (doubling the cost-per-capacity) |
| RAID 5 | Distributed Data Guarding: Parity data is distributed across all drives. Protects against the failure of any one drive and an array. Provides improved performance at a minimum cost | Lower fault tolerance. Not recommended for volumes greater than 14 drives. Too risky for large volumes |
| RAID ADG | Advanced Data Guarding: Two sets of parity data distributed across all drives. Protects against the failure of any two drives in an array. Provides high fault tolerance at a minimum implementation cost | Lower performance than other RAID levels |

Source: HP, 2006, p. 2

APPENDIX D

Approaches To Build RAID 5 Systems with Minimum Build Times

Hot sparing with automatic rebuild. This does not speed up the rebuild but does remove the time delay between drive failures and drive replacement. Multiple arrays on a single controlled can share a single hot spare for automatic rebuild.

Set the rebuild priority to highest level. This will slow the application down during rebuilds but will minimize the exposure time.

Minimize the number of drives per array in line with the storage requirements. The greater the number of drives in a single array, the higher the probability of a second drive’s failure.

The higher the MTBF (Mean time between failures) of the drive, the lower the probability of a drive failure to begin with. Always look for the highest rated drives for your RAID 5 array.

Use a higher number of smaller drives. As stated before, the bigger the drive the longer the re-build time. Smaller drives will shorten the drive re-build time. In addition, smaller capacity drives tend to be significantly cheaper so the cost savings may cover the cost of a hot spare.

Source: Cleland, S., 2005, p. 2

APPENDIX E

RAID 6 Sample Configuration

Source: Cleland, S., 2005, p. 1

APPENDIX F

Maxtor Shared Storage II 1 TB Features and Benefits

| Feature | Benefit |
| --- | --- |
| Sharing | Access files including digital video, music, and photos from any networked PC or Mac for enhanced productivity, seamless workflow and easy file sharing and printing. |
| 1 TB Storage | 1000GB capacity stores 768,000 photos, or 72 hours of DV video, or 16,600 hours of music. |
| Automatic Back Up | No hassle back up. Back up all the computers on the network automatically to a single location. |
| Automatic Data Mirroring | RAID 1 automatically mirrors a copy of your data for an extra level of security (up to 500 GB). |
| Media Streaming | Play back photos, videos, and music to your networked home entertainment center* for more enjoyment. (*Requires UPnPAV certified adapter) |
| Drag and Sort™ | Just drag documents onto the Shared Storage II icon and Maxtor’s Drag and Sort organizes your files, placing them in appropriate shared folders. |
| Maxtor Simple View™ | Provides at-a-glance views of back up and storage status for all users. |
| Password Protection | Set privacy levels on shared folders for full public access or limited access for an extra measure of security. |
| Simple set up | Plug it in, connect the Ethernet cable, and the Maxtor Shared Storage II automatically configures to your network. Within minutes you can set up the Maxtor Shared Storage II to appear as a local hard drive on every configured PC and Mac on the network. |
| USB Expansion | 2 USB ports make it easy to add a printer for sharing or connect additional drives for even more storage |

Source: Maxtor Solutions, 2006