=====================  DOCUMENTATION FOR SUPPORT 1.00 ======================

INDEX

[1] What is Support?

[2] Requirements

[3] Installing and uninstalling Support

[4] Detecting Support

[5] Accessing the keyboard service

[6] Accessing the timer service

[7] Contacting the author

[8] Distribution

[1] What is Support?

Support is a simple TSR COM program (i.e.: a resident program for

DOS) that loads into memory a couple of routines that enhance programmer's

control over the keyboard and timer.

You run Support and it modifies interrupt vectors 8 and 9 (that is

timer and keyboard respectively), installing there two routines. The one

in the timer interrupt enables you to create a time counter that you can

easily access and modify and that can run at a different rate than the BIOS

one. This way, you can control your game (or other applications) with more

accuracy during time. The system time, however, will not get out of synchro-

nization, so you can program freely at higher speeds without messing your

system.

The keyboard routine creates an array of 256 bytes where you can

check individually if each key is pressed or not. Now you can detect many

keys at the same time!  This is very important in games, because sometimes

you need something to follow key combinations instead of single presses.

Support is specially useful when you're working on an interface that

does not let you install ISRs or has problems with them when enabled at run

time (like QuickBasic).

[2] Requirements

Support runs OK on any PC AT with a 80386 or better processor. The

TSR wastes barely more than half a kilobyte of conventional memory. The whole

package uncompressed does not reach 21K of disk space requirement and the

size of the main executable file SUPPORT.COM is less than 1K.

[3] Installing and uninstalling Support

This is very simple: to load Support into memory, simply run the

file SUPPORT.COM with no parameters. To unload Support, use the /u modifier

at the command line. Any other string will bring up the help message.

But there are some things you should take into consideration:

- Do never uninstall Support if you're currently using the routines

- Do not try to unload it if it is not loaded (I will fix this)

- Unload Support after leaving your program and before executing

something else, just in case it is not compatible

A good idea would be to make a batch file to run your game. For

example: suppose your program is called "TETRIS.EXE", make a batch file with

the name "PLAY.BAT" that contains:

@ECHO OFF

SUPPORT

TETRIS.EXE

SUPPORT /U

[4] Detecting Support

Since version 1.00, you can have Support report its version number

and date of release, but even previous versions could be easily detected.

Because you do not want your program to crash if Support is not loaded, it

is good to check first if it is in memory, before trying to access the

services. One way to do this is the following:

- Check the F1h interrupt vector. You can find the offset and segment

at address 0000:03C4. Once you have them, add 12 to the offset and

read memory at that location. You should find the text "SUPPORT".

See the example below.

DEF SEG = 0           'Get segment and offset of interrupt F1h

offs% = CVI(CHR$(PEEK(&H3C4)) + CHR$(PEEK(&H3C5)))

segm% = CVI(CHR$(PEEK(&H3C6)) + CHR$(PEEK(&H3C7)))

offs% = offs% + 12    'Set the displacement

text$ = ""            'Initialize text

DEF SEG = segm%       'Point to the captured segment

FOR i% = offs% TO offs% + 6

text$ = text$ + CHR$(PEEK(i%))

NEXT i%

IF text$ <> "SUPPORT" THEN

PRINT "Error: SUPPORT not found in memory"

END

END IF

Now that you have detected Support, you want to know the version

number, in order to be sure that the services you are going to demand are

available. This is much easier; you just have to call interrupt F1h.

- Whenever you want to call interrupts, you must load QB with the

/L option and include the QB.BI file in your program code.

- After doing that, you can get the version number by calling int F1h

with AH = 2. But remember, versions prior to 1.00 did not have

this function, so they will return registers unchanged and version

number will be incorrect.

'$INCLUDE: 'QB.BI'

DIM regs AS RegTypeX

.

.  (here comes the routine above to detect Support)

.

regs.AX = &H200    'Set AH = 2, get version

CALL INTERRUPTX(&HF1, regs, regs)

IF regs.AX = &H200 THEN

'AX is unchanged, then function 2 is unavailable

PRINT "Support version is prior to 1.00"

ELSE

'AX returns the version number times 100

'BX returns the year

'CH returns the month

PRINT USING "Support version is #.##"; regs.AX / 100

PRINT USING "Release month is ##_-####"; regs.CX \ 256; regs.BX

END IF

[5] Accessing the keyboard service

Once Support is loaded, you can control it through interrupt F1h.

Since this is a user interrupt, it is not recommended that Support is kept

in memory while executing other applications (not made for it).

When you have just loaded Support, the routines are ready for use,

but are still not working: you have to enable them. So there you are, with

your standard BIOS keyboard handler, willing to read many keys at the same

time. Now, what do you do?

Create an array of 256 bytes long. For example, in QuickBasic do

this:

DIM kbuffer(256) AS STRING * 1

Now we're going to call the routine. (In QB, you will need the QB.BI

file included and the /L option in the QB command line). So we must call

interrupt F1h with the following parameters:

AH = 0

DS:DX = Pointer to the new keyboard buffer

BL = Which routine to use

0 = none (you should never need to do this)

1 = BIOS routine only (the default)

2 = Support routine only

3 = Both BIOS and Support routines running together

So in QuickBasic you would do this:

'$INCLUDE: 'QB.BI'

DIM regs AS RegTypeX

DIM kbuffer(256) AS STRING * 1

regs.ax = 0

regs.ds = VARSEG(kbuffer(0))

regs.dx = VARPTR(kbuffer(0))

regs.bx = 3

CALL INTERRUPTX(&HF1, regs, regs)

Now, whenever a key is pressed or realeased, the state of the bytes

in the buffer will change. For example, when you press ESC, you will find that

kbuffer(1) = CHR$(1) and when you release it, kbuffer(1) = CHR$(0). Check the

list of keyboard scancodes to know which element is assigned to every key. If

the key is extended (for example, the grey arrows), the scancode will be

incremented in 128.

Once you've finished using the routine, and before you leave your

program, do not forget to disable it or, as the kbuffer will no longer exist,

Support will overwrite "private property" and you will have a system crash.

To disable the routine, just call it with BX = 1, thus letting the BIOS take

care of the keyboard again on her own.

[6] Accessing the timer service

The other function of Support is creating a timer that goes faster

than the one of the BIOS. This will or will not change the actual timer

frequency, depending on what you specify in register CH. Older versions of

Support (earlier than 1.00) do not give the possibility of changing the

speed, so you have to do it manually. If that is the way you want to proceed,

for any reason, then the following is what you have to do; else, you may

skip this part and go directly to (*).

Do you already know how to change the timer speed manually?

Well, if not, I will tell you quickly without many details, so you can get

the next step.

The timer has a certain maximum speed, which is divided by a value

that is what you really can change. By default, this value is set to 65536,

which gets a speed of 18.2 tics per second. If you'd like to double this

speed, you should half the divisor (i.e.: 32768). To change this value do

this:

OUT &H43, &H3C       'Out 3Ch to port 43h

OUT &H40, lobyte     'Out low-order byte of divisor to port 40h

OUT &H40, hibyte     'Out high-order byte of divisor to port 40h

The divisor must be decremented by one. So, to doulbe the timer

speed, lobyte = &hFF, and hibyte = &h7F. To put it back to normality, you

have lobyte = hibyte = &hFF.

Ok. Now we have a little problem: if we accelerate the timer, then

the whole system will accelerate, that is, the system clock and what is worse,

many applications, like disk chachs and other stuff that are wandering

around at interrupt 8. That could be bad, and even if not, we don't want the

clock to get crazy. So we tell Support to resynchronize with the BIOS, which

takes the clock back to the right speed, but leaves Support at a higher rate.

To do this, we call interrupt F1h with:

AH = 1

CL = frequency (how many times a BIOS tick, we will have a real tick)

DS:DX = pointer to a 4 byte integer (new counter)

In our case, as we want to duplicate speed, CL will be equal to 2.

Picking CL = 1 creates a counter that runs at the same speed that the

BIOS counter. Picking CL = 0 disables the routine, so you can free the

counter. You'd better do it before leaving the program. So, let's make a

sample program:

'$INCLUDE: 'QB.BI'

DIM regs AS RegTypeX

DIM counter as LONG

OUT &H43, &H3C

OUT &H40, &HFF

OUT &H40, &H7F

regs.AX = &H100    'This is AH = 1

regs.CX = 2

regs.DS = VARSEG(counter)

regs.DX = VARPTR(counter)

CALL INTERRUPTX(&HF1, regs, regs)

CLS

DO

LOCATE 1,1

PRINT counter

PRINT TIME$;

LOOP UNTIL INKEY$ <> ""

reg.AX = &H100

reg.CX = 0

CALL INTERRUPTX(&HF1, regs, regs)

OUT &H43, &H3C

OUT &H40, &HFF

OUT &H40, &HFF

(*) Letting Support change the timer speed automatically is very good,

unless you have some very special reason. The system timer won't even know

that you have changed something and you will have control over the timer at

a faster rate. All you have to do is call interrupt F1h with the following

parameters:

AH = 1

CL = frequency (how many times a BIOS tick, we will have a real tick)

CH = EEh (this means the clock rate will be adjusted)

DS:DX = pointer to a 4 byte integer (new counter)

So you no longer need to do the OUTs, that's the only difference.

No more calculations. But be warned that older versions do not support this

feature.

[7] Contacting the author

I will appreciate any postcard, greeting letter or e-mail and will

do my best to try to answer as many questions or suggestions as possible that

you send.  I know Support is a small project,  but I am putting all of my

enthusiasm in it, to get it better and better.

To contact me by e-mail, my current box is pedrosa_lucas@yahoo.com.ar

and I often use Yahoo Messenger too. If you want to have my city address to

send something (which anyway I do not see probable), send me an e-mail first,

and I will give it to you. I do not trust this medium very much... he, he. ;)

Cas

[8] Distribution

You are free to distribute this program, along with the source code

and other files included in this package. It is my intention and my personal

suggestion that these files are kept unmodified when distributing; this way,

people who read this file or the source code will know what this is about or

be able to contact me if he needs some help. Consider this software FREEWARE

or simply FREE.

Because this is scientific or artistic, but under no circumstances

commercial software, I offer no guarantee that the program will work on your

computer or will not damage it, but of course, I have tested this and put all

my effort in fixing errors, and it never failed in my computer. Plus, I never

knew of a piece of software that could damage a computer physically (except,

maybe, BIOS ROM upgrades or things like that).