IsSpice4 introduces a dead-time in your bridge simulations

Christophe BASSO, MOTOROLA Semiconductor, Toulouse, France

February 1998

Bridge or half-bridge designs using MOSFETs or IGBTs need some dead-time between the commutations to avoid any cross-conduction current spikes. This statement is also valid in Switch Mode Power Supplies (SMPS) implementing synchronous rectification. In a simulation environment, it is not always an easy task to write the stimuli so as to define a dead-time between commutations. Classical PULSE or PWL commands are unpractical, especially when either frequency or pulse width are changed during the simulation run. Figure 1 shows the solution built around a few logical XOR gates. The principle uses the truth table of an XOR or XNOR gate which states that its output is at a high or low level when both inputs are at different logical states:

| A | B | S |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

Figure 1

The discrete solution to implement a dead-time generator

This difference between the levels is made through the RC networks R1-C1 and R4-C5. The output of the A1-A5 gates is then a short pulse whose width is dependent upon the RC constant of its input network. This pulse will blank the signal delivered to the output and thus generates the required dead-time.

These logical functions can easily be implemented using INTUSOFT’s IsSpice4 (San-Pedro, CA) Analog Behavioral Modeling features as demonstrated by the netlist given below:

.SUBCKT DEADTIME 1 50 51 {DT=500N VHIGH=10V VLOW=100M RS=10}

* Clock_In Q Qbar

* Developed by Christophe BASSO (FRANCE)

RIN 1 0 1MEG

B6 17 0 V=V(1)>2V ? 10 : 0

R3 17 18 1k

C3 18 0 {DT/(1000*4.14)}

B4 21 0 V=V(25,19)<100MV ? {VLOW} : {VHIGH}

RCQ 21 60 100

CCQ 60 0 10P

BQ 61 0 V=V(60)

RSQ 61 50 {RS}

R4 22 23 1k

C4 23 0 {DT/(1000*4.14)}

B5 24 0 V=V(26,20)<100MV ? {VLOW} : {VHIGH}

RCQB 24 70 100

CCQB 70 0 10P

BQB 71 0 V=V(70)

RSQB 71 51 {RS}

R5 17 25 1k

C5 25 0 {DT/(1000*41.4)}

R6 22 26 1k

C6 26 0 {DT/(1000*41.4)}

D3 23 22 DISCH

D4 18 17 DISCH

B1 22 0 V=V(1)>2V ? 0 : 10

B2 19 0 V=V(17,18)<100MV ? 0 : 10

B3 20 0 V=V(22,23)<100MV ? 0 : 10

.MODEL DISCH D BV=100V CJO=4PF IS=7E-09 M=.45 N=2 RS=.8

+ TT=6E-09 VJ=.6V

.ENDS

The subcircuit needs to be fed with the dead-time value as well as the output high and low levels. The input clock is TTL-CMOS compatible. By changing B5 line to V=V(26,20)<100MV ? {VHIGH} : {VLOW}, the generator becomes suitable to drive a synchronous rectifier, as demonstrated by figure 2.

Figure 2

Application schematic in synchronous rectification

Figure 3 details the output signals delivered by the generator that clearly save the MOSFETs from any conducting overlap.

Figure 3

This picture clearly shows the absence of overlap between commutations