1]	You need RS232C cable to connect between your PC and receiver

2]	You need “bin” file which is upgrade source file.  If the upgrade source file you have is for Positioner model, do not upgrade in FTA digital or Anlog/Digital model.  (Positioner file to positioner receiver, FTA digital file to digital receiver…)

3]	Hyper-terminal setup:

Select in order of Start -> Sub-program -> Hyper-terminal.  Then, Hypertrm icon will be displayed.

4]	If you double click the icon, connection setup screen will appear for connect explanation.

5]	After selecting any icon you like and input any name above the icon, press the “confirm or OK” button.  Then, select Com1 or Com 2 for proper port connection.  If your modem speed is 38400bps, set the bit rate as 38400.

6]	After pressing “YES” or “Confirm” from the above mentioned screen, select “Upgrade IRD S/W” of “System Settings” of your receiver.  Then, another screen will be displayed.

7]	On the screen of left hand side, press “OK” key of front panel or by remote controller unit once more.  Then screen of right hand side will be displayed.  Then, execute the Hyperterminal (by pressing the icon) which you have selected previously.

8]	To send a file for upgrading, select “transfer” among the menu of Hyperterminal and select “Browse” to seek the needed file.

9]	When the proper file is selected, “SEND” can be pressed.  Then, the following screen will be displayed.  The file transfer procedure will be marked as moving dots.  If you do not see any proceeding dots, COM Port setup may be wrong.  To set Com port, select registration information from the file item.

10]	By repeating the procedure, upgrade configuration will be performed differently.

11]	Be sure to turn off receiver or turn off PC during downloading.  It will lead to malfunction of receiver.  When time out sign appears, it means you did all above downloading process to slow.  In such a case, please try again faster.  After successful downloading, the receiver will be initialized by turning off and on automatically.  If it doesn’t get initialized automatically after a while, go to System Settings of Main Menu and press Ok from “Initialization”.  If you see “time out” messages, it means your process time took too long.  Please do again little faster.