PCB v1.8

Exterior Ballistics Software for the IBM PC

(c) Odd Hvard Skevik, 1996

1.0 Short info to the user

1.1     User's licence

1.2     Short description and hardware requirements

1.3     Shortcomings compared to other similar software.

1.4     Known malfunctions and problems

1.5     Description of the files in PCB18-EN.ZIP

2.0 User's guide to PCB

2.1     Calculation of trajectory tables

2.2     Finding the maximum point blank range

2.3     Tolerance interval for range estimation

2.4     Calculate ballistic coefficient

2.5     Intersection between the line of sight and the bore axis.

2.6     Recoil energy and -velocity.

2.7     Estimation of minimum rifling twist.

2.8     Database of bullets.

3.0 Frequent questions and problems.

3.1     I can not find the BC for the bullet I am using.

3.2     The calculations don't coincide with my test firings.

3.3     How do I make graphs of the calculated data.

3.4     PCB halts with a "Run time error" or behaves suspiciously.

3.5     Where can I find more info?

4.0 Short comments on exterior ballistics, programming etc.

4.1     Some comments on exterior ballistics computations.

5.0 How to get in touch with me.

-----------------------------------------------------------------------------

1.1 User's licence.

PCB is not like most other software. It's FREE!

But - that doesn't mean you can treat it anyway you like :

1)  PCB is my work, and I'm a bit proud of it. Therefore, you may

not remove my name from the program or documentation.

2)  You may give PCB to anybody, but give them the original package.

That way, all get the same functionality, and I'm not blamed

for errors that are not mine. You may change your own version as

much as you desire.

3)  You may NOT sell it. When PCB is meant to be free, it shall be

so. And if anybody is to make money on my work, that person

shall be me! (But I don't blame you if you charge a reasonable

sum for the diskette it's stored on.)

4)  You use PCB on your own risk. I assume no liability for any

damage, direct or consequential that the use of PCB may cause

your computer equipment. Neither do I guarantee that PCB is

suitable for any purpose, or that the calculations are correct.

Should PCB cause you any harm, direct or indirect, you assume

the full liability for that damage, economic as well as moral.

I have tested PCB on several computers without problems, but I

have to include this statement anyhow.

BY TAKING PCB INTO USE, YOU ACCEPT ALL THE FOLLOWING TERMS. IF YOU

DO NOT ACCEPT THESE TERMS, YOU ARE NOT ALLOWED TO USE PCB.

Please tell me what you think about PCB. Does it lack anything?

Do you have ideas for improvements? You'll find my address at the

end of this document.

1.2 Short description and hardware requirements.

PCB needs an IBM-PC compatible computer with 640K of RAM to run.

PCB v1.8 has the following possibilities/characteristics :

*   Calculation of exterior ballistics data:

-   Time of flight.

-   Remaining velocity.

-   Kinetic energy

-   Bullet path compared to sight line.

-   Sight adjustments from zero for different ranges.

-   Wind drift, displayable as sight clicks or cm/inches.

Variable parameters :

-   Bullet weight

-   Ballistic coefficient

-   Zero range

-   Point of impact at zero range

-   Crosswind velocity and direction

-   Temperature

-   Altitude above sea level

-   Firing angle, up- or downhill

-   The cant angle of the weapon

*   Other calculations :

-   Maximum point blank range

-   Tolerance interval for range estimations.

-   Near zero range + boresighting distance.

-   Ballistic coefficient.

-   Recoil energy and -velocity

-   Minimum rifling twist

I have included a database of about 830 bullets, so the chances

are good you'll find data for the one you're shooting.

PCB handles both metrical and english units of measurements. You may

also enter data in one set of units and have the calculations

presented in the other set. Easily! Few, if any, cheap programs

handle this as flexibly.

PCB is very fast, and runs fine on a 8088-8Mhz. On faster computers,

waiting time is almost negligible.

1.3 Shortcomings compared to other similar software.

PCB can not make graphical presentations of it's calculations.

This is possible in some other ballistic software. I originally made

PCB for my own use, and since I don't need graphs, the possibility

is not included. I can't really see why hobbyists would need graphs,

either.

Some commercial software can handle different BC values for the same

bullet at changing velocities. This is theoretically more correct

than using a constant value for BC. The effects in trajectory are

however negligible at normal ranges. You might find a difference in

wind drift. To my knowledge Sierra is the only manafacturer to state

BC's for different velocities. As it is difficult to find data, and

as the errors yielded are very small, I have chosen not to include

this functionality.

With these exceptions, I know of no significant shortcomings.

1.4 Known malfunctions and problems.

PCB may be sensitive to unnormal conditions, eg. "wild" input values

or suspicious hardware. If you encounter inexplicable problems,

check your input values.

Besides that, I do not know of any malfunctions in v1.8

As I haven't got the time to test PCB with all possible combinations

of input values, I'm very glad to hear about it if you run into

problems. You'll find my address at the end of this document.

1.5 Description of the files in PCB18-EN.ZIP

PCB.EXE             The main progam executable file.

PCB.DOC             The user's guide. You're reading it now.

TABELL.DAT          Table of data used in the calculations.

NEWS                What's new since the last version? Text file.

DAT2TXT.EXE         Accessory to the bullet database.

TXT2DAT.EXE         Accessory to the bullet database.

RIFLE.DAT           Database of bullets.

2.1 Calculation of trajectory tables.

Select "Calculate Trajectory" from the main menu.

You will now see a screen showing all input variables, and a menu at

the bottom of the screen. This menu is later referred to as the

"input menu"

You may pick a bullet from the database by selecting "Pick bullet",

or you can enter data manually.

If the input values are displayed in metrical units of measurement

and you want english, or vice versa, select "Config" from the input

menu. Place the cursor over the field you want to change and press

enter.

To alter one or more of the input parmeters, select "Change data".

Place the cursor over the variable you want to change and press

<CR>. In the field that was the input menu, you will see a

description of the variable you are about to change, and possibly

also a range the variable has to be within. Type in the new value,

and press <CR> again. Repeat this until all input values are

correct. Return to the input menu by pressing <ESC>.

If you now select "Calculate" from the input menu, you are presented

with a screen showing the calculated data. You can scroll back and

forward with the normal cursor keys, page up / down and home/end.

To return to the input menu, press <ESC>. (This applies to most

situations in PCB: if you want to go back, press <ESC>).

There are a few quick keys that may be used instead of the input

menu. To find out more about them, select "Help".

In the "CONFIG" menu, you can choose what units of measurement

PCB will use for input and output. There's two more switches :

1) Fixed range values / Variable range values.

2) Horizontal zero angle / Variable zero angle.

The first selects wether PCB recalculates the numerical values of

the range values given when you change units of measurement for

input data. Try setting it on/off, then change units and see what

happens.

The latter has to do with shooting up-/downhill. If you select

horizontal zero angle, PCB assumes your rifle is zeroed at a

horizontal range, and calculates the differences that occur when

you fire your rifle at the given angle. Note that the "path" value

at the zeroing distance is non-zero.

With "variable zero angle", PCB assumes that your rifle is zeroed at

the given firing angle. Now, the path at the zeroing range will

be zero.

To save the calculated data to a text file or a printer, select

"save" or "print" from the input menu.

A brief explanation of the different columns in the tables :

1:  Range

2:  Velocity

3:  Energy          1 through 3 are quite obvious.

4:  Time of flight  The time elapsed from the bullet exits the

muzzle until it reaches the given distance.

5:  Drop            The drop of the bullet from the bore axis.

6:  Max path height Max distance between line of sight and bullet

if the rifle were zeroed at this range.

Meaningless at very short ranges.

7:  Path            The distance between LOS and the bullet path.

This is probably what you're looking for.

8:  Drift           Sideways movement of the bullet. Caused either

by wind drift or a canted gun. Set one of them

to 0 to see the effects of the other one.

9:  Clicks up       The number of clicks to adjust the sights UP

from the zeroed posision.

10: Clicks side     The number of clicks to adjust the sights sideways

to compensate for the indicated bullet drift.

2.2 Maximum point blank range.

Select correct input parmeters as described under 2.1.

Select "Routines" from the input menu, and "Point Blank Range" from

the pop-up menu.

A window pops up, and you are prompted for max distance from the

line of sight. This is the maximum distance you can tolerate between

the line of sight and the average point of impact. Note that this

is not the same as half the target height! Remember to detract some

to compensate for normal group sizes.

You are given three answers :

Optimal zero range.

Point of impact at 100m / 100yds

Maximum point blank range.

2.3 Tolerance interval for range estimations.

Let's assume you have a 4" target at 200yds. How much wrong can your

range estimation be without that causing a total miss? This routine

finds the answer.

Select the proper input values as described in 2.1

Select "Routines" and "Tolerance interval"

Enter target heigth and range to target. As for point blank range,

remember to compensate for normal group sizes.

You will be presented with the shortest and the longest distance at

which you could zero your gun and still hit within the given limits.

2.4 Calculate ballistic coefficient.

Select proper input values as described in 2.1

Select "Routines" and "Ballistic Coefficient".

Enter the velocities at two different ranges, and PCB calculates the

corresponding ballistic coefficient.

Velocities at two different ranges can be found from printed data or

by test firing over two accurate chronographs. When test firing,

it's important to use very accurate chronographs and to fire enough

shots. If you wish to do research in this field, I suggest you

read a good book on statistics first, especially the chapter on

confidence-intervals.

2.5 Intersection between line of sight and bore axis.

Bore-sighting is a common way to make a basic adjustment of the

sights. This routine is included as a help for this.

Select proper input values as described in 2.1

Select "routines" and "Boresight distance"

The zero-point closest to the gun, and the intersection distance,

will be displayed in a pop-up window.

2.6 Recoil energy and -velocity.

Select "Other calculations" from the main menu", then

"Recoil velocity/energy". Enter the necessary data :

Bullet mass and muzzle velocity

Mass of powder charge.

Mass of the gun.

The answer is the maximum theoretical recoil velocity and the

corresponding kinetic energy.

The answers are theoretical maximums. Felt recoil is dependant upon

stock design, clothing, muscle tensions, the weight of the shooter,

etc. etc.

2.7 Estimation of minimum rifling twist.

The formula derived by Greenhill is meant for jacketed rifle

projectiles at normal rifle velocities. The results are approximate.

Pistol bullets are said to require much faster twist than the

formula suggests.

Select "Other calculations" then "Minimum rifling twist". Enter

projectile length and diameter.

An answer of 10 inches indicates that the rifling needs at least one

twist in 10 inches.

2.8 Database of bullets.

You may access the database from the database menu under the main

menu, or by selecting "Pick bullet" from the input menu as

previously described.

The screen that pops up shows the registered data for the bullets

registered in the base. The displayed data are :

-   Diameter, either in inches or millimeter.

-   Name or description of the bullets.

-   Weight, either in grains or grams.

-   Ballistic coefficient.

-   Sectional density, either in lbs/sq.inch or g/sq.mm.

The keys used to manouver in the base is described at the bottom of

the database screen.

If you have entered the database to select a bullet for

calculations, place the cursor over the bullet you want, and press

<CR>. That's it.

You may add bullets by pressing <INS> and supplying the needed data.

If you want to delete a bullet, place the cursor over it and press

<DEL>.

If you want to change the registered data for a bullet, place the

cursor over it's entry and press "E".

Some users use other ballistic software as well, or want to do major

changes to the database. For those, I have added two small

utilities: DAT2TXT and TXT2DAT. The former converts PCB's database

to a text file, the latter performs the opposite action. (Remember

to remove the header lines from the text file.)

By using these tools and a text editor it becomes very easy to make

changes to the database, to split it into smaller parts etc.

In the database menu, which is a submenu to the main menu, you also

have the option of saving the database in memory to a file, or

reading a database from a file into memory.

If the database in memory has changed since it last was saved, you

are asked if you want to update the file when you exit PCB.

3.0 Common questions + elementary trouble-shooting guide.

3.1 I can not find the BC for the bullet I am using.

Read section 2.4 on calculating ballistic coefficients. If this

method does not solve your problem, there is one last solution:

You may use data for a similar bullet (caliber, type, weight).

The trajectory is usually very similar, but energy and wind drift

might be slightly different.

3.2 The calculations don't coincide with my test firings.

There are basically three error sources :

1) The entry values for the calculations are wrong.

2) Your shooting has not been up to it's usual standard :-)

3) There is a bug in PCB.

4) Your computer is broken.

I have ranked the errors after likelyhood. Check the two former

before you suspect the two latter. #4 is extremely unlikely.

If you still belive there's a bug in PCB, i'd be very glad to hear

about it. You'll find my address at the end of this document.

3.3 How do I make graphical presentations from the calculated data?

PCB has not this functionality. You can, however, save the

calculated data as a text file and import the data to a spreadsheet.

I have had good results with Microsoft Excel, but some other users

report that Lotus can handle this, too.

3.4 PCB halts with a "Run time error" or behaves suspiciously

This is probably caused by corrupted data in PCB.CFG.

Delete PCB.CFG and try again. PCB probably works OK again.

A special case of this problem is if you have an old PCB.CFG

in the directory where you install a new version. This will often

cause PCB to halt with a RTE before anything else happens. The cure

is as mentioned : delete the old PCB.CFG.

3.5 Where can I find more info?

There are some places on the Internet that provides useful

information. Some are linked to from my site, others not.

Try :

ftp://ftp.flubber.cs.edu, /rec/programs     (source code, software)

A search on any of the search engines, eg. Alta Vista

(altavista.digital.com), for the keywords you want info on, should

provide you with some starting points.

Or, you could drop me an e-mail me. But, I warn you, I'm certainly

no expert!

4.0 Short comments on exterior ballistics, programming etc.

4.1 Some comments on exterior ballistics

The methods involved in common exterior ballistics computations

are a bit strange; they are not derived from basic physics. The most

common method is to take some well-documented empirical data, then

try to fit those to the actual case. There's no theoretical

"right-answer", and several methods give predictions that match

range results very well.

The more common approach uses a retardation-table established

by test firings with a projectile defined as standard. This

table can be fitted to any projectile by assuming that the

air drag of the given projectile always is in a constand

proportion to the drag of the standard bullet.

Some prefer to use this standard table to solve a set of

differential equations. Others compute a new table of coefficients

meant to reflect the behavior of the standard projectile. The former

method is theoretically a little bit more correct, the latter is

significantly faster. I practice, the two methods are equally good.

Another method is to prepare a CD-function for the actual

projectile, then solve the diff. equations. This method is very

accurate, but it takes a tremendous amount of shooting to establish

the CD-function. And, as every projectile has it's own CD-function,

this method is very rarely used for small arms projectiles.

Let's go back to the methods depending on a standard retardation

table. The projectile used to establish that table has a BC defined

as 1.0000. The BC of any other projectile is defined as : BC = Rs/R,

where Rs is the retardation of the standard projectile, and R is the

retardation of the actual projectile. Most small arms projectiles

have BCs less than 1.0. This approach would have been perfect if the

key assumption of constand proportion between the retardations held.

It doesn't. At least not for 99.9 percent of the projectiles

available. Some clever mind have improved the method by giving

several BCs for each projectile, depending on its velocity. A good

method, but very few bullet manafacturers take the time to find more

than one BC value for each bullet. (A projectile is not necessarily

a bullet, but let's forget that for now. When I say bullet, I mean

any projectile used in small arms. OK?) If you want to find

several BCs for the bullet you are using, you will need to fire

quite a lot of them over two very accurate cronographs. There's

really little need to do that, because the single-BC approach is

good enough at all normal ranges.

5.0 How to get in touch with me :

I'd be very glad to hear your opinions on PCB. I'm equally

interested in hearing from satisfied and unsatisfied users. Getting

feedback from satisfied users make me feel good, feedback from

unsatisfied users give me inspiration to further improvements.

The simple way to contact me is via E-mail. My address is :

oddske@stud.ntnu.no

I also have a WWW-page where PCB is located. The address is :

http://www.stud.ntnu.no/~oddske

The latest version of PCB is always available at

http://www.stud.ntnu.no/~oddske/ballistics.html

There's also a version available in Norwegian. No major differences,

but the Norwegian version has better .doc files and are updated more

often. The latest Norwegian version is always available at

http://www.stud.ntnu.no/~oddske/ballistikk.html

If you understand Norwegian, I recommend you use that version.

I hope you will find PCB useful. Good luck !

Odd Hvard Skevik

PS. "Odd" is a common Norwegian name, not an English description!

"Hvard" is pronounced very much like "Howard", and I suspect it

is really the same name. Don't get confused by the "".

I mention this because it seems like some of you have been

wondering.....